Gateway Node Cape Boards – Circuit Verification

Table of Contents

DC-DC Converter Cape	2

DC Voltage Verification	2

SN Switching Cape	2

Switching and Load Measurements	2

Serial (RS-232) Circuits	3

Ground Fault Detection and Environmental Sensing Cape	3

DC Volts Verification	3

Ground Fault Detection Circuit Verification	3

Ground Fault Detection Circuit Calibration	3

Water Sense Leads Circuit Verification	3

Ambient Temperature and Humidity Circuit Verification	3

Thermocouple Measurement Circuit Verification	4

# DC-DC Converter Cape

## DC Voltage Verification

Verify 5V at TP2

Verify 3.3V at TP4

Verify 5V_BB at TP5

Attach load at J2 and verify current measurement at U3-1

Vary input DC voltage (level range TBD) at J1 and measure voltage at U4-1

# SN Switching Cape

| Signal | GPIO | Chip | Pin | Type |
| --- | --- | --- | --- | --- |
| NODE_1_OFF | GPIO1_15 | 1 | 15 | Digital Out |
| NODE_1_ON | GPIO1_14 | 1 | 14 | Digital Out |
| NODE_2_OFF | GPIO1_13 | 1 | 13 | Digital Out |
| NODE_2_ON | GPIO1_12 | 1 | 12 | Digital Out |
| NODE_3_OFF | GPIO1_2 | 1 | 2 | Digital Out |
| NODE_3_ON | GPIO1_3 | 1 | 28 | Digital Out |
| NODE_4_OFF | GPIO1_6 | 1 | 6 | Digital Out |
| NODE_4_ON | GPIO1_7 | 1 | 7 | Digital Out |
| EEPROM_WP2 | GPIO0_27 | 0 | 27 | Digital Out |
| NODE_1_STATE | GPIO2_22 | 2 | 22 | Digital In |
| NODE_2_STATE | GPIO2_23 | 2 | 23 | Digital In |
| NODE_3_STATE | GPIO2_24 | 2 | 24 | Digital In |
| NODE_4_STATE | GPIO2_25 | 2 | 25 | Digital In |

## Switching and Load Measurements

Channel #1

Attach a variable load to J2

Verify relay activation by toggling NODE_1_OFF and NODE_1_ON

Monitor NODE_1_STATE

Listen for relay click

Note the load current at U3-1

Vary the load at J2 and plot the resultant current measurement from U3-1

Channel #2

Attach a variable load to J5

Verify relay activation by toggling NODE_2_OFF and NODE_2_ON

Monitor NODE_2_STATE

Listen for relay click

Note the load current at U6-1

Vary the load at J5 and plot the resultant current measurement from U6-1

Channel #3

Attach a variable load to J7

Verify relay activation by toggling NODE_3_OFF and NODE_3_ON

Monitor NODE_3_STATE

Listen for relay click

Note the load current at U9-1

Vary the load at J7 and plot the resultant current measurement from U9-1

Channel #4

Attach a variable load to J9

Verify relay activation by toggling NODE_4_OFF and NODE_4_ON

Monitor NODE_4_STATE

Listen for relay click

Note the load current at U12-1

Vary the load at J9 and plot the resultant current measurement from U12-1

## Serial (RS-232) Circuits

# Ground Fault Detection and Environmental Sensing Cape

| Signal | GPIO | Chip | Pin | Type |
| --- | --- | --- | --- | --- |
| 48V_GF | GPIO1_0 | 1 | 0 | Digital Out |
| 48V_RET_GF | GPIO1_4 | 1 | 4 | Digital Out |
| TEST_GF | GPIO1_29 | 1 | 29 | Digital Out |
| TEST_RET_GF | GPIO1_1 | 1 | 1 | Digital Out |
| EEPROM_WP1 | GPIO0_26 | 0 | 26 | Digital Out |
| H2O_ALARM | GPIO1_5 | 1 | 5 | Digital In |
| ENVR_ALARM | GPIO1_31 | 1 | 31 | Digital In |

## DC Volts Verification

Verify 5V_GF at TP6

Verify -5V_GF at TP7

Verify 5V_ISO at TP1 (w/r to TP3)

Verify -5V_ISO at TP2 (w/r to TP3)

Verify VOLT_GF (15Vdc) at TP4 (w/r to TP5)

## Ground Fault Detection Circuit Verification

Toggle all four ground fault circuits one at time

48V_RET_GF

48V_GF

TEST_RET_GF

TEST_GF

## Ground Fault Detection Circuit Calibration

[Needs an extensive write up, see old notes & Multisim simulations]

## Water Sense Leads Circuit Verification

Attach a water sense lead to J5.

Dip into seawater – H2O_ALARM should go high

## Ambient Temperature and Humidity Circuit Verification

Using the HIH6131 driver, read back the temperature and humidity measurements.

## Thermocouple Measurement Circuit Verification

Attach a K-Type thermocouple to J4.

Using MAX31855 driver, read back the temperature measurement.