Shallow Water FOCE Gateway Node Control System

Hardware Interface Document, v0.1

Chad Kecy

Monterey Bay Aquarium Research Institute

June 23, 2014

Table of Contents

Overview	5

System Level Description	5

BeagleBone Computer Board	6

Functional Description	7

Serial Communication	7

Analog Signals	7

Digital Signals	8

SPI Signals	8

Expansion Headers	8

DC-DC Converter Cape	8

Functional Description	9

DC-DC Conversion	9

Optional Auxiliary DC Power Supply	9

Source Voltage and Current Monitoring	9

Scaled Voltage	9

Measured Current	10

Test Connector	10

Sensor Node Switching Cape	10

Functional Description	11

Load Switching and Monitoring	11

Serial Interface	12

Boot Isolation Pins	12

Cape EEPROM	12

Test Connectors	12

Ground Fault Detection and Environmental Sensor Cape	12

Functional Description	13

Ground Fault Detection	14

Environmental Sensing	14

Boot Isolation Pins	14

Cape EEPROM	14

Test Connectors	14

Board Level Signal Descriptions	16

Digital Output Signals	16

Digital Input Signals	21

Analog Input Signals	22

SPI Bus Signals	25

I2C Signals	25

Serial Signals	25

Appendicies	26

Figure 1: Possible Gateway Node configuration on swFOCE	5

Figure 2: Block Diagram of the Gateway Node System	6

Figure 3: BeagleBone Computer Board	7

Figure 4: DC-DC Converter Cape	8

Figure 5: Gateway Node DC-DC Conveter Cape Block Diagram	9

Figure 6: Sensor Node Switching Cape	10

Figure 7: Gateway Node SN Switching Cape Block Diagram	11

Figure 8: Ground Fault Detection and Environmental Sensing Cape	13

Figure 9: Gateway Node Ground Fault Detection and Environmental Sensing Cape Block Diagram	14

# Overview

The Gateway Node is an open source computer system which can be used to control a wide range of oceanographic experiments. It is a series of stackable circuit boards based around the BeagleBone computer board. The daughter boards, or capes, have each been designed with a specific function. These include a DC-DC Converter cape, a Sensor Node Switching Cape, and a Ground Fault Detection and Environmental Sensing Cape. Because this is a modular system, the end-user can customize their Gateway Node system to meet their specific needs. Additional boards can also be fabricated using the BeagleBone cape design specification.

# System Level Description

The Gateway Node serves two primary purposes in the swFOCE experiment: it collects data from all of the instrumentation and acts as the main computer in the control loop. It provides power (48Vdc) and serial communication to a maximum of four Sensor Nodes.

Figure 1:  Gateway Node and Sensor Nodes

Figure 2: Block Diagram of the Gateway Node System

# Vicor Supply

The Gateway Node housing contains a single 250W 48V:48V Vicor Mini DC-DC Converter module. The Vicor supplies power to the Gateway Node as well as to the attached Sensor Nodes. It is an isolating DC-DC converter; being on a separate DC power bus aids in ground fault detection.

# BeagleBone Computer Board

Figure 3: BeagleBone Computer Board

## Functional Description

The heart of the GN is the BeagleBone computer board; it contains a TI AM3358 720MHz ARM Cortex-A8 based microprocessor and 256MB of DDR2 memory. Among its features are an Ethernet interface, USB port, microSD card slot, 7 analog inputs, 4 serial ports and 65 GPIO lines. There are also 2 46-pin expansion headers which allow for custom cape boards to be interfaced. Up to 4 cape boards can be individually addressed via onboard EEPROMs.

## Serial Communication

The four available serial ports from the BeagleBone are used to communicate with the attached Sensor Nodes. Separate one-channel transceivers are used in the event of a failure only affecting one channel.

## Analog Signals

The BeagleBone has seven 12-bit analog inputs which are read at 100K samples per second. The analog signals cannot exceed 1.8V; use a voltage divider for signals that exceed this range. The measured analog signals are the four Sensor Node output currents, the voltage and current of the input 48Vdc, and the measured ground fault current.

## Digital Signals

## SPI Signals

## Expansion Headers

# DC-DC Converter Cape

Figure 4: DC-DC Converter Cape

## Functional Description

The role of the DC-DC converter cape is to generate all of the lower voltages needed by the BeagleBone and the attached capes. Dedicated switching regulators provide 3 amps of 3.3Vdc and 5.0Vdc to the attached capes. An optional adjustable regulator can be used provide power to additional hardware or sensors.

Figure 5: Gateway Node DC-DC Converter Cape Block Diagram

## Logic Power

A pair of LM5576 buck regulators are used to generate the 3.3Vdc and 5.0Vdc used by the attached cape boards. These switchers contain an integrated 170m ohm N-Channel MOSFET and are capable of supplying a continuous output of 3 amps. The output filters were designed to limit the output ripple at less than 1%.

A 1 amp switching regulator (MAX5035) generates the 5V dedicated to the BeagleBone. All DC voltages share a common return.

While the BeagleBone can supply logic power to attached capes, the DC buses are limited to one amp. As part of the board population, the male pins are removed (cut off) to prevent two separate regulators from supplying the logic power to the capes.

Figure 6: Block diagram of the logic power routing on the Gateway Node cape stack

## Optional Auxiliary DC Power Supply

A spare LM5576 switching regulator allows the user to generate a specific voltage for optional circuitry or hardware at J3. The output voltage is determined by the selection of two feedback resistors and can be any value between 1.22Vdc and 48Vdc. The regulator is rated at 3 amps.

If R17 is set at 4.99Kohm, insert the desired voltage into the following equation and solve for R19:

R19 = 4999 / (Vout/1.225) – 1)

## Source Voltage and Current Monitoring

The voltage level and current usage of the source 48Vdc is measured and read by the BeagleBone computer board. The maximum allowed analog input voltage for the ADC on the BeagleBone is 1.8V.

### Scaled Voltage

U4, R3, and R7 are used to scale the incoming 48Vdc to an appropriate level for the ADC onboard the BeagleBone. For 48Vdc, the scaled voltage should be 1.58Vdc. This is the signal 48V_VOLT on the cape bus.

Vout = 0.0329 * Vin

### Measured Current

U2, U3, R6, and C12 are used to measure the current drawn by the 48Vdc loads. The circuit outputs a scaled voltage which is representative of the load current. The maximum measured current is 6 amps. This is the signal 48V_CURR on the cape bus.

Vout = 0.264 * ILoad  [voltage to ADC]

I = Vout/0.264  [actual load current based on measured voltage]

## Test Connector

A test connector, J4, with signals of interest is available to assist with calibration and/or troubleshooting.

# Sensor Node Switching Cape

Figure 7: Sensor Node Switching Cape

## Functional Description

The Sensor Node Switching Cape provides two primary functions: switching power to and communicating with four Sensor Nodes.

Figure 8: Gateway Node Sensor Node Switching Cape Block Diagram

## Load Switching and Monitoring

Four separate load paths are used to switch power (48Vdc) and monitor the current used by each Sensor Node. The DK series relays from Panasonic PEWA are pc mounted dual coil latching 2 Form C relays; electromechanical relays are used to provide complete galvanic isolation in the event of a ground fault. Separate on and off pulses from the BeagleBone drive P-channel MOSFETs for coil activation. The use of latching relays cuts down on the quiescent current draw of the system. Relay state feedback is provided by opto-isolated relays.

Current sense amplifiers (LT1787) are used to measure the current delivered to each Sensor Node. A proportional voltage output is sent through a first order low pass filter prior to being sent to an analog in port on the BeagleBone. The maximum current per load path is 2 amps.

Each current sense amplifier uses a 0.100 ohm sensing resistor and has an internal gain of 8. The voltage proportional to the load current is calculated by:

Vout = ILoad x 0.100 ohms x 8

## Serial Interface

The TTL level serial lines from the BeagleBone are routed through the cape bus to the Sensor Node Switching cape. Individual serial transceivers translate the signals to EIA levels before being routed to the Sensor Nodes.

## Boot Isolation Pins

The BeagleBone gives the users the ability to change the boot order by setting the state of 16 reserved digital input/output pins during the boot cycle. Once the boot cycle is complete, these 16 lines are then available for general use. This makes it necessary for these 16 pins to be held in a high impedance state during the boot cycle if one wishes to use the default boot order. Two octal tri-state buffers on the Sensor Node Switching cape are used to keep the 16 pins in a high impedance state during boot up.

## Cape EEPROM

Each cape in a BeagleBone stack must have a unique ID number associated with it if it intends to interface with other commercially available cape boards. An EEPROM was added to keep the Sensor Node Switching cape compatible with the BeagleBone format. If using only boards dedicated to the Gateway Node, then no IDs need to be used. Consult the BeagleBoard User’s Manual for more detailed information.

## Test Connectors

Four test connectors were added allowing easy access to signals of interest. See table xx for a listing of the signals.

# Ground Fault Detection and Environmental Sensor Cape

Figure 9: Ground Fault Detection and Environmental Sensing Cape

## Functional Description

Text of functional Description

Figure 10: Gateway Node Ground Fault Detection and Environmental Sensing Cape Block Diagram

## Ground Fault Detection

The GN Carrier board has the ability to check for ground faults on the +48Vdc line and Return that go to the SNs. Digital signals from the BB test the positive and negative rails of the 48Vdc against a seawater reference return. The analog signal GF_CURR will register a fault in either direction. There is also a test channel that places a known intentional short to ground on an isolated voltage source in order to test and calibrate the GF circuitry.

## Environmental Sensing

The GN Carrier board will eventually be housed in a PVC housing and placed underwater, therefore standard sensors are included. These include ambient temperature, humidity, and a water presence detection circuit. There is also a thermocouple conditioning circuit for monitoring the temperature of the Vicor Mini module (type K).

## Boot Isolation Pins

## Cape EEPROM

## Test Connectors

| Name | Signal Type | Dir (w/r to BB) | BB Expan Signal | BB Expan Port | BB Mode | Active (Outputs) |
| --- | --- | --- | --- | --- | --- | --- |
| NODE_1_ON | Digital | Out | GPIO1_14 | P8-16 | 7 | Low Pulse |
| NODE_1_OFF | Digital | Out | GPIO1_15 | P8-15 | 7 | Low Pulse |
| NODE_1_STATE | Digital | In | GPIO2_22 | P8-27 | 7 |  |
| NODE_1_CURR | Analog | In | AIN0 | P9-39 |  |  |
| NODE_2_ON | Digital | Out | GPIO1_12 | P8-12 | 7 | Low Pulse |
| NODE_2_OFF | Digital | Out | GPIO1_13 | P8-11 | 7 | Low Pulse |
| NODE_2_STATE | Digital | In | GPIO2_23 | P8-29 | 7 |  |
| NODE_2_CURR | Analog | In | AIN1 | P9-40 |  |  |
| NODE_3_ON | Digital | Out | GPIO1_28 | P9-12 |  | Low Pulse |
| NODE_3_OFF | Digital | Out | GPIO1_2 | P8-5 | 7 | Low Pulse |
| NODE_3_STATE | Digital | In | GPIO2_24 | P8-28 | 7 |  |
| NODE_3_CURR | Analog | In | AIN2 | P9-37 |  |  |
| NODE_4_ON | Digital | Out | GPIO1_7 | P8-4 | 7 | Low Pulse |
| NODE_4_OFF | Digital | Out | GPIO1_6 | P8-3 | 7 | Low Pulse |
| NODE_4_STATE | Digital | In | GPIO2_25 | P8-30 | 7 |  |
| NODE_4_CURR | Analog | In | AIN3 | P9-38 |  |  |
| UART_1_TTL_TXD | Digital | Out | UART1_TXD | P9-24 |  |  |
| UART_1_TTL_RXD | Digital | In | UART1_RXD | P9-26 |  |  |
| UART_2_TTL_TXD | Digital | Out | UART2_TXD | P9-21 |  |  |
| UART_2_TTL_RXD | Digital | In | UART2_RXD | P9-22 |  |  |
| UART_4_TTL_TXD | Digital | Out | UART4_TXD | P9-13 |  |  |
| UART_4_TTL_RXD | Digital | In | UART4_RXD | P9-11 |  |  |
| UART_5_TTL_TXD | Digital | Out | UART5_TXD | P8-37 |  |  |
| UART_5_TTL_RXD | Digital | In | UART5_RXD | P8-38 |  |  |
| 48V_RET_GF | Digital | Out | GPIO1_4 | P8-23 |  | High SS |
| 48V_GF | Digital | Out | GPIO1_0 | P8-25 |  | High SS |
| TEST_RET_GF | Digital | Out | GPIO1_1 | P8-24 |  | High SS |
| TEST_GF | Digital | Out | GPIO1_29 | P8-26 |  | High SS |
| GF_CURR | Analog | In | AIN4 | P9-33 |  |  |
| H2O_ALARM | Digital | In | GPIO1_5 | P8-22 |  |  |
| 48V_VOLT | Analog | In | AIN5 | P9-36 |  |  |
| 48V_CURR | Analog | In | AIN6 | P9-35 |  |  |
| ENVR_ALARM | Digital | In | GPIO1_31 | P8-20 |  |  |
| EEPROM_WP1 | Digital | Out | GPIO0_26 | P8-14 |  | High SS |
| EEPROM_WP2 | Digital | Out | GPIO0_27 | P8-17 |  | High SS |
| EEPROM_WP3 | Digital | Out | GPIO2_1 | P8-18 |  | High SS |
| EEPROM_WP4 | Digital | Out | GPIO1_30 | P8-21 |  | High SS |

| Chip | Pin | Addr | Dir | Act | Val | Signal |
| --- | --- | --- | --- | --- | --- | --- |
| 1 | 15 | 47 | Out | Low | High | NODE_1_OFF |
| 1 | 14 | 46 | Out | Low | High | NODE_1_ON |
| 2 | 22 | 86 | In | Low | N/A | NODE_1_STATE |
| 1 | 13 | 45 | Out | Low | High | NODE_2_OFF |
| 1 | 12 | 44 | Out | Low | High | NODE_2_ON |
| 2 | 23 | 87 | In | Low | N/A | NODE_2_STATE |
| 1 | 2 | 34 | Out | High | Low | NODE_3_OFF |
| 1 | 28 | 60 | Out | High | Low | NODE_3_ON |
| 2 | 24 | 88 | In | Low | N/A | NODE_3_STATE |
| 1 | 6 | 38 | Out | High | Low | NODE_4_OFF |
| 1 | 7 | 39 | Out | High | Low | NODE_4_ON |
| 2 | 25 | 89 | In | Low | N/A | NODE_4_STATE |
| 1 | 4 | 36 | Out | High | Low | 48V_RET_GF |
| 1 | 0 | 32 | Out | High | Low | 48_GF |
| 1 | 1 | 33 | Out | Low | Low | TEST_RET_GF |
| 1 | 28 | 60 | Out | Low | Low | GPIO1_28 |
| 1 | 29 | 61 | Out | Low | Low | TEST_GF |
| 1 | 5 | 37 | In | Low | N/A | H2O_ALARM |
| 1 | 31 | 63 | In | Low | N/A | ENVR_ALARM |
| 1 | 30 | 62 | Out | High | Low | EEPROM_WP |

# Board Level Signal Descriptions

## Digital Output Signals

All digital output signals originate on the BeagleBone board and are used on one or more of the attached cape boards. They use +3.3Vdc logic. The default (boot up) state and active levels are assigned in the BeagleBone firmware.

NODE_1_ON

GPIO1_14

Addr = 46

P8-16

Sensor Node Switching Cape

NODE_1_ON is used to apply power to Sensor Node #1; it does this by activating the reset coil of relay K1. The default state is high and its active level is a negative going pulse 5 msec in duration.

NODE_1_OFF

GPIO1_15

Addr = 47

P8-15

Sensor Node Switching Cape

NODE_1_OFF is used to remove power from Sensor Node #1; it does this by activating the set coil of relay K1. The default state is high and its active level is a negative going pulse 5 msec in duration.

NODE_2_ON

GPIO1_12

Addr = 44

P8-12

Sensor Node Switching Cape

NODE_2_ON is used to apply power to Sensor Node #2; it does this by activating the reset coil of relay K3. The default state is high and its active level is a negative going pulse 5 msec in duration.

NODE_2_OFF

GPIO1_13

Addr = 45

P8-11

Sensor Node Switching Cape

NODE_2_OFF is used to remove power from Sensor Node #2; it does this by activating the set coil of relay K3. The default state is high and its active level is a negative going pulse 5 msec in duration.

NODE_3_ON

GPIO1_28

Addr = 60

P9-12

Sensor Node Switching Cape

NODE_3_ON is used to apply power to Sensor Node #3; it does this by activating the reset coil of relay K5. The default state is high and its active level is a negative going pulse 5 msec in duration.

NODE_3_OFF

GPIO1_5

Addr = 37

P8-5

Sensor Node Switching Cape

NODE_3_OFF is used to remove power from Sensor Node #3; it does this by activating the set coil of relay K5. The default state is high and its active level is a negative going pulse 5 msec in duration.

NODE_4_ON

GPIO1_7

Addr = 39

P8-4

Sensor Node Switching Cape

NODE_4_ON is used to apply power to Sensor Node #4; it does this by activating the reset coil of relay K7. The default state is high and its active level is a negative going pulse 5 msec in duration.

NODE_4_OFF

GPIO1_6

Addr = 38

P8-3

Sensor Node Switching Cape

NODE_4_OFF is used to remove power from Sensor Node #4; it does this by activating the set coil of relay K7. The default state is high and its active level is a negative going pulse 5 msec in duration.

48V_GF

GPIO1_0

Addr = 32

P8-25

Ground Fault and Environmental Sensing Cape

48V_GF is used to test the source 48Vdc for conductivity to the seawater reference; it does this by activating opto-relay K4. The default state is low and its active level is a steady state high.

48V_RET_GF

GPIO1_4

Addr = 36

P8-23

Ground Fault and Environmental Sensing Cape

48V_RET_GF is used to test the source 48Vdc Return for conductivity to the seawater reference; it does this by activating opto-relay K1. The default state is low and its active level is a steady state high.

TEST_GF

GPIO1_29

Addr = 61

P8-26

Ground Fault and Environmental Sensing Cape

TEST_GF is used to test a reference voltage for conductivity to the seawater reference; it does this by activating opto-relays K3 and K6. This signal allows for the calibration and verification of the ground fault detection circuitry. The default state is low and its active level is a steady state high.

TEST_RET_GF

GPIO1_1

Addr = 33

P8-24

Ground Fault and Environmental Sensing Cape

TEST_RET_GF is used to test a reference voltage return for conductivity to the seawater reference; it does this by activating opto-relays K2 and K5. This signal allows for the calibration and verification of the ground fault detection circuitry. The default state is low and its active level is a steady state high.

EEPROM_WP1

GPIO0_26

Addr = 26

P8-14

Ground Fault and Environmental Sensing Cape

EEPROM_WP1 is a write protect signal for the CAT24C256WI-GT3 EEPROM, U14. When high, the memory of U14 is protected against write operations, but can still be read back. When low, write operations may be performed.

EEPROM_WP2

GPIO0_27

Addr = 27

P8-17

Sensor Node Switching Cape

EEPROM_WP2 is a write protect signal for the CAT24C256WI-GT3 EEPROM, U14. When high, the memory of U14 is protected against write operations, but can still be read back. When low, write operations may be performed.

EEPROM_WP3

GPIO2_1

Addr = 65

P8-18

[future cape board]

EEPROM_WP3 is a write protect signal for the CAT24C256WI-GT3 EEPROM. When high, the memory is protected against write operations, but can still be read back. When low, write operations may be performed.

EEPROM_WP4

GPIO1_30

Addr = 62

P8-21

[future cape board]

EEPROM_WP4 is a write protect signal for the CAT24C256WI-GT3 EEPROM. When high, the memory is protected against write operations, but can still be read back. When low, write operations may be performed.

## Digital Input Signals

All digital inputs signals originate on the attached cape boards and are read by the BeagleBone computer board. They use +3.3Vdc logic. The active level is assigned in the BeagleBone firmware.

NODE_1_STATE

GPIO2_22

Addr = 86

P8-27

Sensor Node Switching Cape

NODE_1_STATE indicates the power state of Sensor Node #1. Opto relay K2 drives this signal high when Sensor Node #1 is energized. It is low when Sensor Node #1 is off.

NODE_2_STATE

GPIO2_23

Addr = 87

P8-29

Sensor Node Switching Cape

NODE_2_STATE indicates the power state of Sensor Node #2. Opto relay K4 drives this signal high when Sensor Node #2 is energized. It is low when Sensor Node #2 is off.

NODE_3_STATE

GPIO2_24

Addr = 88

P8-28

Sensor Node Switching Cape

NODE_3_STATE indicates the power state of Sensor Node #3. Opto relay K6 drives this signal high when Sensor Node #3 is energized. It is low when Sensor Node #3 is off.

NODE_4_STATE

GPIO2_25

Addr = 89

P8-30

Sensor Node Switching Cape

NODE_4_STATE indicates the power state of Sensor Node #4. Opto relay K8 drives this signal high when Sensor Node #4 is energized. It is low when Sensor Node #4 is off.

H2O_ALARM

GPIO1_5

Addr = 37

P8-22

Ground Fault and Environmental Sensing Cape

H2O_ALARM indicates the presence of seawater in the Gateway Node subsea housing; it does this by driving an opto-isolator, U13, high if seawater is detected. A high reading indicates water in the housing while a low indicates a dry interior.

ENVR_ALARM

GPIO1_31

Addr = 63

P8-20

Ground Fault and Environmental Sensing Cape

ENVR_ALARM indicates the either the ambient temperature or humidity has gone out of the programmed range. Its default state is high; a low indicates an alarm. The actual values can then be read back over the SPI bus.

## Analog Input Signals

All analog input signals originate on the attached cape boards and are read by the onboard ADC on the BeagleBone computer board. The maximum voltage level on all analog input signals is 1.8V.

NODE_1_CURR

AIN0

P9-39

Sensor Node Switching Cape

NODE_1_CURR is a voltage which is representative of the current drawn by Sensor Node #1. The maximum current drawn and measured is 2 amps, which corresponds to an output of 1.6Vdc.

NODE_1_CURR (volts) = ILoad (amps) * 0.1 (ohms) * 8

Once this value is read by the ADC on the BeagleBone, the actual current may be found:

ILoad = NODE_1_CURR (volts) / 0.8 (ohms)

NODE_2_CURR

AIN1

P9-40

Sensor Node Switching Cape

NODE_2_CURR is a voltage which is representative of the current drawn by Sensor Node #2. The maximum current drawn and measured is 2 amps, which corresponds to an output of 1.6Vdc.

NODE_2_CURR (volts) = ILoad (amps) * 0.1 (ohms) * 8

Once this value is read by the ADC on the BeagleBone, the actual current may be found:

ILoad = NODE_2_CURR (volts) / 0.8 (ohms)

NODE_3_CURR

AIN2

P9-37

Sensor Node Switching Cape

NODE_3_CURR is a voltage which is representative of the current drawn by Sensor Node #3. The maximum current drawn and measured is 2 amps, which corresponds to an output of 1.6Vdc.

NODE_3_CURR (volts) = ILoad (amps) * 0.1 (ohms) * 8

Once this value is read by the ADC on the BeagleBone, the actual current may be found:

ILoad = NODE_3_CURR (volts) / 0.8 (ohms)

NODE_4_CURR

AIN3

P9-38

Sensor Node Switching Cape

NODE_4_CURR is a voltage which is representative of the current drawn by Sensor Node #4. The maximum current drawn and measured is 2 amps, which corresponds to an output of 1.6Vdc.

NODE_4_CURR (volts) = ILoad (amps) * 0.1 (ohms) * 8

Once this value is read by the ADC on the BeagleBone, the actual current may be found:

ILoad = NODE_4_CURR (volts) / 0.8 (ohms)

GF_CURR

AIN4

P9-33

Ground Fault and Environmental Sensing Cape

GF_CURR is a voltage which is representative of the electrical current between seawater reference and the ground fault channel under test.

[add more after GF circuit is calibrated and tested – CDK 08/21/2014]

48V_VOLT

AIN5

P9-36

DC-DC Converter Cape

48V_VOLT is a scaled voltage representative of the actual 48Vdc source. For 48Vdc, the scaled voltage should be 1.58Vdc.

48V_VOLT = 0.0329 * Vin

Once this value is read by the ADC on the BeagleBone, the actual voltage may be found:

V = 30.4 * 48V_VOLT

48V_CURR

AIN6

P9-35

DC-DC Converter Cape

48V_CURR is a voltage which is representative of the current drawn by the entire load of the 48Vdc source. The maximum current drawn and measured is 6 amps, which corresponds to an output of 1.6Vdc.

48V_CURR (volts) = ILoad (amps) * 0.033 (ohms) * 8

Once this value is read by the ADC on the BeagleBone, the actual current may be found:

ILoad = NODE_4_CURR (volts) / 0.264 (ohms)

** Add in AMB_HUM, AMB_TEMP, and VICOR_TEMP **

## SPI Bus Signals

SPI1_MOSI

SP1_MISO

SPI1_SCLK

SPI1_CS0

SPI1_CS1

## I2C Signals

I2C2_SDA

I2C2_SCL

## Serial Signals

UART_1_TTL_TXD

UART_1_TTL_RXD

UART_2_TTL_TXD

UART_2_TTL_RXD

UART_4_TTL_TXD

UART_4_TTL_RXD

UART_5_TTL_TXD

UART_5_TTL_RXD

# Appendicies