Shallow Water FOCE Sensor Node Prototype

Interface Control Document  v0.4

Chad Kecy

03-08-2013

System Level Description

The role of the Sensor Node (SN) as part of the swFOCE experiment is to interface a wide range of underwater instruments. The SN provides power and serial communication to three instruments, and also has one high-current power only port. Up to four SNs connect to the Gateway Node (GN) and communicate via RS-232 serial communication. The GN provides the SN with 48V (75W).The SN can provide 12Vdc to 48Vdc with a maximum of 50 watts available per SN. The SN utilized RS-232 serial communication with a maximum baud rate of 1Mbps. Data from the individual instruments is stored locally at the SN until requested from the GN. The GN handles all decision making; the SN is simply a data aggregator.

Function Description

DC-DC Conversion

The SN contains a single DC-DC converter which provides a common DC voltage for the three attached instruments. The board has been designed to accept two families of converters which allow the user to determine the output voltage and the maximum output power (for greater efficiency). The input and output filters were designed to accommodate all of the available converters. A jumper selectable trim up/down potentiometer is also included. The current and voltage level of the instrument bus voltage (BUS_VOLT) is monitored and recorded. A digital signal from the PIC can disable the DC-DC converter in the event power to all three instruments needs to be removed quickly. The logic power is provided by separate voltage regulator which is supplied directly by the input 48Vdc.

Load Paths

The instrument bus voltage is split into four paths: three standard instrument ports and one high-current power port. The standard instrument ports are limited to 2A of current, regardless of bus voltage. The high-current port can provide up to the maximum of 6A. The current usage of all four ports is monitored.

Load switching to each port is accomplished via dual coil latching relays. Separate on and off pulses are used to change the relay states. A digital signal indicating the state of the relay is sent back to the PIC for independent confirmation.

The high-current port has a different family of relay that allows for higher current usage; it can handle the maximum current at 50W for any of the chosen DC-DC converters.

PIC and Serial Communication

The SN board contains a PIC24F microcontroller with four serial UARTs. Three are used for communication to the instruments, and the fourth is for communication back to the GN. Separate one-channel transceivers are used in the event of a failure only affecting one channel. The PIC handles all of the digital input and output signals, the four UARTs, the spi bus, and, optionally, the analog signals. An onboard boot loader allows the operating program to be loaded remotely. All of the digital signals, as well as the TTL level UART signals, and the spi bus are brought out to 0.100" headers for easy debugging.

Analog Signals

The bus voltage and current, as well as the individual instrument current levels are measured, filtered, and sent to two ADCs: the onboard 10-bit ADC of the PIC or a spi bus 16-bit ADC. The analog signals and power supply voltages are accessible via test points.

Figure One: Possible Sensor Node configuration on Shallow Water FOCE

| PIC24FJ256GA106 Pinout for swFOCE Sensor Node Proto |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Pin # | Func1 | Func2 | Func3 | Func4 | Func5 | Func6 | Func7 | Functional Description |
| 1 | CN63 | PMD5 | RE5 |  |  |  |  | INSTR_3_OFF |
| 2 | SCL3 | CN64 | PMD6 | RE6 |  |  |  | INSTR_4_ON |
| 3 | SDA3 | CN65 | PMD7 | RE7 |  |  |  | INSTR_4_OFF |
| 4 | PMA5 | RP21 | C1IND | CN8 | RG6 |  |  | SPI_*CS |
| 5 | C1INC | RP26 | CN9 | PMA4 | RG7 |  |  | SPI_SCLK |
| 6 | C2IND | RP19 | CN10 | PMA3 | RG8 |  |  | SPI_MISO |
| 7 | *MCLR |  |  |  |  |  |  | *MCLR |
| 8 | C2INC | RP27 | CN11 | PMA2 | RG9 |  |  | SPI_MOSI |
| 9 | Vss |  |  |  |  |  |  | Vss |
| 10 | Vdd |  |  |  |  |  |  | Vdd |
| 11 | PGEC3 | AN5 | C1INA | RP18 | CN7 | RB5 |  | OPEN |
| 12 | PGED3 | AN4 | C1IN | RP28 | CN6 | RB4 |  | OPEN |
| 13 | AN3 | C2INA | CN5 | RB3 |  |  |  | OPEN |
| 14 | AN2 | C2INB | RP13 | CN4 | RB2 |  |  | OPEN |
| 15 | PGEC1 | AN1 | Vref- | RP1 | CN3 | RB1 |  | OPEN |
| 16 | PGED1 | AN0 | Vref+ | RP0 | CN2 | PMA6 | RB0 | VREF |
| 17 | PGEC2 | AN6 | RP6 | CN24 | RB6 |  |  | PROG_CLK |
| 18 | PGED2 | AN7 | RP7 | CN25 | RB7 |  |  | PROG_DATA |
| 19 | AVdd |  |  |  |  |  |  | AVdd |
| 20 | AVss |  |  |  |  |  |  | AVss |
| 21 | AN8 | RP8 | CN26 | RB8 |  |  |  | OPEN |
| 22 | AN9 | RP9 | CN27 | PMA7 | RB9 |  |  | OPEN |
| 23 | TMS | AN10 | CVref | CN28 | PMA13 | RB10 |  | INSTR_CURR |
| 24 | TDO | AN11 | CN29 | PMA12 | RB11 |  |  | INSTR_VOLT |
| 25 | Vss |  |  |  |  |  |  | Vss |
| 26 | Vdd |  |  |  |  |  |  | Vdd |
| 27 | TCK | AN12 | CTED2 | CN30 | PMA11 | RB12 |  | INSTR_1_CURR |
| 28 | TDI | PMA10 | AN13 | CTED1 | CN31 | RB13 |  | INSTR_2_CURR |
| 29 | AN14 | CTPLS | RP14 | CN32 | PMA1 | RB14 |  | INSTR_3_CURR |
| 30 | AN15 | REFO | RP29 | CN12 | PMA0 | RB15 |  | INSTR_4_CURR |
| 31 | SDA2 | RP10 | CN17 | PMA9 | RF4 |  |  | OPEN |
| 32 | SCL2 | RP17 | CN18 | PMA8 | RF5 |  |  | "Test LED" |
| 33 | RP16 | CN71 | RF3 |  |  |  |  | INSTR_4_STATE |
| 34 | RP30 | CN70 | RF2 |  |  |  |  | INSTR_3_STATE |
| 35 | ASCK1 | RPI45 | INT0 | CN72 | RF6 |  |  | DC-DC_CNTL |
| 36 | SDA1 | CN84 | RG3 |  |  |  |  | ADC_BUSY |
| 37 | SCL1 | CN83 | RG2 |  |  |  |  | ADC_*SHDN |
| 38 | Vdd |  |  |  |  |  |  | Vdd |
| 39 | OSCI | CLKI | CN32 | RC12 |  |  |  | OSCI |
| 40 | OSCO | CLKO | CN22 | RC15 |  |  |  | OSCO |
| 41 | Vss |  |  |  |  |  |  | Vss |
| 42 | RTCC | RP2 | CN53 | RD8 |  |  |  | UART_3_*SHDN |
| 43 | RP4 | CN54 | RD9 |  |  |  |  | UART4_TTL_TXD |
| 44 | RP3 | CN55 | PMCS2 | RD10 |  |  |  | UART4_TTL_RXD |
| 45 | RP12 | CN56 | PMCS1 | RD11 |  |  |  | OPEN |
| 46 | RP11 | CN49 | RD0 |  |  |  |  | UART1_TTL_TXD |
| 47 | SOSCI | C3IND | CN1 | RC13 |  |  |  | OPEN |
| 48 | SOSCO | C3INC | RPI37 | CN0 | RC14 |  |  | OPEN |
| 49 | RP24 | CN50 | RD1 |  |  |  |  | UART1_TTL_RXD |
| 50 | RP23 | CN51 | RD2 |  |  |  |  | UART2_TTL_TXD |
| 51 | RP22 | CN52 | PMBE | RD3 |  |  |  | UART2_TTL_RXD |
| 52 | RP25 | CN13 | PMWR | RD4 |  |  |  | UART3_TTL_TXD |
| 53 | RP20 | CN14 | PMRD | RD5 |  |  |  | UART3_TTL_RXD |
| 54 | C3INB | CN15 | RD6 |  |  |  |  | UART_1_*SHDN |
| 55 | C3INA | CN16 | RD7 |  |  |  |  | UART_2_*SHDN |
| 56 | Vcap | Vddcore |  |  |  |  |  | Vcap |
| 57 | ENVREG |  |  |  |  |  |  | ENVREG |
| 58 | CN68 | RF0 |  |  |  |  |  | INSTR_1_STATE |
| 59 | CN69 | RF1 |  |  |  |  |  | INSTR_2_STATE |
| 60 | CN58 | PMD0 | RE0 |  |  |  |  | INSTR_1_ON |
| 61 | CN59 | PMD1 | RE1 |  |  |  |  | INSTR_1_OFF |
| 62 | CN60 | PMD2 | RE2 |  |  |  |  | INSTR_2_ON |
| 63 | CN61 | PMD3 | RE3 |  |  |  |  | INSTR_2_OFF |
| 64 | CN62 | PMD4 | RE4 |  |  |  |  | INSTR_3_ON |

Table 1: Pinout of the PIC24F Microcontroller

|  |  |  | Power | Input | Output | Current |
| --- | --- | --- | --- | --- | --- | --- |
| Manuf | Series | Manufacturer PN# | (Watts) | (Volts) | (Volts) | (Amps) |
| CUI Inc | VYB10W | VYB10W-Q48-S12 | 10 | 18-72 | 12 | 0.833 |
| CUI Inc | VYB10W | VYB10W-Q48-S15 | 10 | 18-72 | 15 | 0.666 |
| CUI Inc | VYB15W | VYB15W-Q48-S12 | 15 | 18-75 | 12 | 1.25 |
| CUI Inc | VYB15W | VYB15W-Q48-S15 | 15 | 18-75 | 15 | 1 |
| CUI Inc | VYB20W | VYB20W-Q48-S12 | 20 | 18-75 | 12 | 1.667 |
| CUI Inc | VYB20W | VYB20W-Q48-S15 | 20 | 18-75 | 15 | 1.333 |
| CUI Inc | VYB20W | VYB20W-Q48-S24 | 20 | 18-75 | 24 | 0.834 |
| CUI Inc | VHB50W | VHB50W-Q48-S12 | 50 | 18-75 | 12 | 4.16 |
| CUI Inc | VHB50W | VHB50W-Q48-S15 | 50 | 18-75 | 15 | 3.33 |
| CUI Inc | VHB50W | VHB50W-Q48-S24 | 50 | 18-75 | 24 | 2.08 |
| CUI Inc | VHB50W | VHB50W-Q48-S28 | 50 | 18-75 | 28 | 1.78 |
| CUI Inc | VHB50W | VHB50W-Q48-S48 | 50 | 18-75 | 48 | 1.04 |
| CUI Inc | VHB75W | VHB75W-Q48-S12 | 75 | 18-75 | 12 | 6.25 |
| CUI Inc | VHB75W | VHB75W-Q48-S15 | 75 | 18-75 | 15 | 5 |
| CUI Inc | VHB75W | VHB75W-Q48-S24 | 75 | 18-75 | 24 | 3.12 |
| CUI Inc | VHB75W | VHB75W-Q48-S48 | 75 | 18-75 | 48 | 1.56 |

Table 2: List of available DC-DC converters

There are two families of DC-DC converters, VYB and VHB, which can be used on the SN board. Components have been selected to accommodate any voltage/power combination.

| Ref # | Color | Signal | Value |
| --- | --- | --- | --- |
| TP1 | Orange | BUS_VOLT | 12V, 15V, 24V, 28V, or 48V |
| TP2 | Red | Vin_Pos | 48V |
| TP3 | Brown | Vin_Neg | Ground |
| TP4 | White | INSTR_VOLT | 0-3V |
| TP5 | White | INSTR_CURR | 0-3V |
| TP6 | Yellow | Logic_Pwr | 3.3V |
| TP7 | Black | Ground | Ground |
| TP8 | Black | Ground | Ground |
| TP9 | Green | INSTR_1_CURR | 0-3V |
| TP10 | Blue | INSTR_2_CURR | 0-3V |
| TP11 | Grey | INSTR_3_CURR | 0-3V |
| TP12 | Purple | INSTR_4_CURR | 0-3V |

Table 3: List of Test Points

INSTR_VOLT

A 0-3V analog voltage signal which corresponds to the output voltage of the DC-DC converter. The conversion factor is Y(volts) = 16.0 x INSTR_VOLT.

INSTR_CURR

A 0-3V analog voltage signal which corresponds to the total current usage of the DC-DC converter. The conversion factor is Y(amps) = 2.0 x INSTR_CURR.

INSTR_x_CURR

A 0-3V analog voltage signal which corresponds to the current usage of the instrument being monitored. For the three standard instrument ports, the conversion factor is Y(amps) = 0.667 x INST_x_CURR(volts). For the high current port, it is Y(amps) = 2.0 x INSTR_4_CURR.

| J8-x | Signal Name |
| --- | --- |
| 1 | ADC_*SHDN |
| 2 | ADC_BUSY |
| 3 | SPI_SCLK |
| 4 | SPI_CS |
| 5 | SPI_MISO |
| 6 | SPI_MOSI |
| 7 | Ground |

Table 4: J8 - SPI Signal Test Connector

ADC_*SHDN

An active low input to the ADS8344 ADC. Pulling this line low puts the ADS8344 in a low power shutdown mode.

ADC_BUSY

An output of the ADS8344 that indicates the operating status of the ADC. Its operation is different depending on whether using internal or external clocking. It is in a high impedance state when *CS is high.

SPI_SCLK

The clock input for the SPI bus. Can also be used to control the ADC conversion steps.

SPI_*CS

The active low SPI chip select signal.

SPI_MISO

The SPI data output of the ADS8344.

SPI_MOSI

The SPI data input to the ADS8344.

| J9-x | Signal Name |
| --- | --- |
| 1 | UART_1_TTL_TXD |
| 2 | UART_1_TTL_RXD |
| 3 | UART_2_TTL_TXD |
| 4 | UART_2_TTL_RXD |
| 5 | UART_3_TTL_TXD |
| 6 | UART_3_TTL_RXD |
| 7 | UART_4_TTL_TXD |
| 8 | UART_4_TTL_RXD |
| 9 | UART_1_*SHDN |
| 10 | UART_2_*SHDN |
| 11 | UART_3_*SHDN |
| 12 | Ground |
| 13 | Ground |
| 14 | Ground |
| 15 | Ground |
| 16 | Ground |

Table 5: J9 - UART Signal Test Connector

UART_X_*SHDN

A logic low digital signal which puts the corresponding transceiver into a low power shutdown mode.

| J10-x | Signal Name |
| --- | --- |
| 1 | INSTR_1_ON |
| 2 | INSTR_1_OFF |
| 3 | INSTR_2_ON |
| 4 | INSTR_2_OFF |
| 5 | INSTR_3_ON |
| 6 | INSTR_3_OFF |
| 7 | INSTR_4_ON |
| 8 | INSTR_4_OFF |
| 9 | INSTR_1_STATE |
| 10 | INSTR_2_STATE |
| 11 | INSTR_3_STATE |
| 12 | INSTR_4_STATE |
| 13 | DC-DC_CNTL |
| 14 | Ground |
| 15 | Ground |
| 16 | Ground |

Table 6: J10 - Digital Signal Test Connector

INSTR_x_ON

A logic level positive going pulse which turns on the corresponding power relay to the instrument. The minimum pulse width is 10msec and should not exceed 50msec.

INSTR_x_OFF

A logic level positive going pulse which turns off the corresponding power relay to the instrument. The minimum pulse width is 10msec and should not exceed 50msec.

INSTR_x_STATE

A logic level feedback signal which corresponds to the state of the relay. Logic level high indicates the relay if on.

DC-DC_CNTL

An active low control signal which disables the onboard DC-DC converter. This removes power to all three instruments simultaneously.

| PIC24FJ256GA106 Pin and Coding Assignments |  |  |  |  |
| --- | --- | --- | --- | --- |
| Pin # | Func1 | Signal Type | Code (Default State) | Schematic Name |
| 1 | RE5 | Digital Output | OUT_BIT_7 (0) | INSTR_3_OFF |
| 2 | RE6 | Digital Output | OUT_BIT_4 (1) | INSTR_4_ON |
| 3 | RE7 | Digital Output | OUT_BIT_8 (1) | INSTR_4_OFF |
| 4 | RP21 | SPI |  | SPI_*CS |
| 5 | RP26 | SPI |  | SPI_SCLK |
| 6 | RP19 | SPI |  | SPI_MISO |
| 7 | *MCLR | Programming |  | *MCLR |
| 8 | RP27 | SPI |  | SPI_MOSI |
| 9 | Vss |  |  | Vss |
| 10 | Vdd |  |  | Vdd |
| 11 | RB5 | Digital Output | OUT_BIT_20 (1) | OPEN |
| 12 | RB4 | Digital Output | OUT_BIT_19 (1) | OPEN |
| 13 | RB3 | Digital Output | OUT_BIT_18 (1) | OPEN |
| 14 | RB2 | Digital Output | OUT_BIT_17 (1) | OPEN |
| 15 | RB1 | Digital Output | OUT_BIT_16 (1) | OPEN |
| 16 | Vref+ |  |  | Vref |
| 17 | PGEC2 | Programming |  | PROG_CLOCK |
| 18 | PGED2 | Programming |  | PROG_DATA |
| 19 | AVdd |  |  | AVdd |
| 20 | AVss |  |  | AVss |
| 21 | RB8 | Digital Output | OUT_BIT_21 (1) | OPEN |
| 22 | RB9 | Digital Output | OUT_BIT_22 (1) | OPEN |
| 23 | AN10 | Analog Input |  | INSTR_CURR |
| 24 | AN11 | Analog Input |  | INSTR_VOLT |
| 25 | Vss |  |  | Vss |
| 26 | Vdd |  |  | Vdd |
| 27 | AN12 | Analog Input |  | INSTR_1_CURR |
| 28 | AN13 | Analog Input |  | INSTR_2_CURR |
| 29 | AN14 | Analog Input |  | INSTR_3_CURR |
| 30 | AN15 | Analog Input |  | INSTR_4_CURR |
| 31 | RF4 | Digital Output | OUT_BIT_15 (1) | OPEN |
| 32 | RF5 | Digital Output | OUT_BIT_11 (0) | "Test LED" |
| 33 | RF3 | Digital Input | IN_BIT_3 | INSTR_4_STATE |
| 34 | RF2 | Digital Input | IN_BIT_2 | INSTR_3_STATE |
| 35 | RF6 | Digital Output | OUT_BIT_12 (1) | DC-DC_CNTL |
| 36 | RG3 | Digital Input | IN_BIT_4 | ADC_BUSY |
| 37 | RG2 | Digital Output | OUT_BIT_13 (1) | ADC_*SHDN |
| 38 | Vdd |  |  | Vdd |
| 39 | OSCI | Clock |  | OSCI |
| 40 | OSCO | Clock |  | OSCO |
| 41 | Vss |  |  | Vss |
| 42 | RD8 | Digital Output | OUT_BIT_10 (1) | UART_3_*SHDN |
| 43 | RP4 | Serial |  | UART4_TTL_TXD |
| 44 | RP3 | Serial |  | UART4_TTL_RXD |
| 45 | RD11 | Serial | OUT_BIT_14 (1) | OPEN |
| 46 | RP11 | Serial |  | UART1_TTL_TXD |
| 47 | RC13 | Digital Output | OUT_BIT_23 (1) | OPEN |
| 48 | RC14 | Digital Output | OUT_BIT_24 (1) | OPEN |
| 49 | RP24 | Serial |  | UART1_TTL_RXD |
| 50 | RP23 | Serial |  | UART2_TTL_TXD |
| 51 | RP22 | Serial |  | UART2_TTL_RXD |
| 52 | RP25 | Serial |  | UART3_TTL_TXD |
| 53 | RP20 | Serial |  | UART3_TTL_RXD |
| 54 | RD6 | Digital Output | OUT_BIT_0 (1) | UART_1_*SHDN |
| 55 | RD7 | Digital Output | OUT_BIT_9 (1) | UART_2_*SHDN |
| 56 | Vcap |  |  | Vcap |
| 57 | ENVREG |  |  | ENVREG |
| 58 | RF0 | Digital Input | IN_BIT_0 | INSTR_1_STATE |
| 59 | RF1 | Digital Input | IN_BIT_1 | INSTR_2_STATE |
| 60 | RE0 | Digital Output | OUT_BIT_1 (0) | INSTR_1_ON |
| 61 | RE1 | Digital Output | OUT_BIT_5 (0) | INSTR_1_OFF |
| 62 | RE2 | Digital Output | OUT_BIT_2 (0) | INSTR_2_ON |
| 63 | RE3 | Digital Output | OUT_BIT_6 (0) | INSTR_2_OFF |
| 64 | RE4 | Digital Output | OUT_BIT_3 (0) | INSTR_3_ON |