Kelp Forest Array Redeployment Tasks

Chad Kecy

03-27-2014

Central Node Ground Fault Troubleshooting

We have decided to repopulate the CN bulkhead connectors with the SeaConn MINK style connectors. The bulkhead connectors have been replaced; we still need to modify two cables and test the CN for ground faults using only the SeaConn connectors (no SubConns).

|  | Who | Time |
| --- | --- | --- |
| Terminate 2m whip with 2nd SeaConn connector | ET | 2 days |
| Terminate 15m whip with SeaConn connector | ET | 2 days |
| Test CN & 15 meter whip in test tank | Chad/Jim | 3 days |

Micronode Modifications

In order to accommodate the SeaConn connectors and minimize the time/work while onboard the R/V Paragon, a new endcap will be made ahead of time. The MINK FCR connectors will be installed and prewired as much as possible at MBARI.

|  | Who | Time |
| --- | --- | --- |
| Design new endcap | Farley | 2 days |
| Fabricate new endcap | Mach | 1 week |
| Populate endcap with 2 SeaConn connectors | ET | 1 day |
| Prewire the connectors | ET | 1 day |

swFOCE – Micronode Whip

The swFOCE team has decided to install a 30 meter section of whip at their output on the KFA micronode. This will ease future deployments and recoveries.

|  | Who | Time |
| --- | --- | --- |
| Order 30 meter section of cable | Chad | 1 week |
| Terminate/pot both ends of cable | ET | 2 days |

Kelp Forest Array – Redeployment

This redeployment of the KFA will be a fairly complex operation due to the work that needs to be performed on the micronode. We will also be performing a megger test of the tether while it is temporarily disconnected from the Micronode. This operation will involve the use of the R/V Paragon, multiple divers (3-4), and a technician in the pumphouse.

Proposed operation:

R/V Paragon – Chad and Tom Marion (plus captain)

Divers – George, Jamie, Paul Leary, Steve Litvin (or others depending on availability)

Pumphouse – Jim Montgomery

The Paragon will arrive onsite at the KFA mooring and will communicate with the divers from shore via phone. The Paragon will also have radio contact to Jim in the pumphouse.

The divers will deploy and recover the micronode (attached to the tether) to the Paragon

On the Paragon, we will remove the original endcap.

Once the conductors are cut, and safely taped off, Jim will perform an isolated megger test on the tether itself.

We will then install the new endcap. This will involve connectorizing the internal power conductors. The Ethernet connectors will have been pre-installed on will only need to be plugged into the Optical-to-Electrical converters in the micronode.

We will then perform another megger test with the miconode connected.

The KFA Central Node will be attached to the Micronode with a 2 meter section. The 30 meter swFOCE whip will also be attached. The KFA and swFOCE will then be energized from the pumphouse and tested for ground faults (one at a time, with system still on Paragon).

Both systems will be de-energized and deployed to the seafloor by the divers. The central node and micronode will be securely strapped together while on the Paragon (method TBD).

Once the divers have returned safely to shore, both systems will once again be energized and checked for ground faults.