NetGear 5-Port GigE Ethernet Switch

Network Ports

- 5 auto-sensing UTP ports

Forwarding Mode

- Store-and-forward

- 128 KB on-chip packet buffering

Performance

- Bandwidth: 10 Gbps (non-blocking)

- Forwarding rate: 
- 10 Mbps port: 14,800 packets/sec 
- 100 Mbps port: 148,000 packets/sec 
- 1000 Mbps port: 1,480,000 packets/sec

- Latency (using 1500-byte packets): 
- 10 Mbps: 30μs (max) 
- 100 Mbps: 6μs (max) 
- 1000 Mbps: 4μs (max)

- MAC address database: 4,000

- Mean time between failures (MTBF): 
>1 million hours (~114 years)

Status LEDs

- System power

- Link, speed, and activity indicators per port built into each RJ-45 port

AC Power

- Power adapter: 12W, 12V DC, 1A

- Maximum power consumption: 3.5W

Physical Specifications

- Dimensions (w x d x h): 94 x 104 x 28 mm (3.7 x 4.1 x 1.1 in)

- Weight: 0.30 kg (0.66 lb)

Environmental Specifications

- Operating temperature: 0º to 50º C (32º to 122º F)

- Operating humidity: 10% to 90% non-condensing

- Storage temperature: -20º to 70° C (-4º to 158º F)

- Storage humidity: 10% to 95% relative humidity

p>Standards Compliance

- IEEE 802.3i 10BASE-T Ethernet

- IEEE 802.3u 100BASE-TX Fast Ethernet

- IEEE 802.3ab 1000BASE-T Gigabit Ethernet

- Honors IEEE 802.1p and DSCP priority tags

- Jumbo frame: up to 9,720 bytes

Safety Agency Approvals

- C-Tick, CE Mark

Electromagnetic Compliance

- FCC Class B, VCCI Class B

System Requirements

- Windows®, Linux®, Mac OS®

- 10 Mbps: UTP Cat 3 (or better)

- 100 Mbps: UTP Cat 5 (or better)

- 1000 Mbps: UTP Cat 5e (or better)

- Network card for each PC or server

Warranty

- Switch: NETGEAR Lifetime Warranty††

- Power adapter: NETGEAR 2-year warranty

ProSupport Service Packs Available

OnCall 24x7, Category 1

- PMB0331-100 (US)

- PMB0331 (non-US)

XPressHW, Category 1

- PRR0331

Package Contents

- ProSafe 5-port or 8-port Gigabit Desktop Switch 10/100/1000 Mbps (GS105v3 or GS108v3)

- Wall-mount kit

- AC adapter

- Installation guide

- Warranty/support information card

NETGEAR Related Products

- Gigabit Ethernet PCI Adapter (GA311)

- Gigabit Ethernet PC Card (GA511)

- ProSafe® 16-port Gigabit Desktop Switch (GS116)

Ordering Information

- North America: GS105NA

- Europe - General: GS105GE

- UK: GS105UK

- Asia: GS105AU

- Japan: GS105A-300JPS