| PST-INV2424 | Function: DC/AC Inverter with 24VAC output |
| --- | --- |
| Country of Origin | USA, NAFTA qualified |
| Output power | 40VA continuous (40 Watts) (24VAC 1.6Amps nominal) 50VA for 1 minute at 50% duty factor( 1 minute above 40VAC, one minute below 40VA |
| Output Current | 1.6 Amps continuous, 2 amps peak |
| Input voltage | DC 22VDC to 29VDC start-up, 20VDC to 30VDC operating. |
| Output voltage | AC 24VAC |
| Output voltage regulation | ± 10% |
| Output wave form | Pure sine wave |
| Output frequency | 60Hz ± 1% (50Hz available, ask) |
| Standby current | 0.2A, 200 mA typical |
| Efficiency | 90% max. |
| Operating Temperature | -10°C to +45°C |
| Cooling | Convection cooling, no fan |
| Protections | Output short Polarity reverse Under voltage input |
| Under voltage protection | Unit will shut itself down when the input voltage is less than 20VDC ±5%. It will restart when the voltage is above 22VDC |
| DC Input | Screw terminals |
| AC output | Screw Terminals |
| Isolation | DC input is galvanically isolated from the AC output, greater than 100V hipot |
| Dimensions | 132 X 75 X 38 mm , 5.2 x 3 x 1.5 inches |
| Weight | 277 gms., 10oz, 0.6 lbs |
| Drawing | Drawing and Mounting dimensions click here |
| More options | Please ask if you need different input voltages, output voltage, output power, surge protection, connectorization, or form factor. |

# 40 Watt 24VDC to 24VAC Pure Sine Wave DC/AC Inverter for CCTV