Shallow Water FOCE Sensor Node Prototype

Analog Signal Scaling

Chad Kecy

03-11-2013

Analog Signal Descriptions

The swFOCE Sensor Node (SN) board monitors the individual load currents as well as total load current and voltage. The three primary loads can deliver upto 2 amps and the fourth, a high power port, can deliver up to the maximum 6 amps.

Primary Loads

Circuit Description

The Primary loads are defined as Instruments #1, #2, and #3. They can deliver up to 2 amps at any required bus voltage as well as serial communications to the instrument. The three circuits are identical; load #1 will be described as a reference. Refer to page 2 of the swFOCE Sensor Node Proto Board schematic; the four load paths are displayed. Each load path circuit provides three functions: load current monitoring, load switching, and load state feedback.

The main bus voltage node is split three ways to each of the Primary loads. A low ohm (0.15) sense resistor creates a sense voltage representative of that load's current draw. U7 (LT1787) is a dedicated current sense amplifier with a fixed internal gain of 8. At the maximum current draw of 2 amps, U7 will output 2.4V. The output voltage passes through a first order low pass filter consisting of U8, R17, and C24 before entering the Analog-to-Digital Converter (ADC). The voltage reference for the ADC is 3.0V. Setting the maximum output voltage to 2.4V allows for some headroom in the event of an overcurrent condition.

Power switching is provided by U6 and K1. K1 is a dual coil latching relay; it requires a 3.3V level positive going pulse on each of its coils to be activated. Its default state is in the Reset (Off) condition. U6 is a dual N-channel mosfet which is used for low side switching to the coils of K1. Pull down resistors on the gates ensure both mosfets are off by default. Positive going PIC signals INSTR_1_ON and INSTR_1_OFF turn on the mosfet which passes a ground signal to the relay coils. The pulse durations should be between 10msec and 50msec.

INSTR_1_STATE is a feedback signal indicating the state of K1. It is low when the relay is off, high when it is on.

Load Testing

The Primary load paths were load tested from the lowest expected current up to their maximum expected current. The data from these measurements is displayed in Table one. TP9 is the output of the current sensing circuit and is fed to the ADC. The conversion factor between TP9 and the actual current is also shown in the table. The nonlinear response of U7 (LT1787) is easily seen at the low current measurements. The results were plotted (Figure one) and two separate scaling factors were derived. The crossover point between the scaling factors occurs when TP9 = 0.70V.

| RL | IL | TP9 | IL/TP9 |
| --- | --- | --- | --- |
| 1000 | 0.013 | 0.037 | 0.351 |
| 900 | 0.014 | 0.037 | 0.378 |
| 800 | 0.015 | 0.038 | 0.395 |
| 775 | 0.015 | 0.038 | 0.395 |
| 750 | 0.016 | 0.038 | 0.421 |
| 725 | 0.016 | 0.038 | 0.421 |
| 700 | 0.017 | 0.039 | 0.436 |
| 675 | 0.017 | 0.039 | 0.436 |
| 650 | 0.018 | 0.039 | 0.462 |
| 625 | 0.019 | 0.039 | 0.487 |
| 600 | 0.020 | 0.040 | 0.500 |
| 575 | 0.020 | 0.040 | 0.500 |
| 550 | 0.021 | 0.040 | 0.525 |
| 525 | 0.022 | 0.041 | 0.537 |
| 500 | 0.024 | 0.042 | 0.571 |
| 475 | 0.025 | 0.042 | 0.595 |
| 450 | 0.026 | 0.043 | 0.605 |
| 425 | 0.028 | 0.044 | 0.636 |
| 400 | 0.030 | 0.045 | 0.667 |
| 375 | 0.032 | 0.046 | 0.696 |
| 350 | 0.034 | 0.048 | 0.708 |
| 325 | 0.036 | 0.049 | 0.735 |
| 300 | 0.040 | 0.052 | 0.769 |
| 275 | 0.043 | 0.054 | 0.796 |
| 250 | 0.048 | 0.059 | 0.814 |
| 225 | 0.053 | 0.064 | 0.828 |
| 200 | 0.060 | 0.072 | 0.833 |
| 190 | 0.063 | 0.075 | 0.840 |
| 180 | 0.066 | 0.079 | 0.835 |
| 170 | 0.070 | 0.083 | 0.843 |
| 160 | 0.075 | 0.090 | 0.833 |
| 150 | 0.080 | 0.096 | 0.833 |
| 140 | 0.085 | 0.102 | 0.833 |
| 130 | 0.092 | 0.111 | 0.829 |
| 120 | 0.099 | 0.119 | 0.832 |
| 110 | 0.108 | 0.129 | 0.837 |
| 100 | 0.119 | 0.143 | 0.832 |
| 90 | 0.133 | 0.160 | 0.831 |
| 80 | 0.149 | 0.179 | 0.832 |
| 70 | 0.170 | 0.204 | 0.833 |
| 60 | 0.198 | 0.239 | 0.828 |
| 50 | 0.238 | 0.286 | 0.832 |
| 40 | 0.297 | 0.357 | 0.832 |
| 30 | 0.393 | 0.473 | 0.831 |
| 20 | 0.586 | 0.705 | 0.831 |
| 10 | 1.144 | 1.376 | 0.831 |

Table one: Load testing of load path #1

Figure one: Scaling factor plot for the Primary load ports.

The scaling factors are:

For TP9 > 0.070v   y = 0.831x

For TP9 < 0.070v   y = -23.54x^2 + 3.86x - 0.097

High Current Load

Circuit Description

The high current port can deliver up to the maximum 6 amps to a load but has no serial communication available. It's circuit is shown on page 2 of the swFOCE Sensor Node Proto Board schematic. The high current load path circuit provides three functions: load current monitoring, load switching, and load state feedback.

The main bus voltage is fed directly to the current sense circuit, U16 and U17. R28 (0.05 ohm) develops a sense voltage which is representative of the load current. R32 was added to modify the fixed gain of U16 to 9.5. At the maximum current draw of 6 amps, U16 will output 2.8V. U17, R31, and C30 provide filtering prior to the ADC.

Power switching is provided by U15 and K4. K4 is a dual coil latching relay; it requires a 3.3V level positive going pulse on each of its coils to be activated. Its default state is in the Reset (off) condition. U15 is a dual P-channel mosfet used for high-side switching to K4. PIC signal INSTR_4_ON and INSTR_4_OFF are active low signals which turn on the mosfets, passing 3.3V to the relay's coils. The signals should be between 10msec and 50msec.

INSTR_4_STATE is a feedback signal indicating the state of K1; it is low when the relay is off, high when it is on.

Load Testing

The high current load path was load tested from the lowest expected current up to its maximum expected current. The data from these measurements is displayed in Table two. TP12 is the output of the current sensing circuit and is fed to the ADC. The conversion factor between TP12 and the actual current is also shown in the table. The nonlinear response of U16 (LT1787) is easily seen at the low current measurements. The results were plotted (Figure two) and two separate scaling factors were derived. The crossover point between the scaling factors occurs when TP12 = 0.70V.

| RL | IL | TP12 | IL/TP12 |
| --- | --- | --- | --- |
| 1000 | 0.013 | 0.037 | 0.35 |
| 900 | 0.013 | 0.037 | 0.35 |
| 800 | 0.015 | 0.038 | 0.39 |
| 775 | 0.015 | 0.038 | 0.39 |
| 750 | 0.016 | 0.038 | 0.42 |
| 725 | 0.016 | 0.038 | 0.42 |
| 700 | 0.017 | 0.038 | 0.45 |
| 675 | 0.017 | 0.038 | 0.45 |
| 650 | 0.018 | 0.038 | 0.47 |
| 625 | 0.019 | 0.038 | 0.50 |
| 600 | 0.020 | 0.038 | 0.53 |
| 575 | 0.020 | 0.038 | 0.53 |
| 550 | 0.021 | 0.038 | 0.55 |
| 525 | 0.022 | 0.039 | 0.56 |
| 500 | 0.024 | 0.039 | 0.62 |
| 475 | 0.025 | 0.039 | 0.64 |
| 450 | 0.026 | 0.039 | 0.67 |
| 425 | 0.028 | 0.039 | 0.72 |
| 400 | 0.030 | 0.040 | 0.75 |
| 375 | 0.032 | 0.040 | 0.80 |
| 350 | 0.034 | 0.040 | 0.85 |
| 325 | 0.037 | 0.041 | 0.90 |
| 300 | 0.040 | 0.041 | 0.98 |
| 275 | 0.043 | 0.042 | 1.02 |
| 250 | 0.048 | 0.043 | 1.12 |
| 225 | 0.053 | 0.044 | 1.20 |
| 200 | 0.060 | 0.045 | 1.33 |
| 190 | 0.063 | 0.046 | 1.37 |
| 180 | 0.066 | 0.046 | 1.43 |
| 170 | 0.070 | 0.047 | 1.49 |
| 160 | 0.075 | 0.049 | 1.53 |
| 150 | 0.080 | 0.050 | 1.60 |
| 140 | 0.085 | 0.051 | 1.67 |
| 130 | 0.092 | 0.054 | 1.70 |
| 120 | 0.100 | 0.056 | 1.79 |
| 110 | 0.109 | 0.059 | 1.85 |
| 100 | 0.119 | 0.063 | 1.89 |
| 90 | 0.133 | 0.069 | 1.93 |
| 80 | 0.148 | 0.076 | 1.95 |
| 70 | 0.170 | 0.086 | 1.98 |
| 60 | 0.198 | 0.100 | 1.98 |
| 50 | 0.238 | 0.119 | 2.00 |
| 40 | 0.298 | 0.148 | 2.01 |
| 30 | 0.396 | 0.195 | 2.03 |
| 20 | 0.590 | 0.288 | 2.05 |
| 10 | 1.161 | 0.564 | 2.06 |

Table two: Load testing of the high-current port

Figure two: Scaling factor plot for the high-current port.

The scaling factors are:

For TP12 < 0.070v   y = 2.0755x - 0.01

For TP12 > 0.070v   y = -82.33x^2 + 12.93x - 0.333

Total Load Power

Circuit Description

The total load current as well as bus voltage level can be measured for a quick power measurement. It's circuit is shown on page 1 of the swFOCE Sensor Node Proto Board schematic. The output of the DC-DC converter is fed through a sense resistor, R2 (0.05) that develops a sense voltage which is representative of the total load current. R11 was added to modify the fixed gain of U1 to 9.5. At the maximum current draw of 6 amps, U1 will output 2.8V. U3, R10, and C14 provide filtering prior to the ADC. U2 and the voltage divider of R4/R9 provide a scaled voltage representative of the bus voltage.

Load Testing

The total instrument power usage was load tested from the lowest expected current up to its maximum expected current. The data from these measurements is displayed in Table three. TP5 is the output of the current sensing circuit and is fed to the ADC. The conversion factor between TP5 and the actual current is also shown in the table. The nonlinear response of U1 (LT1787) is easily seen at the low current measurements. The results were plotted (Figure three) and two separate scaling factors were derived. The crossover point between the scaling factors occurs when TP5 = 0.70V.

The load voltage was adjusted and measured at TP4.

The scaling factor for load voltage is:

y = 16.823x - 0.008

| RL | IL | TP5 | IL/TP5 |
| --- | --- | --- | --- |
| 1000 | 0.012 | 0.036 | 0.333 |
| 900 | 0.013 | 0.036 | 0.361 |
| 800 | 0.015 | 0.037 | 0.405 |
| 775 | 0.015 | 0.037 | 0.405 |
| 750 | 0.016 | 0.037 | 0.432 |
| 725 | 0.016 | 0.037 | 0.432 |
| 700 | 0.017 | 0.037 | 0.459 |
| 675 | 0.017 | 0.037 | 0.459 |
| 650 | 0.018 | 0.037 | 0.486 |
| 625 | 0.019 | 0.037 | 0.514 |
| 600 | 0.020 | 0.037 | 0.541 |
| 575 | 0.020 | 0.037 | 0.541 |
| 550 | 0.021 | 0.037 | 0.568 |
| 525 | 0.022 | 0.038 | 0.579 |
| 500 | 0.024 | 0.038 | 0.632 |
| 475 | 0.025 | 0.038 | 0.658 |
| 450 | 0.026 | 0.038 | 0.684 |
| 425 | 0.028 | 0.038 | 0.737 |
| 400 | 0.030 | 0.039 | 0.769 |
| 375 | 0.032 | 0.039 | 0.821 |
| 350 | 0.034 | 0.039 | 0.872 |
| 325 | 0.036 | 0.040 | 0.900 |
| 300 | 0.039 | 0.040 | 0.975 |
| 275 | 0.043 | 0.041 | 1.049 |
| 250 | 0.048 | 0.042 | 1.143 |
| 225 | 0.053 | 0.043 | 1.233 |
| 200 | 0.059 | 0.044 | 1.341 |
| 190 | 0.063 | 0.045 | 1.400 |
| 180 | 0.066 | 0.046 | 1.435 |
| 170 | 0.070 | 0.046 | 1.522 |
| 160 | 0.075 | 0.048 | 1.563 |
| 150 | 0.080 | 0.049 | 1.633 |
| 140 | 0.085 | 0.051 | 1.667 |
| 130 | 0.092 | 0.053 | 1.736 |
| 120 | 0.099 | 0.055 | 1.800 |
| 110 | 0.109 | 0.058 | 1.879 |
| 100 | 0.119 | 0.062 | 1.919 |
| 90 | 0.133 | 0.068 | 1.956 |
| 80 | 0.149 | 0.074 | 2.014 |
| 70 | 0.170 | 0.084 | 2.024 |
| 60 | 0.198 | 0.097 | 2.041 |
| 50 | 0.238 | 0.116 | 2.052 |
| 40 | 0.298 | 0.144 | 2.069 |
| 30 | 0.396 | 0.189 | 2.095 |
| 20 | 0.590 | 0.280 | 2.107 |
| 10 | 1.161 | 0.547 | 2.122 |

Table three: Load testing of total instrument power.

Figure three: Scaling factor plot for the total instrument current.

The scaling factors are:

For TP5 > 0.070v   y = 2.14x - 0.01

For TP5 < 0.070v   y = -77.46x^2 +11.726x -0.309

Analog-to-Digital Conversion

For the prototype version of the SN board, the user has the option of using the built-in 10-bit ADC on the PIC or using a SPI based 16-bit ADC (U20, ADS8344).

(has not been tested yet, add more text later. CDK).