Engineering requirements for the Shallow Water FOCE System

Revision A

Electrical input - Kecy, Maughan

Software input - Headley, Herlien, Salamy, O'Reilly

Mechanical input - Shane, Scholfield

A. Electrical

1. Power Distribution

Main Tether

Power down the main tether will be delivered at +375Vdc to minimize copper losses.

The current on the +375Vdc line will not exceed 6.0 amps.

Central Node

The connection to the central node from shore will be diver-disconnectable?

The central node will provide inrush limiting on the +375Vdc line.

The central node will provide voltage and current monitoring of the +375Vdc line.

Conversion to a medium voltage (+48Vdc) will occur in a central node located outside of each SW-FOCE unit to remove the presence of high voltage in the electronics housing.

The central node will provide individually switched power for up to four SW-FOCE units at +48Vdc.

The connections from the central node to the SW-FOCE units will be diver-disconnectable.

The central node will provide +375Vdc to a second central node (the requirements of the second central node are identical to the first).

Each of the eight SW-FOCE outputs will have complete galvanic isolation when not powered up.

The +48Vdc line to each SW-FOCE unit will have voltage and current monitoring.

SW-FOCE

Each SW-FOCE unit will have inrush limiting on its +48Vdc line.

Each SW-FOCE unit will have a DC Power Chassis for downconverting to necessary DC voltages.

Each low voltage DC value will have:

- its voltage and current monitored.

- ground fault monitoring.

- all variables reporting to the GUI.

2. Instrument Control

Each external instrument attached to the SW-FOCE will have:

- switchable power

- complete galvanic isolation when powered down.

- its source voltage and load current monitored.

- a resettable circuit breaker that is controlled by following conditions:

- over current

- over voltage

- under voltage

- ground fault monitoring

- all variables reporting to the GUI

Each external instrument attached to the SW-FOCE will have its own power and communication path. This can be a cabled whip or wireless power/comms connection. The goal is to have easy diver access and servicability.

Each external instrument must have biofouling protection or be accessible for diver cleaning on a regular basis.

3. Communication

SW-FOCE

Each SW-FOCE will communicate to shore via 1Gb/s ethernet line.

The DC Power Chassis will be accessible via ethernet.

The Instrument Chassis will be accessible via ethernet.

Each SW-FOCE will provide adequate serial ports for all external instruments and internal sub-systems.

Central Node

The central node will have a 1x8 1Gb/s switch for ethernet distribution.

The medium voltage converter board will be accessible via ethernet.

4. Environmental Montioring

SW-FOCE

Each SW-FOCE will have internal environmental monitoring including:

- ambient temperature

- ambient humidity

- ambient pressure

- temperature monitoring of specific subsystems (TBD)

Each SW-FOCE will have active water presence detectors.

All environmental variables will be reported to the GUI.

Central Node

The central node will have approriate environmental monitoring including:

- ambient temperature

- ambient humidity

- temperatures of specific electronic devices (TBD later)

The central node will have active water presence detectors.

All environmental variables will be reported to the GUI.

B. Software

C. Mechanical