|  | Must | Should | May | Other | Notes |
| --- | --- | --- | --- | --- | --- |
| Engineering Design Requirements (Instruments) | Things the system does from an engineering perspective |  |  |  |  |
| ENVIRONMENTAL |  |  |  |  |  |
| Operate in depth range of 0-1000m |  |  |  |  |  |
| Operate in depth range of 0-300m |  |  |  |  |  |
| Operate in temp range of 0-25degC |  |  |  |  |  |
| Operate in presence of water currents and surges (specs TBD) |  |  |  |  |  |
| Has biolfouling protection/prevention |  |  |  |  |  |
| Instruments are diver replaceable |  |  |  |  |  |
| GENERAL OPERATION |  |  |  |  |  |
| Operate continuosly for 30 days |  |  |  |  |  |
| Operate autonomously for 30 days |  |  |  |  |  |
| Support sample and update rates up to 2Hz |  |  |  |  |  |
| Support dynamically enable/disabled instrument drivers |  |  |  |  |  |
| POWER CONTROL (per instr) |  |  |  |  |  |
| Enable power switching to instr |  |  |  |  |  |
| Monitor Instrument voltage |  |  |  |  |  |
| Monitor Instrument current |  |  |  |  |  |
| Monitor for over voltage to instr |  |  |  |  |  |
| Monitor for under voltage to instr |  |  |  |  |  |
| Monitor for over current to instr |  |  |  |  |  |
| Provide a resettable circuit breaker |  |  |  |  |  |
| Monitor for ground faults |  |  |  |  |  |
| Provide complete galvanic isolation |  |  |  |  |  |
| COMMUNICATION |  |  |  |  |  |
| Support RS-232 communication upto 115200 bps |  |  |  |  |  |

|  | Must | Should | May | Other | Notes |
| --- | --- | --- | --- | --- | --- |
| Engineering Design Requirements (System) | Things the system does from an engineering perspective |  |  |  |  |
| COMMUNICATION |  |  |  |  |  |
| Support TCP/IP protocols |  |  |  |  |  |
| Support 10BaseT ethernet |  |  |  |  |  |
| Support 100BaseT ethernet |  |  |  |  |  |
| Support 1000BaseT ethernet |  |  |  |  |  |
| Support wired communication to shore |  |  |  |  |  |
| Support wireless communication to shore |  |  |  |  |  |
| CORE SOFTWARE |  |  |  |  |  |
| Use open source software |  |  |  |  |  |
| Use low cost software |  |  |  |  |  |
| Utilize C language |  |  |  |  |  |
| Utilize Java language |  |  |  |  |  |
| Utilize BASIC language |  |  |  |  |  |
| Utilitze LabView language |  |  |  |  |  |
| Utilitze MatLab language |  |  |  |  |  |
| Utilize a visual prog language |  |  |  |  |  |
| Support scripting languages |  |  |  |  |  |
| CONTROL SYSTEM |  |  |  |  |  |
| Provide a configurable control system |  |  |  |  |  |
| Provide an in-situ configurable control system |  |  |  |  |  |
| Provide a modular control system |  |  |  |  |  |
| GENERAL OPERATION |  |  |  |  |  |
| Support hardware power management (sleep modes, etc...) |  |  |  |  |  |
| Support some form of IPC (inter-process communication) |  |  |  |  |  |
| Work with COTS processors |  |  |  |  |  |
| Provide onboard data storage |  |  |  |  |  |

|  | Must | Should | May | Other | Notes |
| --- | --- | --- | --- | --- | --- |
| Engineering Design Requirements (Power Dist) | Things the system does from an engineering perspective |  |  |  |  |
| INPUT VOLTAGE SECTION |  |  |  |  |  |
| Accept input DC volt up to 400Vdc |  |  |  |  |  |
| Accept input DC volt up to 50Vdc |  |  |  |  |  |
| Accept input DC current to x amps |  |  |  |  |  |
| Provide galvanic isolation to source |  |  |  |  |  |
| Provide inrush limiting |  |  |  |  |  |
| Be fused / cicuit breaker |  |  |  |  |  |
| LOW VOLTS DC CONVERSION |  |  |  |  |  |
| Provide multiple low volts upto 50V |  |  |  |  |  |
| Provide voltage monitoring |  |  |  |  |  |
| Provide current monitoring |  |  |  |  |  |
| Monitor for over voltage to instr |  |  |  |  |  |
| Monitor for under voltage to instr |  |  |  |  |  |
| Monitor for over current to instr |  |  |  |  |  |
| Provide a resettable circuit breaker |  |  |  |  |  |
| Monitor for ground faults |  |  |  |  |  |

|  | Must | Should | May | Other | Notes |
| --- | --- | --- | --- | --- | --- |
| Engineering Design Requirements (Environmental) | Things the system does from an engineering perspective |  |  |  |  |
| INTERNAL ELEC HOUSING |  |  |  |  |  |
| Provide ambient temp monitoring |  |  |  |  |  |
| Provide ambient humidity monitoring |  |  |  |  |  |
| Provide ambient pressure monitoring |  |  |  |  |  |
| Provide water presence detection |  |  |  |  |  |
| Provide temp monitoring of various subsystems (specs TBD) |  |  |  |  |  |

|  | Must | Should | May | Other | Notes |
| --- | --- | --- | --- | --- | --- |
| Engineering Design Requirements (Instrument Listing) | Things the system does from an engineering perspective |  |  |  |  |
| FOCE INSTRUMENTATION |  |  |  |  |  |
| Experimental pH probe |  |  |  |  |  |
| Reference pH probe |  |  |  |  |  |
| Experimental CTD |  |  |  |  |  |
| Reference CTD |  |  |  |  |  |
| Experimental O2 sensor |  |  |  |  |  |
| Reference O2 sensor |  |  |  |  |  |
| Experimental H2O velocimeter |  |  |  |  |  |
| Reference H2O velocimeter |  |  |  |  |  |
| Experimental camera / light |  |  |  |  |  |
| Water thrusters (fans+motor) |  |  |  |  |  |
| ESW INSTRUMENTATION |  |  |  |  |  |
| pH probe |  |  |  |  |  |
| Pump |  |  |  |  |  |
| Valve |  |  |  |  |  |

|  | Type | Must | Should | May | Other | Notes |
| --- | --- | --- | --- | --- | --- | --- |
| Other Requirements |  |  |  |  |  |  |
| Core instrument suite consists of pH (1 int 1ext) CTD (1 int 1 ext) |  |  |  |  |  | Number? Redundancy/Reliability? Science requirements |
| Supplemental instrument suite Velocimeter/ADCP O2 Camera Lights CO2 delivery pH |  |  |  |  |  |  |
| Core actuator set CO2 delivery system (e.g. pumps, valves) Thrusters |  |  |  |  |  | Different way of implementing e.g. w/o valves Is there a possible to have a thrusterless FOCE? |
| Supplemental actuators |  |  |  |  |  |  |
|  |  |  |  |  |  | Is FOCE set/forget autonomous |