Electrical Tasks for swFOCE/KFA Labor Estimates

Chad Kecy

07-15-2015

The following is a list of known issues with swFOCE & KFA. The projected hours are worst case scenario numbers with the KFA repair having the most uncertainty. Total labor hours are listed at the bottom.

1. Build Communications Power Supply

Background

Due to the Kelp Forest Array being recovered, the 2 Optical-to-Electrical (OE) converters no longer have power and therefore we are unable to communicate with the swFOCE experiment. I am proposing to build a temporary 12Vdc power supply using an existing housing. The 300Vdc from the KFA will power a 300:12 Vicor.

Labor

This should take me no more than 2 days to put together with some incidental ET shop time.		Kecy: 18 hours (2 days)

ET Shop: 5 hours

Parts

This should require no purchases as there are spare KFA Vicors inhouse and the preferred housing already has the correct bulkhead connectors. We can make a whip from existing cable.

2. Re-Terminate the swFOCE Cable (to the KFA micronode)

Background

The subsea cable which connects the KFA micronode to the swFOCE Central Node has corroded at the KFA end (the brass locking sleeve is compromised to the internal o-ring). The cable is 52 meters in length and is attached the Central Node, which is extremely difficult to recover, and too big to perform and underwater connection (refer to George M. for explicit details). The KFA end needs re-terminated without fully recovering the cable.

Proposed Plan

We will prepare a replacement whip onshore to replace the corroded end. Two divers and the R/V Paragon will be used for the repair. The divers will bring the damaged end of the cable as well as the tether/micronode up to the R/V Paragon. The damaged end will be cut off and the new end will be connected to the cable. Prior to applying scotch cast, the communications power supply (from #1 above) will be applied to the KFA connector on the micronode. The repaired cable will be connected to the swFOCE connector on the micronode. We will test the power/communications of the repaired cable before potting. Once satisfied, we will apply the scotch cast, using a permanent mold instead of the typical heatshrink method. A 4 hour curing time is recommended.

Labor

The preparation of the replacement whip will take me 2 hours plus 2 hours of ET Shop time. The molding of the connector to the cable will take ~ 4 hours (over the course of 2 days). The repair will be a full day on the R/V Paragon with me, Tom Marion, and a pilot. We will need someone (Kent) at the pumphouse to assist with power/comm testing.

Kecy: 16 hours

ET Shop: 16 hours

Parts

We will need a CCP connector and section of whip (already have). We will need to purchase ~$100 worth of scotchcast and a $25 mold.

3. Repair the Kelp Forest Array

Background

The Kelp Forest Array (KFA) is suffering from an extreme ground fault of (currently) unknown origin. The aluminum endcap is degraded beyond use. Troubleshooting of the KFA Central Node (currently onshore at MBARI) and other system components needs to occur.

Labor

Having only given the KFA a cursory examination, I would guess that I will need ~10 days for troubleshooting, ~2 days for actual repairs, and another 5 days for test tank testing. I will also need some ET shop time (Jim M is very familiar with the KFA). I will also be holding a review at some point with other EEs and DMO folks.

Kecy: 153 hours (17 days)

ET Shop: 45 hours (5 days)

Parts

As this is a Center for Ocean Solutions (COS) asset, the cost of any repair parts will come out of their budget.

4. Repair the Valve Manifolds

Background

There are two valve manifolds used in the swFOCE experiment. Once of them flooded in the test tank prior to deployment. The other is having a communications/feedback problem. Unfortunately the 2 valves from the flooded housing are now inoperable due to circuit board corrosion.

Proposed Plan

Kent and I need to troubleshoot the remaining valve manifold and repair as needed. With respect to the flooded housing, we will need to replace the valves. However, each manifold only needs one valve, so we can use the 2nd valve from the existing manifold.

Labor

Kent and I will need to work side-by-side (for the most part) on this task. I would say 3 days each for the troubleshooting. I will need 1 day for repair, and another 3 days (of both Kent and I) for testing. This labor estimate is for both manifolds. There will need to be some mechanical support for replumbing the valves. There will also be some incidental ET shop time (elec repairs)

Kecy: 63 hours (7 days)

ET Shop: 5 hours

Parts

It is unknown at this time as I don’t know the nature of the fault. It is conceivable that I will need to order replacement B&B communication modules ($150 total).

5. Repair the Thrusters

Background

Both thrusters (one per chamber) have been degrading in performance since deployment. I believe their current status is completely nonoperational. It is unknown whether this is a flooding issue or electrical (meaning the motor controllers in the Main Electronics Housings)

Labor

I will need to assist with troubleshooting which will most likely entail making test fixtures for shore testing at MBARI. 5 days.

Kecy: 45 hours (5 days)

ET Shop: 5 hours

Parts

Potentially may need to replace the thrusters (not sure of cost, Jim S purchased these). There may be some incidental costs associated with better sealing (potting compound, tape).

6. Repair the Flowmeters

Background

The deployed flowmeters are not returning data and the divers cannot see the external flashing LEDs. They are plugged into the Sensor Node (port configured for 4-20ma comms).

Proposed Plan

A dummy instrument meant to return a 5ma signal has been deployed in order to verify correct functionality of the Sensor Node and subsequent data chain. Testing was paused due to KFA failure.

Labor

I will assist with the troubleshooting. 3 days.

Kecy: 27 hours (3 days)

Parts

If the flowmeters are found to be defective, they will need to be replaced (not sure of cost, Jim S purchased these). There may also be some cost associated with cabling/connectorizing the replacements.

Total Labor

Kecy: 322 hours (36 days)

ET Shop: 76 hours (8.5 days)