FOCE Instrument Chassis Environmental Board ECO List

Board: 10020577 Rev A

A: 	SPI bus #1 CS needs a pullup resistor.

Add a 10k pullup resistor to U4-11. A 1206 sized resistor tied between U4-11 and U4-14 fits 	perfectly if the resistor is mounted ontop of the IC.

B: 	SPI bus #1 has MISO and MOSI reversed going into the PIC.

Lift U6-2 and U6-6 legs.

Solder wire from U6-2 raised leg to U6-6 pad.

Solder wire from U6-2 pad to U6-6 raised leg.