|  | Simulation of Gateway Node Aluminum Cap Date: Thursday, November 13, 2014 Designer: Farley Shane Study name: Gateway Node Thermal Conductivity Analysis type: Thermal(Steady state). Table of Contents Description 1 Assumptions 2 Model Information 2 Study Properties 3 Units 3 Material Properties 3 Thermal Loads 4 Contact Information 4 Mesh Information 5 Sensor Details 6 Study Results 7 Conclusion 7. |
| --- | --- |
| # Description 3/8" 6061-T6 Aluminum Plate as part of the Gateway Node Endcap |  |

# Assumptions

# Model Information Model name: Gateway Node Aluminum Cap Current Configuration: Default. Solid Bodies. Document Name and Reference; Treated As; Volumetric Properties; Document Path/Date Modified. Split Line2; Solid Body; Mass:1.56943 kg Volume:0.000581269 m^3 Density:2700 kg/m^3 Weight:15.3804 N; X:\901219.SW-FOCE\MECHANICAL\Gateway Electronics Housing\Gateway Node Aluminum Cap.SLDPRT Nov 12 11:17:59 2014.

# Study Properties Study name; Gateway Node Thermal Conductivity. Analysis type; Thermal(Steady state). Mesh type; Solid Mesh. Solver type; FFEPlus. Solution type; Steady state. Contact resistance defined?; No. Result folder; SolidWorks document (X:\901219.SW-FOCE\MECHANICAL\Gateway Electronics Housing).

# Units Unit system:; SI (MKS). Length/Displacement; mm. Temperature; Kelvin. Angular velocity; Rad/sec. Pressure/Stress; N/m^2.

# Material Properties Model Reference; Properties; Components. Name:; 6061-T6 (SS). Model type:; Linear Elastic Isotropic. Default failure criterion:; Max von Mises Stress. Thermal conductivity:; 166.9 W/(m.K). Specific heat:; 896 J/(kg.K). Mass density:; 2700 kg/m^3.; SolidBody 1(Split Line2)(Gateway Node Aluminum Cap). Curve Data:N/A.

# Thermal Loads Load name; Load Image; Load Details. Temperature-1; Entities:; 1 face(s). Temperature:; 48 Fahrenheit.. Heat Power-1; Entities:; 1 face(s). Heat Power Value:; 250 W.. Temperature-2; Entities:; 2 face(s). Temperature:; 48 Fahrenheit..

# Contact Information No Data

# Mesh Information Mesh type; Solid Mesh. Mesher Used:; Standard mesh. Automatic Transition:; Off. Include Mesh Auto Loops:; Off. Jacobian points; 4 Points. Element Size; 0.242379 in. Tolerance; 0.012119 in. Mesh Quality; High. ## Mesh Information - Details Total Nodes; 35111. Total Elements; 20506. Maximum Aspect Ratio; 12.591. % of elements with Aspect Ratio < 3; 99.8. % of elements with Aspect Ratio > 10; 0.00488. % of distorted elements(Jacobian); 0. Time to complete mesh(hh;mm;ss):; 00:00:04. Computer name:; ENIGMA.

# Sensor Details No Data

# Study Results Name; Type; Min; Max. Thermal1; TEMP: Temperature; 48 Fahrenheit Node: 1; 55.763 Fahrenheit Node: 3239. Gateway Node Aluminum Cap-Gateway Node Thermal Conductivity-Thermal-Thermal1.

# Conclusion