Shallow Water FOCE Sensor Node Board

Hardware Interface Document, v0.6

Chad Kecy

Monterey Bay Aquarium Research Institute

June 6, 2014

Table of Contents

Overview	5

System Level Description	5

Functional Description	8

DC-DC Conversion	8

Load Paths	8

PIC and Instrument Communication	9

Configuration Bits	9

Analog Signals	9

Board Population Options	11

DC-DC Converter Selection	11

Bus Voltage Trim Up/Down	12

Logic Power LED	12

Port #3 Configuration	13

Port #4 Feedback Resistor	13

Heartbeat LED	13

ADC Calibration	14

Connector Population	15

Programming and Serial Debugging	15

Board Configuration Feedback	15

Board Level Signal Descriptions	18

Analog Signals	18

SPI Bus Signals	19

Digital Output Signals	19

Digital Inputs	20

Appendices	21

Table 1: Default and user configuration options	11

Table 2: List of available DC-DC converter	12

Table 3: Jumper selection for instrument port #3	13

Table 4: Current limiting resistors for U17	13

Table 5: User configuration settings for SW1	17

Table 6: Software settings for user configuration settings	18

Figure 1: Possible Sensor Node configuration on swFOCE	6

Figure 2: Front side of Sensor Node board	7

Figure 3: Rear side of Sensor Node board	8

Figure 4: Sensor Node block diagram	11

Figure 5: PYB series (left) and the higher power VHB series (right) DC-DC converters	12

Figure 6: Output relays and connectors with trim circuit on far left (configured for trim up)	14

Figure 7: Serial transceiver circuits and connectors	15

Figure 8: User settable dip switch (SW1) and the microcontroller (U18)	16

Appendix 1: Electrical Specifications	21

Appendix 2: Mechanical Specifications	22

Appendix 3: PIC pin assignment for SN board	22

Appendix 4: PIC pin software assignment	23

Appendix 5: Bill of Materials for Sensor Node Board	24

# Overview

The Sensor Node (SN) is an open source board which is used to interface a wide range of oceanographic instruments. The SN provides individual power switching and communication to three instruments; a fourth high-current power only port can be used to drive simple devices, such as underwater lights. The SN can be configured to output 12Vdc to 48Vdc and can provide up to 75 watts of total power. The input and output communication protocol is RS-232 serial, at a maximum baud rate of 1Mbps. One of the instrument ports may also be configured to accept data from 4-20ma instruments. An onboard PIC24 microcontroller can be programmed in any manner to meet the user’s needs.

# System Level Description

The role of the SN as part of the Shallow Water Free Ocean Carbon Enrichment (swFOCE) experiment is to interface various underwater instruments such as pH probes, CTDs, PAR sensors, and ADCPs among others. The Gateway Node (GN) provides power and communication to four SNs; this configuration allows up to sixteen instruments to be attached to swFOCE. Data from the individual instruments are requested by the GN using Modbus protocol. The SN parses the requests to the appropriate instruments and routes the data back to the GN.

Figure 1: Possible Sensor Node configuration on swFOCE

Figure 2: Front side of Sensor Node board

Figure 3: Rear side of Sensor Node board

# Functional Description

## DC-DC Conversion

The SN contains a single DC-DC converter which provides a common DC voltage for the attached instruments. The board has been designed to accept two families of converters which allow the user to determine the output voltage and the maximum output power (for greater system efficiency). The input and output filters were designed to accommodate all of the available converters. A jumper selectable trim up/down potentiometer is also included. The current and voltage level of the instrument bus voltage (BUS_VOLT) are monitored and recorded. A digital signal from the PIC can disable the DC-DC converter in the event power to all of the instruments needs to be removed quickly. The logic power is provided by a separate voltage regulator which is supplied directly by the input 48Vdc.

## Load Paths

The instrument bus voltage is split into four paths: three standard instrument ports and one high-current power port. The standard instrument ports are limited to 2A of current, regardless of bus voltage. The high-current port can provide up to the maximum of 6A. The current usages of all four ports are monitored individually. Load switching to each port is accomplished via dual coil latching relays. Separate on and off pulses are used to change the relay states. Digital signals indicating the states of the individual relays are sent back to the PIC for independent confirmation. The high-current port has a different family of relay that allows for higher current usage; it can handle the maximum current at 75W for any of the chosen DC-DC converters.

The total power consumption of the four output ports should not exceed the rated power level of the installed DC-DC converter. There is no hardware monitoring of this power usage; this will need to be monitored in software.

## PIC and Instrument Communication

The SN board contains a PIC24F microcontroller with four serial UARTs. Three are used for communication to the instruments, and the fourth is for communication back to the GN. Separate one-channel transceivers are used in the event of a failure only affecting one channel. The three instrument serial transceivers may be placed into a sleep mode to reduce overhead power consumption. The third port may be configured to accept 4-20ma instruments. In this case, serial port #3 is inactive and the 4-20ma data is read through the ADC. The PIC handles all of the digital input and output signals, the four UARTs, and the spi bus. An onboard boot loader allows the operating program to be loaded remotely.

## Configuration Bits

Due to all of the possible hardware configurations of the Sensor Node board, and the fact that the board may be in a potted housing, there are eight bits which may be read back to confirm the hardware configuration. These are set by eight dip switches prior to potting.

## Analog Signals

A SPI bus based 16-bit Analog-to-Digital Converter (ADC) is used to measure the various analog signals on board. All of the voltages are scaled to 3.0 volts. The signals measured are the bus voltage and current, the individual instrument currents, and the 4-20ma signal, if used.

Figure 4: Sensor Node block diagram

# Board Population Options

The Sensor Node can be configured in a number of ways to suit the needs of the end user. The board is initially populated in a default configuration; the user can modify it accordingly. Table one displays the default population of the Sensor Node board as well as a convenient form for the user to fill out. Each of the options are described below.

|  | Default | User |
| --- | --- | --- |
| DC-DC Converter | Open |  |
| Trim Up/Down | None |  |
| R2 | Open |  |
| R6 | Open |  |
| Logic Power LED | On |  |
| R12 | Installed |  |
| Port #3 Config | Serial |  |
| R44 | Open |  |
| R45 | 0 ohms |  |
| R46 | 0 ohms |  |
| R47 | Open |  |
| R51 | 0 ohms |  |
| Port #4 Feedback |  |  |
| R33 | Open |  |
| Hearbeat LED |  |  |
| D19 | Open |  |
| ADC Calibration |  |  |
| R43 | 0 ohms |  |
| R54 | Open |  |

Table 1: Default and user configuration options

## DC-DC Converter Selection

There are two families of CUI Inc. DC-DC converters, PYB and VHB, which can be used on the SN board. External filtering and control components have been selected to accommodate any voltage/power combination.

Figure 5: PYB series (left) and the higher power VHB series (right) DC-DC converters

|  |  |  | Power | Input | Output | Current |
| --- | --- | --- | --- | --- | --- | --- |
| Manuf | Series | Manufacturer PN# | (Watts) | (Volts) | (Volts) | (Amps) |
| CUI Inc | PYB10 | PYB10-Q48-S12 | 10 | 18-75 | 12 | 0.833 |
| CUI Inc | PYB10 | PYB10-Q48-S15 | 10 | 18-75 | 15 | 0.667 |
| CUI Inc | PYB15 | PYB15-Q48-S12 | 15 | 18-75 | 12 | 1.250 |
| CUI Inc | PYB15 | PYB15-Q48-S15 | 15 | 18-75 | 15 | 1.000 |
| CUI Inc | PYB20 | PYB20-Q48-S12 | 20 | 18-75 | 12 | 1.667 |
| CUI Inc | PYB20 | PYB20-Q48-S15 | 20 | 18-75 | 15 | 1.333 |
| CUI Inc | PYB20 | PYB20-Q48-S24 | 20 | 18-75 | 24 | 0.834 |
| CUI Inc | VHB50W | VHB50W-Q48-S12 | 50 | 18-75 | 12 | 4.160 |
| CUI Inc | VHB50W | VHB50W-Q48-S15 | 50 | 18-75 | 15 | 3.330 |
| CUI Inc | VHB50W | VHB50W-Q48-S24 | 50 | 18-75 | 24 | 2.080 |
| CUI Inc | VHB50W | VHB50W-Q48-S28 | 50 | 18-75 | 28 | 1.780 |
| CUI Inc | VHB50W | VHB50W-Q48-S48 | 50 | 18-75 | 48 | 1.040 |
| CUI Inc | VHB75W | VHB75W-Q48-S12 | 75 | 18-75 | 12 | 6.250 |
| CUI Inc | VHB75W | VHB75W-Q48-S15 | 75 | 18-75 | 15 | 5.000 |
| CUI Inc | VHB75W | VHB75W-Q48-S24 | 75 | 18-75 | 24 | 3.120 |
| CUI Inc | VHB75W | VHB75W-Q48-S48 | 75 | 18-75 | 48 | 1.560 |

Table 2: List of available DC-DC converter

## Bus Voltage Trim Up/Down

The main bus voltage may be trimmed up or down based on the needs of the user. To trim up, populate R6 with a zero ohm resistor, leaving R2 open. To trim down, populate R2 with a zero ohm resistor and leave R6 open. In both cases, the potentiometer R5 adjusts the bus voltage.

## Logic Power LED

During the bench top test and debug phase it may be useful to have a power on indication. When R12 is populated with a zero ohm resistor, LED D2 indicates the presence of +3.3Vdc. Removing R12 reduces the power consumption by approximately 9mW.

## Port #3 Configuration

In place of serial communication, instrument port #3 can be configured to accept 4-20ma devices. Resistors R44-R47, and R51 are used to select between serial communication and 4-20ma signals.

|  | R44 | R45 | R46 | R47 | R51 |
| --- | --- | --- | --- | --- | --- |
| Serial | OPEN | 0 OHMS | 0 OHMS | OPEN | 0 OHMS |
| 4-20ma | 0 OHMS | OPEN | OPEN | 0 OHMS | OPEN |

Table 3: Jumper selection for instrument port #3

In addition, the user has the option of not populating certain components if one or the other port #3 options will not be used.

If the user plans on using serial communication only and never 4-20ma, the following components can be considered No Stuff: F1, R44, R47, R48, R49, R50, R52, R53, C58, C59, C60, C62, U24, and U25.

If the user plans on using 4-20ma on port #3 and never serial communication, the following components can be considered No Stuff:  R45, R46, C53, C54, C55, C56, C57, and U23.

Note on R51: Special attention must be made with respect to R51. If using serial communication, it needs to be populated with a zero ohm resistor. This prevents electrical noise from interfering with the ADC (U20). If using 4-20ma, R51 needs to be No Stuff in order to present the 4-20ma signal to the ADC.

## Port #4 Feedback Resistor

Due to the switching relay on port #4 not having a second set of poles, an opto-isolator is used to read the relay state feedback. In order to keep the forward diode current in the nominal operating range, the current limiting resistor has to match the bus voltage.

| BUS_VOLT | R33 | Watts |
| --- | --- | --- |
| 12 Vdc | 12k | 0.250 W |
| 15 Vdc | 12k | 0.250 W |
| 24 Vdc | 24k | 0.250 W |
| 28Vdc | 24k | 0.250 W |
| 48 Vdc | 47k | 0.250 W |

Table 4: Current limiting resistors for U17

## Heartbeat LED

D19 is a through-hole LED on extended legs. This allows the user to mount the LED in such a way that it is visible through any potting compound. D19 can be used during initial test, final deployment, or both.

## ADC Calibration

Channel 7 of the ADC (U20) is unused and by default is tied to ground via a zero ohm resistor. The user has the option of creating a voltage divider via R54 and R43 to calibrate the analog-to-digital hardware and software. If Vmeas is the voltage to be measured by the ADC, then:

R54 = R43 * (3.0V – Vmeas) / Vmeas))

Vmeas = 3.0V * (R43 / (R43 + R54))

For optimal use of the voltage reference, U22, the current draw should be between 100uA and 1mA. For example, R43 = 10k and R54 = 20k results in Vmeas = 1.0V and the current drawn is 100uA.

If no calibration is desired, install a zero ohm resistor for R43 and leave R54 open.

Figure 6: Output relays and connectors with trim circuit on far left (configured for trim up)

Figure 7: Serial transceiver circuits and connectors

## Connector Population

The power and signal connectors for the SN board should be populated during initial testing; this facilitates easier bench testing. If ultimately potting the SN board in an underwater housing, the connectors can be removed and the whips can be directly soldered to the board. However, depending on the user’s final deployment method, the power and signal connectors can remain populated. The power connectors (J1, J2, J3, J4, and J5) were designed to accommodate 18AWG wire. The communication connectors (J7, J8, J9, and J10) were designed to accommodate 24AWG wire. In addition, the programming connector, J6, may be removed prior to deployment, if desired.

# Programming and Serial Debugging

The PIC is programmed through J6; these signals are compatible with Microchip’s ICD programmers. See the Sensor Node PIC Test Code document for details on programming. Jumper JP1 allows the user to manually reset the PIC in the event of a programmer lockup or malfunction. Serial port #4 (J10) can be used a debug port during the testing phase.

# Board Configuration Feedback

The user configuration settings can be set into memory by using the 8 position dip switch, SW1, as indicated in table five. The 8-bit word can be read back using CONFIG_BIT_0 through CONFIG_BIT_7; these correspond to digital port B, signals 8 through 15. See table six.

Figure 8: User settable dip switch (SW1) and the microcontroller (U18)

| Watts | Voltage | Port #3 | SW1-1 | SW1-2 | SW1-3 | SW1-4 | SW1-5 | SW1-6 | SW1-7 | SW1-8 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 75 | 48 | Serial | OFF | OFF | OFF | OFF | OFF | OFF | ON | ON |
| 75 | 24 | Serial | ON | OFF | OFF | OFF | OFF | OFF | ON | ON |
| 75 | 15 | Serial | OFF | ON | OFF | OFF | OFF | OFF | ON | ON |
| 75 | 12 | Serial | ON | ON | OFF | OFF | OFF | OFF | ON | ON |
| 50 | 48 | Serial | OFF | OFF | ON | OFF | OFF | OFF | ON | ON |
| 50 | 28 | Serial | ON | OFF | ON | OFF | OFF | OFF | ON | ON |
| 50 | 24 | Serial | OFF | ON | ON | OFF | OFF | OFF | ON | ON |
| 50 | 15 | Serial | ON | ON | ON | OFF | OFF | OFF | ON | ON |
| 50 | 12 | Serial | OFF | OFF | OFF | ON | OFF | OFF | ON | ON |
| 20 | 24 | Serial | ON | OFF | OFF | ON | OFF | OFF | ON | ON |
| 20 | 15 | Serial | OFF | ON | OFF | ON | OFF | OFF | ON | ON |
| 20 | 12 | Serial | ON | ON | OFF | ON | OFF | OFF | ON | ON |
| 15 | 15 | Serial | OFF | OFF | ON | ON | OFF | OFF | ON | ON |
| 15 | 12 | Serial | ON | OFF | ON | ON | OFF | OFF | ON | ON |
| 10 | 15 | Serial | OFF | ON | ON | ON | OFF | OFF | ON | ON |
| 10 | 12 | Serial | ON | ON | ON | ON | OFF | OFF | ON | ON |
| 75 | 48 | 4-20ma | OFF | OFF | OFF | OFF | ON | OFF | ON | ON |
| 75 | 24 | 4-20ma | ON | OFF | OFF | OFF | ON | OFF | ON | ON |
| 75 | 15 | 4-20ma | OFF | ON | OFF | OFF | ON | OFF | ON | ON |
| 75 | 12 | 4-20ma | ON | ON | OFF | OFF | ON | OFF | ON | ON |
| 50 | 48 | 4-20ma | OFF | OFF | ON | OFF | ON | OFF | ON | ON |
| 50 | 28 | 4-20ma | ON | OFF | ON | OFF | ON | OFF | ON | ON |
| 50 | 24 | 4-20ma | OFF | ON | ON | OFF | ON | OFF | ON | ON |
| 50 | 15 | 4-20ma | ON | ON | ON | OFF | ON | OFF | ON | ON |
| 50 | 12 | 4-20ma | OFF | OFF | OFF | ON | ON | OFF | ON | ON |
| 20 | 24 | 4-20ma | ON | OFF | OFF | ON | ON | OFF | ON | ON |
| 20 | 15 | 4-20ma | OFF | ON | OFF | ON | ON | OFF | ON | ON |
| 20 | 12 | 4-20ma | ON | ON | OFF | ON | ON | OFF | ON | ON |
| 15 | 15 | 4-20ma | OFF | OFF | ON | ON | ON | OFF | ON | ON |
| 15 | 12 | 4-20ma | ON | OFF | ON | ON | ON | OFF | ON | ON |
| 10 | 15 | 4-20ma | OFF | ON | ON | ON | ON | OFF | ON | ON |
| 10 | 12 | 4-20ma | ON | ON | ON | ON | ON | OFF | ON | ON |

Table 5: User configuration settings for SW1

|  |  |  | B8 | B9 | B10 | B11 | B12 | B13 | B14 | B15 | Dec |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Watts | Voltage | Port #3 | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 |  |
| 75 | 48 | Serial | 1 | 1 | 1 | 1 | 1 | 1 | 0 | 0 | 63 |
| 75 | 24 | Serial | 0 | 1 | 1 | 1 | 1 | 1 | 0 | 0 | 62 |
| 75 | 15 | Serial | 1 | 0 | 1 | 1 | 1 | 1 | 0 | 0 | 61 |
| 75 | 12 | Serial | 0 | 0 | 1 | 1 | 1 | 1 | 0 | 0 | 60 |
| 50 | 48 | Serial | 1 | 1 | 0 | 1 | 1 | 1 | 0 | 0 | 59 |
| 50 | 28 | Serial | 0 | 1 | 0 | 1 | 1 | 1 | 0 | 0 | 58 |
| 50 | 24 | Serial | 1 | 0 | 0 | 1 | 1 | 1 | 0 | 0 | 57 |
| 50 | 15 | Serial | 0 | 0 | 0 | 1 | 1 | 1 | 0 | 0 | 56 |
| 50 | 12 | Serial | 1 | 1 | 1 | 0 | 1 | 1 | 0 | 0 | 55 |
| 20 | 24 | Serial | 0 | 1 | 1 | 0 | 1 | 1 | 0 | 0 | 54 |
| 20 | 15 | Serial | 1 | 0 | 1 | 0 | 1 | 1 | 0 | 0 | 53 |
| 20 | 12 | Serial | 0 | 0 | 1 | 0 | 1 | 1 | 0 | 0 | 52 |
| 15 | 15 | Serial | 1 | 1 | 0 | 0 | 1 | 1 | 0 | 0 | 51 |
| 15 | 12 | Serial | 0 | 1 | 0 | 0 | 1 | 1 | 0 | 0 | 50 |
| 10 | 15 | Serial | 1 | 0 | 0 | 0 | 1 | 1 | 0 | 0 | 49 |
| 10 | 12 | Serial | 0 | 0 | 0 | 0 | 1 | 1 | 0 | 0 | 48 |
| 75 | 48 | 4-20ma | 1 | 1 | 1 | 1 | 0 | 1 | 0 | 0 | 47 |
| 75 | 24 | 4-20ma | 0 | 1 | 1 | 1 | 0 | 1 | 0 | 0 | 46 |
| 75 | 15 | 4-20ma | 1 | 0 | 1 | 1 | 0 | 1 | 0 | 0 | 45 |
| 75 | 12 | 4-20ma | 0 | 0 | 1 | 1 | 0 | 1 | 0 | 0 | 44 |
| 50 | 48 | 4-20ma | 1 | 1 | 0 | 1 | 0 | 1 | 0 | 0 | 43 |
| 50 | 28 | 4-20ma | 0 | 1 | 0 | 1 | 0 | 1 | 0 | 0 | 42 |
| 50 | 24 | 4-20ma | 1 | 0 | 0 | 1 | 0 | 1 | 0 | 0 | 41 |
| 50 | 15 | 4-20ma | 0 | 0 | 0 | 1 | 0 | 1 | 0 | 0 | 40 |
| 50 | 12 | 4-20ma | 1 | 1 | 1 | 0 | 0 | 1 | 0 | 0 | 39 |
| 20 | 24 | 4-20ma | 0 | 1 | 1 | 0 | 0 | 1 | 0 | 0 | 38 |
| 20 | 15 | 4-20ma | 1 | 0 | 1 | 0 | 0 | 1 | 0 | 0 | 37 |
| 20 | 12 | 4-20ma | 0 | 0 | 1 | 0 | 0 | 1 | 0 | 0 | 36 |
| 15 | 15 | 4-20ma | 1 | 1 | 0 | 0 | 0 | 1 | 0 | 0 | 35 |
| 15 | 12 | 4-20ma | 0 | 1 | 0 | 0 | 0 | 1 | 0 | 0 | 34 |
| 10 | 15 | 4-20ma | 1 | 0 | 0 | 0 | 0 | 1 | 0 | 0 | 33 |
| 10 | 12 | 4-20ma | 0 | 0 | 0 | 0 | 0 | 1 | 0 | 0 | 32 |

Table 6: Software settings for user configuration settings

# Board Level Signal Descriptions

## Analog Signals

INSTR_CURR

This is a 0-3V analog voltage signal which corresponds to the total current usage of the DC-DC converter. The conversion factor is Y(amps) = 2.0 x INSTR_CURR.

INSTR_VOLT

This is a 0-3V analog voltage signal which corresponds to the output voltage of the DC-DC converter. The conversion factor is Y(volts) = 16.0 x INSTR_VOLT.

INSTR_1_CURR

This is a 0-3V analog voltage signal which corresponds to the current usage of the instrument being monitored on port #1. The conversion factor is Y(amps) = 0.667 x INST_1_CURR.

INSTR_2_CURR

This is a 0-3V analog voltage signal which corresponds to the current usage of the instrument being monitored on port #2. The conversion factor is Y(amps) = 0.667 x INST_2_CURR.

INSTR_3_CURR

This is a 0-3V analog voltage signal which corresponds to the current usage of the instrument being monitored on port #3. The conversion factor is Y(amps) = 0.667 x INST_3_CURR.

INSTR_4_CURR

This is a 0-3V analog voltage signal which corresponds to the current usage of the instrument being monitored on port #4. The conversion factor is Y(amps) = 2.0 x INST_4_CURR.

4_20MA_VOLT

This is a 0-3V analog voltage signal which corresponds to the 4-20ma circuit, if used. The conversion factor is Y(milliamps) = 6.1 x 4_20MA_VOLT + 4.0. The scaling for 4 to 20mA ends at 2.62V. A reading of 2.97 volts (22.1ma) indicates an error in the measurement.

## SPI Bus Signals

SPI_SCLK

The clock input for the SPI bus; it can also be used to control the ADC conversion steps.

SPI_MISO

The SPI data output of the ADS8344.

SPI_MOSI

The SPI data input to the ADS8344.

SPI_*CS

This is the active low SPI chip select signal.

## Digital Output Signals

ADC_*SHDN

This is an active low input to the ADS8344 ADC. Pulling this line low puts the ADS8344 in a low power shutdown mode.

UART_1_*SHDN

This is a logic low digital signal which puts the serial transceiver for port #1 into a low power shutdown mode.

UART_2_*SHDN

This is a logic low digital signal which puts the serial transceiver for port #2 into a low power shutdown mode.

UART_3_*SHDN

This is a logic low digital signal which puts the serial transceiver for port #3 into a low power shutdown mode.

INSTR_1_ON

This is a logic high level positive going pulse which turns on the power relay to the instrument on port #1. The minimum pulse width is 10msec and should not exceed 50msec.

INSTR_1_OFF

This is a logic high level positive going pulse which turns off the power relay to the instrument on port #1. The minimum pulse width is 10msec and should not exceed 50msec.

INSTR_2_ON

This is a logic high level positive going pulse which turns on the power relay to the instrument on port #2. The minimum pulse width is 10msec and should not exceed 50msec.

INSTR_2_OFF

This is a logic high level positive going pulse which turns off the power relay to the instrument on port #2. The minimum pulse width is 10msec and should not exceed 50msec.

INSTR_3_ON

This is a logic high level positive going pulse which turns on the power relay to the instrument on port #3. The minimum pulse width is 10msec and should not exceed 50msec.

INSTR_3_OFF

This is a logic high level positive going pulse which turns off the power relay to the instrument on port #3. The minimum pulse width is 10msec and should not exceed 50msec.

INSTR_4_ON

This is a logic low level positive going pulse which turns on the power relay to the instrument on port #4. The minimum pulse width is 10msec and should not exceed 50msec.

INSTR_4_OFF

This is a logic low level positive going pulse which turns off the power relay to the instrument on port #4. The minimum pulse width is 10msec and should not exceed 50msec.

DC-DC_CNTL

This is an active low control signal which disables the onboard DC-DC converter. This removes power to all of the attached instruments simultaneously.

## Digital Inputs

INSTR_1_STATE

This is a logic level feedback signal which corresponds to the state of the relay on port #1. Logic level high indicates the relay is on.

INSTR_2_STATE

This is a logic level feedback signal which corresponds to the state of the relay on port #2. Logic level high indicates the relay is on.

INSTR_3_STATE

This is a logic level feedback signal which corresponds to the state of the relay on port #3. Logic level high indicates the relay is on.

INSTR_4_STATE

This is a logic level feedback signal which corresponds to the state of the relay on port #4. Logic level high indicates the relay is on.

ADC_BUSY

This is an output of the ADS8344 that indicates the operating status of the ADC. Its operation is different depending on whether using internal or external clocking. It is in a high impedance state when *CS is high.

CONFIG_BIT_x (0-7)

These are 8 digital bits which can be read back by the user to verify the hardware configuration of the SN module. CONFIG_BIT_7 is the MSB. See tables four and five for the exact codes.

# Appendices

Appendix 1: Electrical Specifications

| Input Voltage | 48Vdc |
| --- | --- |
| No Load Conditions |  |
| Nominal Current | 29ma |
| Power Usage | 1.4W |
| Max Current Out |  |
| Channel 1-3 | 2A |
| Channel 4 | 6A |
| Max Power Out | 75W |
| Converter Effiiciency (max) | 83% |

Appendix 2: Mechanical Specifications

| Length | 5.75 in |
| --- | --- |
| Width | 4.0 in |
| Height | 1.0 in |
| Operating Temperature |  |
| Min | 32 degF |
| Max | 70 degF |
| Operating Depth | 300m |

Appendix 3: PIC pin assignment for SN board

| PIC24FJ256GA106 Pinout for swFOCE Sensor Node Board |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Pin # | Func1 | Func2 | Func3 | Func4 | Func5 | Func6 | Func7 | Functional Description |
| 1 | CN63 | PMD5 | RE5 |  |  |  |  | INSTR_3_OFF |
| 2 | SCL3 | CN64 | PMD6 | RE6 |  |  |  | INSTR_4_OFF |
| 3 | SDA3 | CN65 | PMD7 | RE7 |  |  |  | INSTR_4_ON |
| 4 | PMA5 | RP21 | C1IND | CN8 | RG6 |  |  | SPI_*CS |
| 5 | C1INC | RP26 | CN9 | PMA4 | RG7 |  |  | SPI_SCLK |
| 6 | C2IND | RP19 | CN10 | PMA3 | RG8 |  |  | SPI_MISO |
| 7 | *MCLR |  |  |  |  |  |  | *MCLR |
| 8 | C2INC | RP27 | CN11 | PMA2 | RG9 |  |  | SPI_MOSI |
| 9 | Vss |  |  |  |  |  |  | Vss |
| 10 | Vdd |  |  |  |  |  |  | Vdd |
| 11 | PGEC3 | AN5 | C1INA | RP18 | CN7 | RB5 |  | OPEN |
| 12 | PGED3 | AN4 | C1IN | RP28 | CN6 | RB4 |  | OPEN |
| 13 | AN3 | C2INA | CN5 | RB3 |  |  |  | OPEN |
| 14 | AN2 | C2INB | RP13 | CN4 | RB2 |  |  | OPEN |
| 15 | PGEC1 | AN1 | Vref- | RP1 | CN3 | RB1 |  | OPEN |
| 16 | PGED1 | AN0 | Vref+ | RP0 | CN2 | PMA6 | RB0 | OPEN |
| 17 | PGEC2 | AN6 | RP6 | CN24 | RB6 |  |  | PROG_CLK |
| 18 | PGED2 | AN7 | RP7 | CN25 | RB7 |  |  | PROG_DATA |
| 19 | AVdd |  |  |  |  |  |  | AVdd |
| 20 | AVss |  |  |  |  |  |  | AVss |
| 21 | AN8 | RP8 | CN26 | RB8 |  |  |  | CONFIG_BIT_0 |
| 22 | AN9 | RP9 | CN27 | PMA7 | RB9 |  |  | CONFIG_BIT_1 |
| 23 | TMS | AN10 | CVref | CN28 | PMA13 | RB10 |  | CONFIG_BIT_2 |
| 24 | TDO | AN11 | CN29 | PMA12 | RB11 |  |  | CONFIG_BIT_3 |
| 25 | Vss |  |  |  |  |  |  | Vss |
| 26 | Vdd |  |  |  |  |  |  | Vdd |
| 27 | TCK | AN12 | CTED2 | CN30 | PMA11 | RB12 |  | CONFIG_BIT_4 |
| 28 | TDI | PMA10 | AN13 | CTED1 | CN31 | RB13 |  | CONFIG_BIT_5 |
| 29 | AN14 | CTPLS | RP14 | CN32 | PMA1 | RB14 |  | CONFIG_BIT_6 |
| 30 | AN15 | REFO | RP29 | CN12 | PMA0 | RB15 |  | CONFIG_BIT_7 |
| 31 | SDA2 | RP10 | CN17 | PMA9 | RF4 |  |  | OPEN |
| 32 | SCL2 | RP17 | CN18 | PMA8 | RF5 |  |  | "Test LED" |
| 33 | RP16 | CN71 | RF3 |  |  |  |  | INSTR_4_STATE |
| 34 | RP30 | CN70 | RF2 |  |  |  |  | INSTR_3_STATE |
| 35 | ASCK1 | RPI45 | INT0 | CN72 | RF6 |  |  | DC-DC_CNTL |
| 36 | SDA1 | CN84 | RG3 |  |  |  |  | ADC_BUSY |
| 37 | SCL1 | CN83 | RG2 |  |  |  |  | ADC_*SHDN |
| 38 | Vdd |  |  |  |  |  |  | Vdd |
| 39 | OSCI | CLKI | CN32 | RC12 |  |  |  | OSCI |
| 40 | OSCO | CLKO | CN22 | RC15 |  |  |  | OSCO |
| 41 | Vss |  |  |  |  |  |  | Vss |
| 42 | RTCC | RP2 | CN53 | RD8 |  |  |  | UART_3_*SHDN |
| 43 | RP4 | CN54 | RD9 |  |  |  |  | UART4_TTL_TXD |
| 44 | RP3 | CN55 | PMCS2 | RD10 |  |  |  | UART4_TTL_RXD |
| 45 | RP12 | CN56 | PMCS1 | RD11 |  |  |  | OPEN |
| 46 | RP11 | CN49 | RD0 |  |  |  |  | UART1_TTL_TXD |
| 47 | SOSCI | C3IND | CN1 | RC13 |  |  |  | OPEN |
| 48 | SOSCO | C3INC | RPI37 | CN0 | RC14 |  |  | OPEN |
| 49 | RP24 | CN50 | RD1 |  |  |  |  | UART1_TTL_RXD |
| 50 | RP23 | CN51 | RD2 |  |  |  |  | UART2_TTL_TXD |
| 51 | RP22 | CN52 | PMBE | RD3 |  |  |  | UART2_TTL_RXD |
| 52 | RP25 | CN13 | PMWR | RD4 |  |  |  | UART3_TTL_TXD |
| 53 | RP20 | CN14 | PMRD | RD5 |  |  |  | UART3_TTL_RXD |
| 54 | C3INB | CN15 | RD6 |  |  |  |  | UART_1_*SHDN |
| 55 | C3INA | CN16 | RD7 |  |  |  |  | UART_2_*SHDN |
| 56 | Vcap | Vddcore |  |  |  |  |  | Vcap |
| 57 | ENVREG |  |  |  |  |  |  | ENVREG |
| 58 | CN68 | RF0 |  |  |  |  |  | INSTR_1_STATE |
| 59 | CN69 | RF1 |  |  |  |  |  | INSTR_2_STATE |
| 60 | CN58 | PMD0 | RE0 |  |  |  |  | INSTR_1_ON |
| 61 | CN59 | PMD1 | RE1 |  |  |  |  | INSTR_1_OFF |
| 62 | CN60 | PMD2 | RE2 |  |  |  |  | INSTR_2_ON |
| 63 | CN61 | PMD3 | RE3 |  |  |  |  | INSTR_2_OFF |
| 64 | CN62 | PMD4 | RE4 |  |  |  |  | INSTR_3_ON |

Appendix 4: PIC pin software assignment

| PIC24FJ256GA106 Pin and Coding Assignments |  |  |  |  |
| --- | --- | --- | --- | --- |
| Pin # | Func1 | Signal Type | Default State | Schematic Name |
| 1 | RE5 | Digital Output | LOW | INSTR_3_OFF |
| 2 | RE6 | Digital Output | HIGH | INSTR_4_OFF |
| 3 | RE7 | Digital Output | HIGH | INSTR_4_ON |
| 4 | RG6 | SPI | HIGH | SPI_*CS |
| 5 | RP26 | SPI |  | SPI_SCLK |
| 6 | RP19 | SPI |  | SPI_MISO |
| 7 | *MCLR | Programming |  | *MCLR |
| 8 | RP27 | SPI |  | SPI_MOSI |
| 9 | Vss |  |  | Vss |
| 10 | Vdd |  |  | Vdd |
| 11 | RB5 | Digital Output | HIGH | OPEN |
| 12 | RB4 | Digital Output | HIGH | OPEN |
| 13 | RB3 | Digital Output | HIGH | OPEN |
| 14 | RB2 | Digital Output | HIGH | OPEN |
| 15 | RB1 | Digital Output | HIGH | OPEN |
| 16 | RB0 | Digital Output | HIGH | OPEN |
| 17 | PGEC2 | Programming |  | PROG_CLOCK |
| 18 | PGED2 | Programming |  | PROG_DATA |
| 19 | AVdd |  |  | AVdd |
| 20 | AVss |  |  | AVss |
| 21 | RB8 | Digital Input |  | CONFIG_BIT_0 |
| 22 | RB9 | Digital Input |  | CONFIG_BIT_1 |
| 23 | RB10 | Digital Input |  | CONFIG_BIT_2 |
| 24 | RB11 | Digital Input |  | CONFIG_BIT_3 |
| 25 | Vss |  |  | Vss |
| 26 | Vdd |  |  | Vdd |
| 27 | RB12 | Digital Input |  | CONFIG_BIT_4 |
| 28 | RB13 | Digital Input |  | CONFIG_BIT_5 |
| 29 | RB14 | Digital Input |  | CONFIG_BIT_6 |
| 30 | RB15 | Digital Input |  | CONFIG_BIT_7 |
| 31 | RF4 | Digital Output | HIGH | OPEN |
| 32 | RF5 | Digital Output | LOW | "Test LED" |
| 33 | RF3 | Digital Input |  | INSTR_4_STATE |
| 34 | RF2 | Digital Input |  | INSTR_3_STATE |
| 35 | RF6 | Digital Output | HIGH | DC-DC_CNTL |
| 36 | RG3 | Digital Input |  | ADC_BUSY |
| 37 | RG2 | Digital Output | HIGH | ADC_*SHDN |
| 38 | Vdd |  |  | Vdd |
| 39 | OSCI | Clock |  | OSCI |
| 40 | OSCO | Clock |  | OSCO |
| 41 | Vss |  |  | Vss |
| 42 | RD8 | Digital Output | HIGH | UART_3_*SHDN |
| 43 | RP4 | Serial |  | UART4_TTL_TXD |
| 44 | RP3 | Serial |  | UART4_TTL_RXD |
| 45 | RD11 | Serial | HIGH | OPEN |
| 46 | RP11 | Serial |  | UART1_TTL_TXD |
| 47 | RC13 | Digital Output | HIGH | OPEN |
| 48 | RC14 | Digital Output | HIGH | OPEN |
| 49 | RP24 | Serial |  | UART1_TTL_RXD |
| 50 | RP23 | Serial |  | UART2_TTL_TXD |
| 51 | RP22 | Serial |  | UART2_TTL_RXD |
| 52 | RP25 | Serial |  | UART3_TTL_TXD |
| 53 | RP20 | Serial |  | UART3_TTL_RXD |
| 54 | RD6 | Digital Output | HIGH | UART_1_*SHDN |
| 55 | RD7 | Digital Output | HIGH | UART_2_*SHDN |
| 56 | Vcap |  |  | Vcap |
| 57 | ENVREG |  |  | ENVREG |
| 58 | RF0 | Digital Input |  | INSTR_1_STATE |
| 59 | RF1 | Digital Input |  | INSTR_2_STATE |
| 60 | RE0 | Digital Output | LOW | INSTR_1_ON |
| 61 | RE1 | Digital Output | LOW | INSTR_1_OFF |
| 62 | RE2 | Digital Output | LOW | INSTR_2_ON |
| 63 | RE3 | Digital Output | LOW | INSTR_2_OFF |
| 64 | RE4 | Digital Output | LOW | INSTR_3_ON |

Appendix 5: Bill of Materials for Sensor Node Board

| Ref | Value | Manufacturer | Manufacturer PN | Description |
| --- | --- | --- | --- | --- |
| C1 | 1000pF | Kemet | PME295RB4100MR30 | CAP FILM 1000PF 480VAC RADIAL |
| C2 | 1000pF | Kemet | PME295RB4100MR30 | CAP FILM 1000PF 480VAC RADIAL |
| C3 | 1.0uF | TDK Corporation | CKG57KX7R2A105M | CAP CER 1UF 100V 20% X7R SMD |
| C4 | 22uF | Vishay Sprague | 597D226X9063F2T | CAP TANT 22UF 63V 10% 2924 |
| C5 | 22uF | Vishay Sprague | 597D226X9063F2T | CAP TANT 22UF 63V 10% 2924 |
| C6 | 100uF | Panasonic | EEV-FK2A101M | CAP ALUM 100UF 100V 20% SMD |
| C7 | 0.1uF | Kemet | C1206F104K3RACTU | CAP CER .1UF 25V X7R 1206 FO |
| C8 | 1.0uF | TDK Corporation | CKG32KX7R1H105M335AH | CAP CER 1UF 50V 20% X7R SMD |
| C9 | 10uF | Kemet | T521X106M063ATE050 | CAP TANT 10UF 63V 20% 2917 |
| C10 | 10uF | Kemet | T521X106M063ATE050 | CAP TANT 10UF 63V 20% 2917 |
| C11 | 22uF | AVX Corporation | TPME226K050R0075 | CAP TANT 22UF 50V 10% 2917 |
| C12 | 22uF | AVX Corporation | TPME226K050R0075 | CAP TANT 22UF 50V 10% 2917 |
| C13 | 1.0uF | TDK Corporation | CKG57KX7R2A105M | CAP CER 1UF 100V 20% X7R SMD |
| C14 | 0.033uF | Yageo | CC1206KRX7R9BB333 | CAP CER 0.033UF 50V 10% X7R 1206 |
| C15 | 1000pF | Kemet | PME295RB4100MR30 | CAP FILM 1000PF 480VAC RADIAL |
| C16 | 1000pF | Kemet | PME295RB4100MR30 | CAP FILM 1000PF 480VAC RADIAL |
| C17 | 0.033uF | Yageo | CC1206KRX7R9BB333 | CAP CER 0.033UF 50V 10% X7R 1206 |
| C18 | 0.1uF | Kemet | C1206F104K3RACTU | CAP CER .1UF 25V X7R 1206 FO |
| C19 | 47uF | Panasonic | EEU-EB1J470S | CAP ALUM 47UF 63V 20% RADIAL |
| C20 | 33uF | AVX Corporation | TPSC336K010R0150 | CAP TANT 33UF 10V 10% 2312 |
| C21 | 0.1uF | Kemet | C1206F104K3RACTU | CAP CER .1UF 25V X7R 1206 FO |
| C22 | 0.1uF | Kemet | C1206F104K3RACTU | CAP CER .1UF 25V X7R 1206 FO |
| C23 | 0.1uF | Kemet | C1206F104K3RACTU | CAP CER .1UF 25V X7R 1206 FO |
| C24 | 0.1uF | Kemet | C1206F104K3RACTU | CAP CER .1UF 25V X7R 1206 FO |
| C25 | 0.033uF | Yageo | CC1206KRX7R9BB333 | CAP CER 0.033UF 50V 10% X7R 1206 |
| C26 | 0.1uF | Kemet | C1206F104K3RACTU | CAP CER .1UF 25V X7R 1206 FO |
| C27 | 0.033uF | Yageo | CC1206KRX7R9BB333 | CAP CER 0.033UF 50V 10% X7R 1206 |
| C28 | 0.1uF | Kemet | C1206F104K3RACTU | CAP CER .1UF 25V X7R 1206 FO |
| C29 | 0.033uF | Yageo | CC1206KRX7R9BB333 | CAP CER 0.033UF 50V 10% X7R 1206 |
| C30 | 0.1uF | Kemet | C1206F104K3RACTU | CAP CER .1UF 25V X7R 1206 FO |
| C31 | 0.033uF | Yageo | CC1206KRX7R9BB333 | CAP CER 0.033UF 50V 10% X7R 1206 |
| C32 | 0.1uF | Kemet | C1206F104K3RACTU | CAP CER .1UF 25V X7R 1206 FO |
| C33 | 0.1uF | Kemet | C1206F104K3RACTU | CAP CER .1UF 25V X7R 1206 FO |
| C34 | 0.1uF | Kemet | C1206F104K3RACTU | CAP CER .1UF 25V X7R 1206 FO |
| C35 | 0.1uF | Kemet | C1206F104K3RACTU | CAP CER .1UF 25V X7R 1206 FO |
| C36 | 10uF | AVX Corp | TPSD106K035R0125 | CAP TANT LOESR 10UF 35V 10% SMD |
| C37 | 0.1uF | Kemet | C1206F104K3RACTU | CAP CER .1UF 25V X7R 1206 FO |
| C38 | 15pF | Kemet | C0805C150J5GACTU | CAP CERAMIC 15PF 50V NP0 0805 |
| C39 | 15pF | Kemet | C0805C150J5GACTU | CAP CERAMIC 15PF 50V NP0 0805 |
| C40 | 0.1uF | Kemet | C1206F104K3RACTU | CAP CER .1UF 25V X7R 1206 FO |
| C41 | 0.1uF | Kemet | C1206F104K3RACTU | CAP CER .1UF 25V X7R 1206 FO |
| C42 | 0.1uF | Kemet | C1206F104K3RACTU | CAP CER .1UF 25V X7R 1206 FO |
| C43 | 0.1uF | AVX Corp | TAJA104K035RNJ | CAP TANTALUM .10UF 35V 10% SMD |
| C44 | 0.1uF | AVX Corp | TAJA104K035RNJ | CAP TANTALUM .10UF 35V 10% SMD |
| C45 | 0.1uF | Kemet | C1206F104K3RACTU | CAP CER .1UF 25V X7R 1206 FO |
| C46 | 0.1uF | Kemet | C1206F104K3RACTU | CAP CER .1UF 25V X7R 1206 FO |
| C47 | 0.1uF | Kemet | C1206F104K3RACTU | CAP CER .1UF 25V X7R 1206 FO |
| C48 | 0.1uF | Kemet | C1206F104K3RACTU | CAP CER .1UF 25V X7R 1206 FO |
| C49 | 4.7uF | Vishay Sprague | 593D475X9050D2TE3 | CAP TANT 4.7UF 50V 10% 2917 |
| C50 | 1.0uF | TDK Corporation | C3216X7R2A105K | CAP CER 1.0UF 100V X7R 1206 |
| C51 | 0.1uF | AVX Corp | TAJA104K035RNJ | CAP TANTALUM .10UF 35V 10% SMD |
| C52 | 0.1uF | AVX Corp | TAJA104K035RNJ | CAP TANTALUM .10UF 35V 10% SMD |
| C53 | 0.1uF | Kemet | C1206F104K3RACTU | CAP CER .1UF 25V X7R 1206 FO |
| C54 | 0.1uF | Kemet | C1206F104K3RACTU | CAP CER .1UF 25V X7R 1206 FO |
| C55 | 0.1uF | Kemet | C1206F104K3RACTU | CAP CER .1UF 25V X7R 1206 FO |
| C56 | 0.1uF | AVX Corp | TAJA104K035RNJ | CAP TANTALUM .10UF 35V 10% SMD |
| C57 | 0.1uF | AVX Corp | TAJA104K035RNJ | CAP TANTALUM .10UF 35V 10% SMD |
| C58 | 0.1uF | Kemet | C1206F104K3RACTU | CAP CER .1UF 25V X7R 1206 FO |
| C59 | 0.1uF | Kemet | C1206F104K3RACTU | CAP CER .1UF 25V X7R 1206 FO |
| C60 | 1.0uF | Kemet | C1206C105K3RAC | CAP, CER, 1.0uF, 25V, 10%, X7R, 1206 |
| C61 | 0.1uF | Kemet | C1206F104K3RACTU | CAP CER .1UF 25V X7R 1206 FO |
| C62 | 10uF | AVX Corp | TPSD106K035R0125 | CAP TANT LOESR 10UF 35V 10% SMD |
| C63 | 0.1uF | Kemet | C1206F104K3RACTU | CAP CER .1UF 25V X7R 1206 FO |
| Ref | Value | Manufacturer | Manufacturer PN | Description |
| C64 | 0.1uF | Kemet | C1206F104K3RACTU | CAP CER .1UF 25V X7R 1206 FO |
| C65 | 0.1uF | AVX Corp | TAJA104K035RNJ | CAP TANTALUM .10UF 35V 10% SMD |
| C66 | 0.1uF | AVX Corp | TAJA104K035RNJ | CAP TANTALUM .10UF 35V 10% SMD |
| D1 | B360A-13-F | Diodes Inc | B360A-13-F | DIODE SCHOTTKY 60V 3A SMA |
| D2 | LTST-C150GKT | Lite-On Inc | LTST-C150GKT | LED GREEN CLEAR 1206 SMD |
| D3 | BZX384 | NXP | BZX384-C3V3,115 | DIODE ZENER 3.3V 300MW SOD323 |
| D4 | DB2J31700L | Panasonic | DB2J31700L | DIODE 30V 1A SMINI2 |
| D5 | BZX384 | NXP | BZX384-C3V3,115 | DIODE ZENER 3.3V 300MW SOD323 |
| D6 | DB2J31700L | Panasonic | DB2J31700L | DIODE 30V 1A SMINI2 |
| D7 | BZX384 | NXP | BZX384-C3V3,115 | DIODE ZENER 3.3V 300MW SOD323 |
| D8 | DB2J31700L | Panasonic | DB2J31700L | DIODE 30V 1A SMINI2 |
| D9 | BZX384 | NXP | BZX384-C3V3,115 | DIODE ZENER 3.3V 300MW SOD323 |
| D10 | DB2J31700L | Panasonic | DB2J31700L | DIODE 30V 1A SMINI2 |
| D11 | BZX384 | NXP | BZX384-C3V3,115 | DIODE ZENER 3.3V 300MW SOD323 |
| D12 | DB2J31700L | Panasonic | DB2J31700L | DIODE 30V 1A SMINI2 |
| D13 | BZX384 | NXP | BZX384-C3V3,115 | DIODE ZENER 3.3V 300MW SOD323 |
| D14 | DB2J31700L | Panasonic | DB2J31700L | DIODE 30V 1A SMINI2 |
| D15 | DB2J31700L | Panasonic | DB2J31700L | DIODE 30V 1A SMINI2 |
| D16 | BZX384 | NXP | BZX384-C3V3,115 | DIODE ZENER 3.3V 300MW SOD323 |
| D17 | DB2J31700L | Panasonic | DB2J31700L | DIODE 30V 1A SMINI2 |
| D18 | BZX384 | NXP | BZX384-C3V3,115 | DIODE ZENER 3.3V 300MW SOD323 |
| D19 | WP710A10 | Kingbright | WP710A10SEC/J4 | 3MM ORANGE LED ROUND |
| F1 | 0154.125DR | Littelfuse Inc | 0154.125DR | FUSEBLOCK W/.125A FUSE SMD FST |
| J1 | 1881558 | Phoenix Contact | 1881558 | CONN TERM BLK HDR 2.5MM 2POS |
| J2 | 1881558 | Phoenix Contact | 1881558 | CONN TERM BLK HDR 2.5MM 2POS |
| J3 | 1881558 | Phoenix Contact | 1881558 | CONN TERM BLK HDR 2.5MM 2POS |
| J4 | 1881558 | Phoenix Contact | 1881558 | CONN TERM BLK HDR 2.5MM 2POS |
| J5 | 1803426 | Phoenix Contact | 1803426 | CONN HEADER VERT 2POS 3.81MM |
| J6 | 87831-0620 | Molex | 87831-0620 | CONN HEADER 6POS 2MM VERT GOLD |
| J7 | 70543-0001 | Molex Inc | 70543-0001 | CONN HEADER 2POS .100 VERT GOLD |
| J8 | 70543-0001 | Molex Inc | 70543-0001 | CONN HEADER 2POS .100 VERT GOLD |
| J9 | 70543-0001 | Molex Inc | 70543-0001 | CONN HEADER 2POS .100 VERT GOLD |
| J10 | 70543-0001 | Molex Inc | 70543-0001 | CONN HEADER 2POS .100 VERT GOLD |
| JP1 | 87224-2 | Tyco Electronics | 87224-2 | CONN HEADER VERT .100 2POS 15AU |
| K1 | TX2-LT-3V-TH | Panasonic | TX2-LT-3V-TH | RELAY GENERAL PURPOSE DPDT 2A 3V |
| K2 | TX2-LT-3V-TH | Panasonic | TX2-LT-3V-TH | RELAY GENERAL PURPOSE DPDT 2A 3V |
| K3 | TX2-LT-3V-TH | Panasonic | TX2-LT-3V-TH | RELAY GENERAL PURPOSE DPDT 2A 3V |
| K4 | DK2a-L2-3V | Panasonic | DK2a-L2-3V | 2 Form A, 3VDC 2 Form A 10A 250VAC |
| L1 | 22uH | Taiyo Yuden | NR10050T220M | INDUCTOR 22UH 2.6A 20% SMD |
| L2 | 15uH | Wurth | 7447709150 | INDUCTOR POWER 15UH 6.5A SMD |
| L3 | 1mH | API Delevan Inc | CM6460R-105 | CHOKE COMMON MODE 1000.0UH SMD |
| L4 | 22uH | Taiyo Yuden | NR10050T220M | INDUCTOR 22UH 2.6A 20% SMD |
| L5 | 68uH | EPCOS Inc | B82464G4683M | INDUCTOR POWER 68UH 1.3A SMD |
| MOD1 | VHB | CUI Inc |  | CONVERTER DC/DC 50W, 75W |
| MOD2 | PYB | CUI Inc |  | CONVERTER DC/DC 10W, 15W, 20W |
| R1 | 0.05 | Stackpole | CSRN2512FK50L0 | RES .05 OHM 2W 1% 2512 SMD |
| R2 | TBD |  |  | TO BE DETERMINED, HEIGHT 0.6mm |
| R3 | 158.0K | Vishay | CRCW1206158KFKEA | RES 158K OHM 1/4W 1206 SMD |
| R4 | 5k | Vishay Dale | ALSR015K000JE12 | RES 5.0K OHM 5% WW AXIAL |
| R5 | 1M | Bourns Inc. | 3299Y-1-105LF | TRIMMER 1M OHM 0.5W PC PIN |
| R6 | TBD |  |  | TO BE DETERMINED, HEIGHT 0.6mm |
| R7 | 475 | Vishay | CRCW1206475RFKEA | RES 475 OHM 1/4W 1206 SMD |
| R8 | 10.0K | Vishay | CRCW120610K0FKEA | RES 10K OHM 1/4W 1206 SMD |
| R9 | 10.0K | Vishay | CRCW120610K0FKEA | RES 10K OHM 1/4W 1206 SMD |
| R10 | 475 | Vishay | CRCW1206475RFKEA | RES 475 OHM 1/4W 1206 SMD |
| R11 | 23.7K | Vishay | CRCW120623K7FKEA | RES 23.7K OHM 1/4W 1206 SMD |
| R12 | 0 | Vishay/Dale | CRCW12060000Z0EA | RES 0.0 OHM 1/4W 1206 SMD |
| R13 | 120 | Vishay | CRCW1206120RJNEA | RES 120 OHM 1/4W 1206 SMD |
| R14 | 10.0K | Vishay | CRCW120610K0FKEA | RES 10K OHM 1/4W 1206 SMD |
| R15 | 0.15 | Stackpole | CSRN2010FKR150 | RES .15OHM 1W 1% 2010 SMD |
| R16 | 10.0K | Vishay | CRCW120610K0FKEA | RES 10K OHM 1/4W 1206 SMD |
| R17 | 10.0K | Vishay | CRCW120610K0FKEA | RES 10K OHM 1/4W 1206 SMD |
| R18 | 475 | Vishay | CRCW1206475RFKEA | RES 475 OHM 1/4W 1206 SMD |
| R19 | 10.0K | Vishay | CRCW120610K0FKEA | RES 10K OHM 1/4W 1206 SMD |
| R20 | 0.15 | Stackpole | CSRN2010FKR150 | RES .15OHM 1W 1% 2010 SMD |
| R21 | 10.0K | Vishay | CRCW120610K0FKEA | RES 10K OHM 1/4W 1206 SMD |
| Ref | Value | Manufacturer | Manufacturer PN | Description |
| R22 | 10.0K | Vishay | CRCW120610K0FKEA | RES 10K OHM 1/4W 1206 SMD |
| R23 | 475 | Vishay | CRCW1206475RFKEA | RES 475 OHM 1/4W 1206 SMD |
| R24 | 10.0K | Vishay | CRCW120610K0FKEA | RES 10K OHM 1/4W 1206 SMD |
| R25 | 0.15 | Stackpole | CSRN2010FKR150 | RES .15OHM 1W 1% 2010 SMD |
| R26 | 10.0K | Vishay | CRCW120610K0FKEA | RES 10K OHM 1/4W 1206 SMD |
| R27 | 10.0K | Vishay | CRCW120610K0FKEA | RES 10K OHM 1/4W 1206 SMD |
| R28 | 475 | Vishay | CRCW1206475RFKEA | RES 475 OHM 1/4W 1206 SMD |
| R29 | 0.05 | Stackpole | CSRN2512FK50L0 | RES .05 OHM 2W 1% 2512 SMD |
| R30 | 10.0K | Vishay | CRCW120610K0FKEA | RES 10K OHM 1/4W 1206 SMD |
| R31 | 10.0K | Vishay | CRCW120610K0FKEA | RES 10K OHM 1/4W 1206 SMD |
| R32 | 475 | Vishay | CRCW1206475RFKEA | RES 475 OHM 1/4W 1206 SMD |
| R33 | TBD |  |  | TO BE DETERMINED, HEIGHT 0.6mm |
| R34 | 23.7K | Vishay | CRCW120623K7FKEA | RES 23.7K OHM 1/4W 1206 SMD |
| R35 | 10.0K | Vishay | CRCW120610K0FKEA | RES 10K OHM 1/4W 1206 SMD |
| R36 | 10.0K | Vishay | CRCW120610K0FKEA | RES 10K OHM 1/4W 1206 SMD |
| R37 | 39 | Vishay | CRCW120639R0JNEA | RES 39 OHM 1/4W 1206 SMD |
| R38 | 10.0K | Vishay | CRCW120610K0FKEA | RES 10K OHM 1/4W 1206 SMD |
| R39 | 10.0K | Vishay | CRCW120610K0FKEA | RES 10K OHM 1/4W 1206 SMD |
| R40 | 10.0K | Vishay | CRCW120610K0FKEA | RES 10K OHM 1/4W 1206 SMD |
| R41 | 10.0K | Vishay | CRCW120610K0FKEA | RES 10K OHM 1/4W 1206 SMD |
| R42 | 10.0K | Vishay | CRCW120610K0FKEA | RES 10K OHM 1/4W 1206 SMD |
| R43 | 0 | Vishay/Dale | CRCW12060000Z0EA | RES 0.0 OHM 1/4W 1206 SMD |
| R44 | TBD |  |  | TO BE DETERMINED, HEIGHT 0.6mm |
| R45 | TBD |  |  | TO BE DETERMINED, HEIGHT 0.6mm |
| R46 | TBD |  |  | TO BE DETERMINED, HEIGHT 0.6mm |
| R47 | TBD |  |  | TO BE DETERMINED, HEIGHT 0.6mm |
| R48 | 31.6K | Vishay | CRCW120631K6FKEA | RES 31.6K OHM 1/4W 1206 SMD |
| R49 | 316 | Vishay | CRCW1206316RFKEA | RES 316 OHM 1/4W 1206 SMD |
| R50 | 16.5K | Vishay | CRCW120616K5FKEA | RES 16.5K OHM 1/4W 1206 SMD |
| R51 | TBD |  |  | TO BE DETERMINED, HEIGHT 0.6mm |
| R52 | 31.6K | Vishay | CRCW120631K6FKEA | RES 31.6K OHM 1/4W 1206 SMD |
| R53 | 16.5K | Vishay | CRCW120616K5FKEA | RES 16.5K OHM 1/4W 1206 SMD |
| R54 | TBD |  |  | TO BE DETERMINED, HEIGHT 0.6mm |
| RN1 | 33k | Bourns Inc. | 4816P-T01-333LF | RES ARRAY 33K OHM 8 RES 16SOIC |
| ST1 | #4 Screw |  |  | #4 SCREW TERMINAL W/ WASHER |
| ST2 | #4 Screw |  |  | #4 SCREW TERMINAL W/ WASHER |
| ST3 | #4 Screw |  |  | #4 SCREW TERMINAL W/ WASHER |
| ST4 | #4 Screw |  |  | #4 SCREW TERMINAL W/ WASHER |
| SW1 | TDA08 | C&K | TDA08H0SB1 | SWITCH DIP 8POS HALF PITCH SMD |
| U1 | AD8603 | Analog Devices | AD8603AUJZ-REEL7 | IC OPAMP GP R-R CMOS TSOT23-5 |
| U2 | LT1787 | Linear Technology | LT1787HVIS8#PBF | IC AMP PREC CURR SENSE HV 8-SOIC |
| U3 | AD8603 | Analog Devices | AD8603AUJZ-REEL7 | IC OPAMP GP R-R CMOS TSOT23-5 |
| U4 | MAX5033 | Maxim IC | MAX5033AASA+ | IC BUCK 3.3V 0.5A 8SOIC |
| U5 | QS6K21TR | Rohm | QS6K21TR | MOSFET N-CH 45V 1A TSMT6 |
| U6 | LT1787 | Linear Technology | LT1787HVIS8#PBF | IC AMP PREC CURR SENSE HV 8-SOIC |
| U7 | AD8603 | Analog Devices | AD8603AUJZ-REEL7 | IC OPAMP GP R-R CMOS TSOT23-5 |
| U8 | QS6K21TR | Rohm | QS6K21TR | MOSFET N-CH 45V 1A TSMT6 |
| U9 | LT1787 | Linear Technology | LT1787HVIS8#PBF | IC AMP PREC CURR SENSE HV 8-SOIC |
| U10 | AD8603 | Analog Devices | AD8603AUJZ-REEL7 | IC OPAMP GP R-R CMOS TSOT23-5 |
| U11 | QS6K21TR | Rohm | QS6K21TR | MOSFET N-CH 45V 1A TSMT6 |
| U12 | LT1787 | Linear Technology | LT1787HVIS8#PBF | IC AMP PREC CURR SENSE HV 8-SOIC |
| U13 | AD8603 | Analog Devices | AD8603AUJZ-REEL7 | IC OPAMP GP R-R CMOS TSOT23-5 |
| U14 | STS4DPF30L | STMicroelectronics | STS4DPF30L | MOSFET 2P-CH 30V 4A 8-SOIC |
| U15 | LT1787 | Linear Technology | LT1787HVIS8#PBF | IC AMP PREC CURR SENSE HV 8-SOIC |
| U16 | AD8603 | Analog Devices | AD8603AUJZ-REEL7 | IC OPAMP GP R-R CMOS TSOT23-5 |
| U17 | LDA100STR | IXYS | LDA100STR | OPTOCOUPLER SGL TRANS-OUT 6-SMD |
| U18 | PIC24FJ256GA106 | Microchip | PIC24FJ256GA106-I/PT | IC PIC MCU FLASH 256K 64TQFP |
| U19 | MAX3226E | Maxim IC | MAX3226ECUE+ | IC TXRX RS232 250KBPS SD 16TSSOP |
| U20 | ADS8344 | Analog Devices | ADS8344NB | IC 16-BIT 8-CH A/D CONV 20-SSOP |
| U21 | MAX3226E | Maxim IC | MAX3226ECUE+ | IC TXRX RS232 250KBPS SD 16TSSOP |
| U22 | AD1583 | Analog Devices | AD1583ARTZ-REEL7 | IC VREF SERIES PREC 3V SOT-23-3 |
| U23 | MAX3226E | Maxim IC | MAX3226ECUE+ | IC TXRX RS232 250KBPS SD 16TSSOP |
| U24 | AD8603 | Analog Devices | AD8603AUJZ-REEL7 | IC OPAMP GP R-R CMOS TSOT23-5 |
| U25 | LT1790-1.25 | Linear Technology | LT1790BCS6-1.25#TRMPBF | IC VREF SERIES 1.25V SOT-23-6 |
| U26 | MAX3226E | Maxim IC | MAX3226ECUE+ | IC TXRX RS232 250KBPS SD 16TSSOP |
| Y1 | 3.6864MHz | ECS Inc | ECS-36-20-5PXDU-TR | CRYSTAL 3.6864 MHZ 20PF SMD |