# MBARI FOCE 
Ocean Acidification Research Tool

The MBARI FOCE (Free Ocean Carbon Experiment) is technology for studying the effects of ocean acidification. FOCE takes the experimental concept of the terrestrial Free Air Carbon Experiments into the ocean, making it possible to perform in situ experiments in a pH-controlled environment.

FOCE may be implemented for shallow water or deep, and the flexible design provides many options for optimizing cost and performance.

## Features

- Maintain pH offset relative to reference sensor

- Maintain specified constant pH

- Manual or small-boat deployable

- Diver serviceable

- Real-time telemetry and system monitoring

- Automatic data logging

- Wide range of operating depths

- Scalable, customizable architecture

## Specifications

|  | Min | Max | Typical |
| --- | --- | --- | --- |
| pH operating range | 7.0 | 8.0 |  |
| pH tracking |  |  | ±0.01 |
| pH resolution |  |  | ±0.001 |
| Instrument suite |  |  | CTD (2) pH (2) DO (2) Water flow (2) Video Camera (1) |
| Data storage |  |  | 128 GB |
| Sample rates |  | 2 Hz |  |
| Device inputs |  |  | 16x RS-232 10/100Base-T Ethernet |
| Supply Voltage |  |  | 12, 24, 48, 400 VDC |

|  | basic | Better | Best |
| --- | --- | --- | --- |
| Self-contained |  |  |  |
| CTD | 2 | 2 | 2 |
| DO | 2 | 2 | 2 |
| pH | 2 | 4 | 4 |
| Velocity/Flow | 2 | 3 | 3 |
| ESW Pump | 1 | 1 | 1 |
| ESW Diverter | 1 | 1 | 2 |
| Thrusters | 1 | 1 | 2 |
| Light Panel | 1 | 2 | 3 |
| Timelapse Cam | ✔ |  |  |
| Video Cam | ✗ | 2 | 3 |
| Bidirectional Flow | ✗ | ✔ | ✔ |

| ✔ | Included |
| --- | --- |
| ✗ | Not Included |
|  | Optional |

| Options |  |
| --- | --- |
| Energy Source | Diesel/Gas/etc. Wind/Solar Shore Utility Batteries |
| Power Generation | Wind Turbine PV Panels Diesel Generator Shore Utility Air Compressor HPU |
| Power Distribution | Cabled Self-contained |
| Power Storage | Batteries |
| CO2 Source | Bottled Gas Diesel Gen |
| ESW Storage | Shore ESW Reservoir In-situ ESW Reservoir In-situ liquid CO2 Reservoir |
| ESW Distribution | Electric Pump Hydraulic Pump Pneumatic Pump |
| Telemetry PHY | RF Cell Satellite Cable Inductive |
| Telemetry Protocols | TCP/IP Serial 802.11 SBD |
| Com PHY | Inductive Twisted pair Coax Fiber-optic Acoustic |
| Com Protocols | TCP/IP RS422 RS232 CAN RS485 |