Contents

Background Assumptions

Design Priorities

Background Assumptions

Goal is to help people get started using FOCE technology, not to solve all science applications once and for all.

Minimum Implementing Group:

1 PI

1-3 research assistants (grad/post-doc)

1-2 technical assistants

Large Implementing Group

5 PI

10 students

Tech support staff

Budget $200-250k USD (materials)

Deployment Scenarios:

- Multiple deployments over ~2 years

- initial deployment shallow, near shore (diver serviceable)

- further deep (800 m) deployments

- relatively short duration experiments rather than long time series (1-6 mo)

Facilities:

- bottled CO2 or shore supply (e.g. diesel generator)

- diesel generator likely source for CO2 and power

- probably need solar/wind or other alternative for remote systems due to permitting constraints

- small boat/diver deployment and maintenance

Science:

- greater pH offsets easier to maintain in shallow water than deep (amt of CO2)

- greater pH offsets easier to maintain at higher environment pH

- 0.3 offset suitable for many science experiments

- Precise matching of current not required (e.g. 0 environment current)

- 5 cm/sec max velocity will cover most (90%) of science; 10 cm/sec covers 95%

- coastal systems may be experiencing more rapid pH changes than open ocean

- Bio experiments will make observations by comparing pre- and post-experiments samples, in additions to time lapse video

- video support need, but mostly as time lapse frames rather than full video frame rates. Estimate 3-5 images/minute is OK. Resolution TBD.

- Also want to be able to close the chamber for respirometry experiments

- Core instruments: pH, CTD, O2

- Would maintain at least one instrumented control unit, that is run continuously (not turned on and off for reference measurements)

Antarctic deployments

- would use sled mounted experiment

- favor diesel gen for power (how would antifreeze additives affect gases produced)

- lower experiment through hole in ice

Design Priorities

Ease of acquisition (build/buy)

Component availability

Licensing

Skills required to specify and/or build

Ease of use

Ease of modification

Low power

Low cost