Exportable FOCE Software Requirements

Design Priorities

Ease of acquisition (build/buy)

Component availability

Licensing

Skills required to specify and/or build

Ease of use

Ease of modification

Low power

Low cost

User Assumptions

1 PI

2-3 grad student/post doc

1 research technician w/ some mix of electrical, mechanical and software skills

|  | Must |  | Should | May |  | Other | Notes |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Functional Requirements | Things the system does from a user perspective |  |  |  |  |  |  |
| Maintain a pH offset from environment up to from 0 to -0.65+/-0.01 | Y |  |  |  |  |  | Limits? |
| Maintain a specified pH between 7.00 and 7.65 | Y |  |  |  |  |  | Limits? |
| Control and collect data from core instruments and actuators | Y |  |  |  |  |  | pH, CTD, H2O velocity thrusters, pumps, valves (or equivalent) |
| Provide a mechanism for near-real-time data reporting |  |  |  |  |  |  | system health and status science data near real-time (10 min update rate) Not clear (must-may) |
| Provide a mechanism for system configuration | Y |  |  |  |  |  |  |
| Provide a mechanism for instrument configuration |  | Y |  |  |  |  |  |
| Provide mechanism for science data visualization |  | Y |  |  |  |  |  |
| Log instrument data for deployment duration | Y |  |  |  |  |  | Local or shore |
| Timestamp instrument data with TBD precision/accuracy | Y |  |  |  |  |  |  |
| Require less than 1-week software setup time by user |  |  |  |  |  | X | This is a constraint, TBD about what’s reasonable |
| Provide printable electronic documentation |  | Y |  |  |  |  | Engineering requirements Not a FOCE functional requirement Usability requirement |
| Enable modification of control algorithms and strategies |  |  |  |  |  |  | Needs clarification Must-should Can this be known – one size fits all? |
| Allow integration of additional sensors (stand-alone or for control) | Y |  |  |  |  |  | What are limits/specs of expandability? To what software level (e.g. GUI) Must? Should? |
| Support data telemetry | ? | ? |  |  |  |  | Must w/ cable? If not must, how often need to access data? From where? …one month set and forget – reasonable? |
| Provide mechanism for engineering data visualization | Y | Y |  |  |  |  | Less so if stand-alone should-must show benefits/cost to outside users |
| Support in-lab system test/verification | Y |  |  |  |  |  |  |
| Provide support for http and/or digital still camera |  | ? |  |  | Y |  | Application specific Not FOCE requirement? Not required to control pH May-Should Need to better define applications |
| Provide pH sensor calibration tools | Y |  |  |  |  |  | System requirement written documentation more important not necessarily software |
|  |  |  |  |  |  |  | FOCE system includes the shore side - visual |
|  |  |  |  |  |  |  | Can FOCE be stand-alone/fire and forget Dependency on accessibility |
| Support in-situ pH calibration |  |  |  |  |  |  | System requirement? What does science say? Need more definition of application |
|  |  |  |  |  |  |  | Define core instrument suite pCO2? Camera? |

|  | Must | Should | May | Other | Notes |
| --- | --- | --- | --- | --- | --- |
| Design Requirements | Things the system does from an engineering perspective |  |  |  |  |
| Support sample and update rates up to 2 Hz |  |  |  |  |  |
| Operate in the presence of currents and surge (specs TBD) |  |  |  |  |  |
| Operate in temperature range of 0-25 degrees C |  |  |  |  |  |
| Operate in depth range 0-1000 m |  |  |  |  |  |
| Operate continuously and autonomously for 30d |  |  |  |  |  |
| Support RS232 instruments up to 115200 bps |  |  |  |  |  |
| Enable power switching of individual instruments |  |  |  |  |  |
| Provide a configurable or modular control system |  |  |  |  |  |
| Support dynamically enabled/disabled instrument drivers |  |  |  |  |  |
| Support hardware power management (e.g. sleep modes) |  |  |  |  |  |
| Support ground fault detection and isolation (per load) |  |  |  |  |  |
| Support some form of IPC (inter-process communication) |  |  |  |  |  |
| Support 10 Mbps Ethernet connectivity using 10-base t |  |  |  |  |  |
| Support TCP/IP protocols |  |  |  |  |  |
| Work with COTS processors |  |  |  |  |  |
| Be open source/low cost |  |  |  |  |  |
| Use C, Java, BASIC or visual programming language |  |  |  |  |  |
| Support/use scripting languages |  |  |  |  |  |

|  | Type | Must | Should | May | Other | Notes |
| --- | --- | --- | --- | --- | --- | --- |
| Other Requirements |  |  |  |  |  |  |
| Core instrument suite consists of pH (1 int 1ext) CTD (1 int 1 ext) |  |  |  |  |  | Number? Redundancy/Reliability? Science requirements |
| Supplemental instrument suite Velocimeter/ADCP O2 Camera Lights CO2 delivery pH |  |  |  |  |  |  |
| Core actuator set CO2 delivery system (e.g. pumps, valves) Thrusters |  |  |  |  |  | Different way of implementing e.g. w/o valves Is there a possible to have a thrusterless FOCE? |
| Supplemental actuators |  |  |  |  |  |  |
|  |  |  |  |  |  | Is FOCE set/forget autonomous |