Contents

System Functional Requirements

System Engineering Requirements

System Functional Requirements

|  | Must | Should | May | Other | Specification |
| --- | --- | --- | --- | --- | --- |
| System Functional Requirements |  |  |  |  |  |
| PERFORMANCE |  |  |  |  |  |
| Maintain -0.5 pH offset | Y |  |  |  | relative to environment accuracy/precision? @ 5 cm/sec |
| Maintain specified pH value | Y |  |  |  | 7.1 < pH < 7.6 independent of ambient pH internal <= 5 cm/sec |
| Maintain specified VH20 | Y |  |  |  | 2-5 cm/sec Accuracy/precision? |
| ENVIRONMENTAL |  |  |  |  |  |
| Max depth | Y |  |  |  | 100m Note diver-servicable requirement Not suitable for active surf |
| Water temperature range | Y |  |  |  | -2 to 30 degrees C |
| Bio-fouling protection/prevention | Y |  |  |  | Specification? Environmental requirements trades? Ties to reliability and maintenance e.g. passive, Cu… |
| Min external VH20 | Y |  |  |  | 0 cm/sec |
| Max external VH20 | Y |  |  |  | ? cm/sec? |
| Min internal VH20 | Y |  |  |  | 2 cm/sec |
| Max internal VH20 | Y |  |  |  | 5 cm/sec (90% sci goals met, decreases CO2 consumption) |
| Min/max surge | Y |  |  |  | Specification ? |

|  | Must | Should | May | Other | Specification |
| --- | --- | --- | --- | --- | --- |
| System Functional Requirements |  |  |  |  |  |
| CORE INSTRUMENT SUITE |  |  |  |  |  |
| Experiment pH | Y |  |  |  | 1 per experiment, 1 per control Electrical and communications interfaces? Is reference measured in ambient? Control? |
| Experiment CTD 1:2 | Y |  |  |  |  |
| Experiment DO2 1:2 | Y |  |  |  |  |
| Experiment H2O flow (velocity) | Y |  |  |  | 2D? 3D? Either volume or velocity Not both Range, precision, rate |
| Experiment H2O flow (volume) | Y |  |  |  | Either volume or velocity Not both |
| Control pH 1:TBD | Y |  |  |  | Control vs external |
| Control CTD 1:TBD | Y |  |  |  |  |
| Control DO2 1:TBD |  | Y |  |  |  |
| Control H2O flow (velocity) |  | Y |  |  |  |
| Camera (time lapse video) | Y |  |  |  | Must support, but not required for all applications How will video be used? Frame rate (3-5 frames/min?) Resolution: TBD? Interface (serial, USB, HTTP, Ethernet)? |
| Light panel | Y |  |  |  | Required if camera Output, angle, power… |
| Thruster(s) | Y |  |  |  | Quantity: 1 min, app-specific Spec Depends: VH20 range pH offset range |
| ESW injector(s) | Y |  |  |  | Quantity depends number of thrusters, use of valves Spec Depends: pH offset range deployment duration |
| ESW valve(s) | Y |  |  |  | Depends on injection design |
| ESW pH |  | Y |  |  | Useful when at end of ESW supply to modify feed algorithm |
| ESW CTD |  |  |  | X | Not required |
| Instrument expansion capability |  | Y |  |  | Should, but not core Number, type? |

|  | Must | Should | May | Other | Specification |
| --- | --- | --- | --- | --- | --- |
| System Functional Requirements |  |  |  |  |  |
| OPERATION |  |  |  |  |  |
| Small-boat deployable | Y |  |  |  | Zodiac/whaler |
| Diver serviceable | Y |  |  |  | Installation, cleaning, instrument replacement, recovery |
| In-situ replaceable instruments |  | Y |  |  | Must for pH (cal) Others should? Maintenance schedule |
| sealable chamber | Y |  |  |  | Remotely operated is requirement? |
| In-situ pH calibration |  |  | Y |  | Depends on cost trade Nice to have |
| Deployment/experiment duration |  |  |  |  | Duration of experiment and deployment |
| Operate continuously | Y |  |  |  | Runs overnight w/o monitoring min |
| Operate autonomously |  |  | Y |  | Set it up and run it, but person in loop to correct anomalies …requirements for event detection and/or data visualization |
| Maintenance schedule |  |  |  |  | For instruments, For whole system |

|  | Must | Should | May | Other | Specification |
| --- | --- | --- | --- | --- | --- |
| System Functional Requirements |  |  |  |  |  |
| DATA MANAGEMENT |  |  |  |  |  |
| Data logging (remote side) |  |  |  |  | Full-resolution science data Appropriate system data Independent of telemetry |
| Data logging rates |  |  |  |  | Rates up to 2 Hz (10 Hz?) Must support system and science data stream rates |
| Data archive (shore side) |  |  |  |  |  |
| Data log format |  |  |  |  |  |
| Data log capacity |  |  |  |  | 128 GB |
| Near real-time telemetry |  |  |  |  | Full data resolution? Rate? |
| Telemetry data summarization |  |  |  |  |  |
| Near real-time data visualization |  |  |  |  |  |
| Event detection and notification |  |  |  |  |  |
| Timestamp data/ time distribution |  |  |  |  | Accuracy? Precision? |
| USER INTERFACE |  |  |  |  |  |
| System configuration and control | Y |  |  |  | Supports: Power management Data stream read (instantaneous) Instrument config?/mgmt Control tuning Experiment control Retrieve data logs pH cal? |
| Data visualization | Y |  |  |  | Supports: Power management Data charting (time series) Experiment control Control tuning pH cal? |
| POWER MANAGEMENT |  |  |  |  |  |
| Total system power |  |  |  |  | TBD Watts |
| Monitor individual load voltages and currents |  |  |  |  | Range? Rate/resolution/accuracy? |
| Monitor bus voltages and currents |  |  |  |  | Range? Rate/resolution/accuracy? |
| Switch individual load power |  |  |  |  |  |
| Switch bus power |  |  |  |  |  |
| RELIABILITY |  |  |  |  |  |

System Design Requirements

|  | Must | Should | May | Other | Specification |
| --- | --- | --- | --- | --- | --- |
| System Design Requirements |  |  |  |  |  |
| TELEMETRY |  |  |  |  |  |
| Support wired communication to shore |  |  |  |  |  |
| Support wireless communication to shore |  |  |  |  |  |
| Support TCP/IP protocols |  |  |  |  |  |
| Support 10BaseT Ethernet or better |  |  |  |  | Should be single requirement |
| Support remote console |  |  |  |  | e.g., serial, ssh, telnet |
| DEVICE I/O SUPPORT |  |  |  |  |  |
| RS-232 device I/O |  |  |  |  | up to 115200 bps Number of channels? Expandable? |
| Analog device I/O |  |  |  |  | Number of channels? Input range? Expandable? |
| Ethernet device I/O |  |  |  |  |  |
| HTTP device I/O |  |  |  |  | e.g. cameras |
| Memory mapped device I/O |  |  |  |  | Number of channels? Expandable? e.g. enclosure sensors |
| USB device I/O |  |  | Y |  |  |

|  | Must | Should | May | Other | Specification |
| --- | --- | --- | --- | --- | --- |
| System Design Requirements |  |  |  |  |  |
| CORE SOFTWARE |  |  |  |  |  |
| Use open source/free software |  |  |  |  |  |
| Support IPC (inter-process communication) |  |  |  |  |  |
| Thread support |  |  |  |  |  |
| Support for chosen I/O methods |  |  |  |  | RS232, RS422, RS485 (or other bus), Ethernet, Analog… |
| Support in-situ software updates |  |  |  |  |  |
| Support in-situ firmware updates |  |  |  |  |  |
| Support dynamic device driver loading |  |  |  |  | Start, stop, replace individual instrument drivers |
| Support integration of new sensors |  |  |  |  | e.g. instrument service framework Support for serial, analog, Ethernet, multidrop bus Support for polled, streaming |
| Support data pub/sub |  |  |  |  | streaming control: Per channel/per device? |
| Support event detection and notification |  |  |  |  | What events? System data only? Science data? |
| Support off-line system verification test |  |  |  |  |  |
| Enable modification of control algorithms/strategies |  |  |  |  |  |
| System data monitoring |  |  |  |  | Update rates? Logging? Telemetry? Bus voltage(s)/current(s) Device voltage/current Over/under voltage conditions Ground faults Enclosure Temp Enclosure Humidity Enclosure Pressure Enclosure Water intrusion Heat sink Temp(s) Disk, CPU, memory usage |
| Support hardware power management |  |  |  |  | sleep modes, clock throttling, etc. |
| SOFTWARE LANGUAGE SUPPORT |  |  |  |  | For user-modifiable modules e.g. instrument drivers, control algorithms More implementation choice than requirement |
| Support C language |  |  |  |  |  |
| Support Java language |  |  |  |  |  |
| Support BASIC language |  |  |  |  |  |
| Support LabView language |  |  |  |  |  |
| Support a visual prog language |  |  |  |  |  |
| Support MatLab language |  |  |  |  | For what? |
| Support scripting languages |  |  |  |  | For what? |

|  | Must | Should | May | Other | Specification |
| --- | --- | --- | --- | --- | --- |
| System Design Requirements |  |  |  |  |  |
| INPUT VOLTAGE SECTION |  |  |  |  |  |
| Accept input DC volt up to 400Vdc |  |  |  |  |  |
| Accept input DC volt up to 50Vdc |  |  |  |  |  |
| Accept input DC current to x amps |  |  |  |  |  |
| Provide galvanic isolation to source |  |  |  |  |  |
| Provide inrush limiting |  |  |  |  |  |
| Be fused / cicuit breaker |  |  |  |  |  |
| LV DC CONVERSION |  |  |  |  |  |
| Provide multiple low voltages |  |  |  |  | 5, 12, 24, 48 |
| Provide voltage monitoring |  |  |  |  |  |
| Provide current monitoring |  |  |  |  |  |
| Monitor for over voltage to instr |  |  |  |  |  |
| Monitor for under voltage to instr |  |  |  |  |  |
| Monitor for over current to instr |  |  |  |  |  |
| Provide a resettable circuit breaker |  |  |  |  |  |
| Monitor for ground faults |  |  |  |  |  |
| Provide analog reference voltage |  |  |  |  | +/- 5, 12 V |
| Automatic over-voltage protection |  |  |  |  |  |
| Automatic over-current protection | Y |  |  |  |  |
| Ground fault detection |  |  |  |  |  |
| Galvanic isolation (per load) |  |  |  |  |  |

|  | Must | Should | May | Other | Specification |
| --- | --- | --- | --- | --- | --- |
| System Design Requirements |  |  |  |  |  |
| ENVIRONMENTAL SENSORS |  |  |  |  |  |
| Provide ambient temp monitoring |  |  |  |  | Update rate? |
| Provide ambient humidity monitoring |  |  |  |  |  |
| Provide ambient pressure monitoring |  |  |  |  |  |
| Provide water presence detection |  |  |  |  |  |
| Provide temp monitoring of various subsystems |  |  |  |  |  |
| ENVIRONMENTAL |  |  |  |  |  |
| Non-toxic |  |  |  |  |  |
| Non-hazardous to animals |  |  |  |  |  |
| CONTROL SYSTEM |  |  |  |  |  |
| pH control |  |  |  |  | Update rate, precision |
| VH20 control |  |  |  |  | Update rate, precision |