# Contents

# System Functional Requirements

System Engineering Requirements

# System Functional Requirements

|  | Must | Should | May | Other | Specification |
| --- | --- | --- | --- | --- | --- |
| System Functional Requirements |  |  |  |  |  |
| PERFORMANCE |  |  |  |  |  |
| Maintain -0.5 pH offset | Y |  |  |  | relative to environment accuracy/precision? @ VH20 internal= 5 cm/sec |
| Maintain specified pH value | Y |  |  |  | 7.0 < pH < 8.2 independent of ambient pH internal <= 5 cm/sec |
| Maintain specified VH20 | Y |  |  |  | 2-5 cm/sec Accuracy/precision? |
| ENVIRONMENTAL |  |  |  |  |  |
| Max depth | Y |  |  |  | 40m (130 ft) Note diver-servicable requirement Not suitable for active surf, river mouth |
| Water temperature range | Y |  |  |  | -2 to 30 degrees C |
| Bio-fouling protection/prevention | Y |  |  |  | Hard to spec, Can document recommendations Environmental requirements trades (e.g. toxicity) Ties to reliability and maintenance (is a maintenance schedule issue) e.g. passive, Cu… |
| Min external VH20 | Y |  |  |  | 0 cm/sec |
| Max external VH20 | Y |  |  |  | 50 cm/sec |
| Min internal VH20 | Y |  |  |  | 2 cm/sec |
| Max internal VH20 | Y |  |  |  | 5 cm/sec (90% sci goals met, decreases CO2 consumption) |
| Min/max surge | Y |  |  |  | Must work in some surge conditions. Surge is mostly moving water back and forth in chamber – should not affect control (?) |

|  | Must | Should | May | Other | Specification |
| --- | --- | --- | --- | --- | --- |
| System Functional Requirements |  |  |  |  |  |
| CORE INSTRUMENT SUITE |  |  |  |  |  |
| Experiment pH | Y |  |  |  | 1 per experiment, 1 per control Electrical and communications interfaces? Is reference measured in ambient? Control? |
| Experiment CTD 1:2 | Y |  |  |  |  |
| Experiment DO2 1:2 | Y |  |  |  |  |
| Experiment H2O flow (velocity) | Y |  |  |  | Sci wants to know how much (low pH) water has flowed through over time period, and in which direction. Could use inexpensive flow meters instead of velocimeter. 2D OK Either volume or velocity Not both Range, precision, rate |
| Experiment H2O flow (volume) | Y |  |  |  | Either volume or velocity Not both |
| Control pH 1:TBD | Y |  |  |  | Control vs external |
| Control CTD 1:TBD | Y |  |  |  |  |
| Control DO2 1:TBD |  | Y |  |  |  |
| Control H2O flow (velocity) |  | Y |  |  |  |
| Camera (time lapse video) | Y |  |  |  | Must support, but not required for all applications How will video be used? Frame rate 10 frames/min Resolution: publication quality (e.g. HD, Coolpix…5-12 MP) Supplement to measurement data |
| Light panel | Y |  |  |  | Required for camera Output, angle, power… |
| Thruster(s) | Y |  |  |  | Quantity: 1 min, app-specific, number and type may change Spec Depends: VH20 range pH offset range |
| ESW injector(s) | Y |  |  |  | Quantity 1 min; depends number of thrusters, use of valves Spec Depends: pH offset range deployment duration |
| ESW valve(s) | Y |  |  |  | Depends on injection design |
| ESW pH |  | Y |  |  | Useful when at end of ESW supply to modify feed algorithm |
| ESW CTD |  |  |  | X | Not required |
| Instrument expansion capability |  | Y |  |  | Should, but not core Number, type? |

|  | Must | Should | May | Other | Specification |
| --- | --- | --- | --- | --- | --- |
| System Functional Requirements |  |  |  |  |  |
| OPERATION |  |  |  |  |  |
| Small-boat deployable | Y |  |  |  | Zodiac/whaler |
| Diver serviceable | Y |  |  |  | Installation, cleaning, instrument replacement, recovery |
| In-situ replaceable instruments |  | Y |  |  | Must for pH (cal) Others should? Maintenance schedule |
| sealable chamber | Y |  |  |  | Remotely operated is requirement? |
| In-situ pH calibration |  |  | Y |  | Depends on cost trade Nice to have |
| Deployment/experiment duration |  |  |  |  | Duration of experiment and deployment |
| Operate continuously | Y |  |  |  | Runs overnight w/o monitoring min |
| Operate autonomously |  |  | Y |  | Set it up and run it, but person in loop to correct anomalies …requirements for event detection and/or data visualization |
| Maintenance schedule |  |  |  |  | Will depend on application, ranging from every day or two (e.g. at start of shallow experiment) to every few days, with periodic system checks. |

|  | Must | Should | May | Other | Specification |
| --- | --- | --- | --- | --- | --- |
| System Functional Requirements |  |  |  |  |  |
| DATA MANAGEMENT |  |  |  |  |  |
| Data logging (remote side) | Y |  |  |  | Requirement is that all data must be recorded (independent of telemetry function) and available at end of experiment. Full-resolution science and system data (for forensic reconstruction of events/anomalies) Must be independent of telemetry |
| Adjustable device sample rates |  | Y |  |  | Must be able to set sample rates to provide best resolution for diagnostics, and lower sample rates for logging and performance. See data logging rates below |
| Data logging rates (core instruments) | Y |  |  |  | TBD… CTD, pH, velocity/flow Rates up to 2 Hz (10 Hz?) Must support system and science data stream rates Depends on environmental conditions, velocity sampling strategies |
| Data archive (shore side) | ? | ? |  |  | Split opinions – must vs should Single point of failure/enabler Vs Not strictly required to do FOCE Part of system to get data to shore Maybe not automatic Maybe very low tech (scp) Cost vs value |
| Data log format |  |  |  | X | Timestamped data Any format OK as long as complete, robust. If not human readable, need tools to access records. Segmented logs? Must be able to request specific records using specified time range(s) or number of records |
| Data log capacity | X |  |  |  | Must record all science and system data for 6 month deployment. Depends: data rates, number of streams, sample rates, data log format |
| Near real-time telemetry | Y |  |  |  | Full data resolution if possible, but all data must be recorded even w/o telemetry link. Must provide data required to assess health and status of experiment. Rate: best effort to reflect full-resolution logged data. |
| Telemetry data summarization | ? | Y |  |  | Data telemetry must support system health and status reporting. Should make best effort to reflect full-resolution logged telemetry, but this is not required. Alarm is summary Some form probably needed |
| Near real-time data visualization | ? | Y |  |  | Must be able to see from shore whether the system is working. What does it mean to be working? How do we tell? What streams? How often updated? Should, but not must? What is cost? |

|  | Must | Should | May | Other | Specification |
| --- | --- | --- | --- | --- | --- |
| System Functional Requirements |  |  |  |  |  |
| DATA MANAGEMENT |  |  |  |  |  |
| Event detection and notification |  | Y |  |  | Should do if telemetry Data telemetry must support system health and status reporting. Other event detection and reporting would be great, but not required. What events? Sci or Sys-only? |
| Timestamp data | Y |  |  |  | Need to correlate data streams in memory Accuracy? Precision? |
| Time distribution |  |  |  |  | How often to synchronize clocks How to distribute time signal? Do we need to distribute time signal? Is there more than one clock in the system? |
| Timing determinism | Y |  |  |  | ~0.5 max sample rate – Nyquist freq. |

| USER INTERFACE |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| System configuration and control | Y |  |  |  | Supports: Power management Data stream read (instantaneous) Instrument config?/mgmt Control tuning Experiment control Retrieve data logs pH cal? |
| Data visualization | Y |  |  |  | Supports: Power management Data charting (time series) Experiment control Control tuning pH cal? |
| POWER MANAGEMENT |  |  |  |  |  |
| Total system power |  |  |  |  | Minimize… Depends: power system implementation choice Amount of water moved What does current FOCE actually use? TBD Watts |
| Max inrush current |  |  |  |  | Need spec (like sys power) |
| Input voltage range |  |  |  |  | “ |
| Input current range |  |  |  |  | “ |
| Monitor individual load voltages and currents |  |  |  |  | Range? Rate/resolution/accuracy? In hardware must Operator monitoring should |
| Monitor bus voltages and currents |  |  |  |  | Range? Rate/resolution/accuracy? In hardware must Operator monitoring should |
| Switch individual load power | Y |  |  |  | Must be maintainable |
| Switch bus power |  |  |  |  |  |
| RELIABILITY |  |  |  |  |  |

# System Design Requirements

|  | Must | Should | May | Other | Specification |
| --- | --- | --- | --- | --- | --- |
| System Design Requirements |  |  |  |  |  |
| TELEMETRY |  |  |  |  |  |
| Support wired communication to shore |  |  |  | X | Requirement is for some type of communication to shore; this is an implementation option |
| Support wireless communication to shore |  |  |  | X | Requirement is for some type of communication to shore; this is an implementation option |
| Support TCP/IP protocols |  |  |  | X | Requirement is for some type of communication to shore; this is an implementation option |
| Support 10BaseT Ethernet or better |  |  |  | X | Requirement is for some type of communication to shore; this is an implementation option |
| Support remote console | Y |  |  |  | e.g., serial, ssh, telnet |
| DEVICE I/O SUPPORT |  |  |  |  |  |
| RS-232 device I/O |  |  |  | X | Requirement is to support the core set of instruments. This is an implementation option. up to 115200 bps Number of channels? Expandable? |
| Analog device I/O |  |  |  | X | Requirement is to support the core set of instruments. This is an implementation option. Number of channels? Input range? Expandable? |
| Ethernet device I/O |  |  |  | X | Requirement is to support the core set of instruments. This is an implementation option. |
| HTTP device I/O |  |  |  | X | Requirement is to support the core set of instruments. This is an implementation option.e.g. cameras |
| Memory mapped device I/O |  |  |  | X | Requirement is to support the core set of instruments. This is an implementation option.Number of channels? Expandable? e.g. enclosure sensors |
| USB device I/O |  |  |  | X | Requirement is to support the core set of instruments. This is an implementation option. |

|  | Must | Should | May | Other | Specification |
| --- | --- | --- | --- | --- | --- |
| System Design Requirements |  |  |  |  |  |
| CORE SOFTWARE |  |  |  |  |  |
| Use open source/free software |  |  |  | X |  |
| Support IPC (inter-process communication) |  |  |  | X |  |
| Thread support |  |  |  | X |  |
| Support for chosen I/O methods | Y |  |  |  | RS232, RS422, RS485 (or other bus), Ethernet, Analog… |
| Support in-situ software updates |  |  | Y |  | Nice to have |
| Support in-situ firmware updates |  |  | Y |  | Nice to have |
| Support dynamic device driver loading |  |  |  |  | Start, stop, replace individual instrument drivers |
| Support integration of new sensors | Y |  |  |  | Requirement is to lower barriers of initial adoption/participation by new users. Make it easy to try FOCE technology, w/ less emphasis on extending or customizing. Not strictly required to provide generic mechanism here, but do want to show others an expansion path. e.g. instrument service framework Support for serial, analog, Ethernet, multidrop bus Support for polled, streaming |
| Support data pub/sub |  |  |  | X | Implementation option streaming control: Per channel/per device? |
| Support system verification and test Pre- and post-deployment | Y |  |  |  | Must be able to verify that the system is working before (and after) it is deployed |
| Enable modification of control algorithms/strategies | Y |  |  |  | Must make it easy to tune and adjust control strategy and/or algorithms for various applications Configurable at minimum |
| System data monitoring | Y |  |  |  | Update rates? Logging? Telemetry? Bus voltage(s)/current(s) Device voltage/current Over/under voltage conditions Ground faults Enclosure Temp Enclosure Humidity Enclosure Pressure Enclosure Water intrusion Heat sink Temp(s) Disk, CPU, memory usage Must be recorded locally Actual list may depend… |
| Support hardware power management | Y |  |  |  | Requirement is to minimize power consumption where possible. Support whatever modes are available and chosen for selected hardware sleep modes, clock throttling, etc. |
| SOFTWARE LANGUAGE SUPPORT |  |  |  |  |  |
| Support C language |  |  |  |  | Implementation option |
| Support Java language |  |  |  |  | Implementation option |
| Support BASIC language |  |  |  |  | Implementation option |
| Support LabView language |  |  |  |  | Implementation option |
| Support a visual prog language |  |  |  |  | Implementation option |
| Support MatLab language |  |  |  |  | For what? |
| Support scripting languages |  |  |  |  | For what? |

|  | Must | Should | May | Other | Specification |
| --- | --- | --- | --- | --- | --- |
| System Design Requirements |  |  |  |  |  |
| INPUT VOLTAGE SECTION |  |  |  |  |  |
| Accept input DC volt up to 400Vdc | Y |  |  |  | What drives 400V requirement? Most choose a (common) input voltage … somewhat changeable at low cost. Requirement: Accept inputs that may be able to down-convert to 48V |
| Accept input DC volt up to 50Vdc |  |  |  |  | `` see above 400V reqmt |
| Accept input DC current to x amps |  |  |  |  |  |
| Provide galvanic isolation to source |  |  |  |  |  |
| Provide inrush limiting |  |  |  |  |  |
| Be fused / cicuit breaker |  |  |  |  |  |
| LV DC CONVERSION |  |  |  |  |  |
| Provide multiple low voltages |  |  |  |  | 5, 12, 24, 48 |
| Provide voltage monitoring |  |  |  |  | Hardware level |
| Provide current monitoring |  |  |  |  |  |
| Monitor for over voltage to instr |  |  |  |  |  |
| Monitor for under voltage to instr |  |  |  |  |  |
| Monitor for over current to instr |  |  |  |  |  |
| Provide a resettable circuit breaker |  |  |  |  |  |
| Monitor for ground faults |  |  |  |  |  |
| Provide analog reference voltage |  |  |  |  | +/- 5, 12 V |
| Automatic over-voltage protection |  |  |  |  |  |
| Automatic over-current protection | Y |  |  |  |  |
| Ground fault detection |  |  |  |  |  |
| Galvanic isolation (per load) |  |  |  |  |  |

|  | Must | Should | May | Other | Specification |
| --- | --- | --- | --- | --- | --- |
| System Design Requirements |  |  |  |  |  |
| ENVIRONMENTAL SENSORS |  |  |  |  | Goal is graceful degradation And recording context for forensics Hardware and software |
| Provide ambient temp monitoring | Y |  |  |  | 0.016 Hz (1 min) nominal |
| Provide ambient humidity monitoring | Y |  |  |  | 0.016 Hz (1 min) nominal |
| Provide ambient pressure monitoring | Y |  |  |  | 0.016 Hz (1 min) nominal |
| Provide water presence detection | Y |  |  |  | 0.016 Hz (1 min) nominal |
| Provide temp monitoring of various subsystems | ? |  |  |  | More specific |
| ENVIRONMENTAL |  |  |  |  |  |
| Non-toxic |  |  |  |  | May be required in some applications; no general requirement, but less hazardous is better :o) |
| Non-hazardous to animals |  |  |  |  | May be required in some applications; no general requirement, but less hazardous is better :o) |