Pending Issues/Questions

| Issue/Question | Resolution |
| --- | --- |
| Design for single or double ESW inputs? |  |
| Should we assume stand-alone, set/forget operation? | No |
| Autonomous operation spec | Quasi autonomous, |
| Continuous operation spec | 3-4 month operations momentary checks on regular basis, but not autonomous |
| Battery powered implementation? | No |
| In situ pH calibration: what is needed/possible? | People take samples in the water Schedule: when first deployed, need to cal 1/day, then later 1/ week Depends very much on where they are and the temp variation No in situ pH cal required Not at end of river mouth |
| How will video measurements/observations be used? |  |
| What is required video resolution and frame rate? | Don’t need continuous video 10 frames/min HD resolution (journal publication requirement) 12 MPx probably. Coolpix. pan and tilt is upgrade |
| Define pH control performance specs Max offset pH range precision what current conditions | -0.5 8.2-7.0 0.03 pH (0.1 for some) |
| Define VH20 internal/external limits | 5 cm/sec int 50 cm/sec ext |
| Operating depth | Diver depth 130 ft/40 m |
| How to specify surge (if it it needed) | Needed; moving water back and forth |
| How to specify biofouling requirement | Hard to spec, Can document recommendations |
| What water velocity and flow measurements are required (flow (2D/3D?), velocity) | Sci want to know volume of water at lower pH went through box for how long… |
| What communication I/O interfaces must be supported | Not a requirement Look at drivers… We can offer recommendations… |
| Number of ESW injectors required | 1 per leg can be one leg, many would want 2 legs |
| Number of thrusters required |  |
| Chamber remotely sealable | Not required, upgrade Manual closed by diver |
| Near real-time telemetry period, content | Most apps will have surface expression where most hardware is kept. Majority will be on surface (shore on surface expression). Either run a cable or wireless solution. Can buy and plug in – options…we should offer some kind of communication port. Some apps will use solar, battery, wind, etc. |
| Data summarization requirement | Complete data storage on board (not dependent on telemetry link). |
| System data (power, environmental) monitoring Update rate, resolution | Sci requirement is that they get all the data, but not necessarily in real time. All data includes engr data. Must know health and status real-time. |
| Target biology experiments only, or other? What domain requirements are there (e.g. cameras) | Multi-disiplinary But majority of users will be biology driven, bringing in PIs for other components. MBARI drivers: social relevance |
| What are experiment and deployment durations | 3-4 month initial experiment and deployment ** it’s about getting in the water initially, not to solve problem once and for all *** will have to explain what’s reasonably needed to do (not set and forget) |
| Expansion | Erector set: enough to get themselves up and running. Target: creating a future world in a pH controlled environment (control total CO2). |