Integrating cytometers with sea-going mobile robots

Marine microbial communities play a critical role in Earth's biosphere. Phytoplankton produce at least 50% of breathable oxygen, regulate atmospheric CO2, and form the base of the marine food web. Toxic algal blooms have major effects on coastal ecosystems and fisheries as well as human health. Knowledge of phytoplankton distribution, diversity and abundance in the ocean is largely due to instruments called flow cytometers (Figure 1), which produce quantitative multi-parametric measurements of individual particles at rates that can exceed thousands of particles per second or more (Shapiro, 2003). Measurements made using cytometric parameters such as light scatter and fluorescence can reveal cell size and intrinsic pigments, which are useful in characterizing plankton taxonomy, abundance and viability (Figure 2). Most cytometric measurements are made from ships, buoys, and cable-to-shore observatories. Integration of cytometers into autonomous surface and underwater vehicles could greatly advance marine microbiology by enabling better spatial and temporal sampling, faster 'event response', could provide access to climate-sensitive polar under-ice regions, and may lower costs relative to ship operations. In this article we discuss instrument and vehicle development trends that point the way to these 'mobile robot cytometers'.

Long-duration untethered autonomous vehicles complement other data gathering platforms such as research vessels and moorings since they can sample broad areas and volumes, and have proven themselves revolutionary in the ocean sciences [Bellingham, 2011]. The inclusion of cytometers on  these vehicles would greatly advance marine microbiology, but this integration presents challenges. Currrent vehicles have limited size and power footprints which must be considered for successfully integrating cytometers. Traditional flow cytometric designs offer their own unique challenges for AUV integration. The typical hydrodynamic focusing required for optimal positioning of particles for  cytometric analysis adds the need for a source of particle-free “sheath” fluid. Laser light sources, focusing optics and detection systems must maintain precise alignment. In addition many desirable flow cytometric assays (total DNA quantitation, xyz) require processing of material before entering the flow cytometer for analysis. Data handling and analysis adds the final constraint to cytometer integration. Clearly integration of flow cytometers as AUV payloads requires optimized cytometer design.

A consideration of the minimum number of flow cytometric parameters that can provide reliable characterization of plankton is important for producing a focused instrument design. Mention that many marine applications do not require staining or special sample handling (Sosik et al, 2010).  In addition, novel approaches to the traditional flow cytometer design can further minimize the size/power footprint of an instrument destined for AUV implementation. The following instruments simplify sample handling by measuring intrinsic phytoplankton fluorescence without the need for staining cells beforehand or sorting them in the cytometer.

Parameters that have proven beneficial for marine application of cytometers include Coulter volume (ref), light scatter and fluorescence, time-of-flight pulse shape characterization, and imaging techniques. Some instruments have been developed and deployed that combine some of these techniques, such as the CytoBuoy (pulse shape and image capture), and the Imaging Flow CytoBot (image analysis). The wide variety of sizes  (0.2um-100um) and shapes (round, rodlike, chains) of marine phytoplankton adds a daunting challenge in designing one cytometric system that can accurately identify all plankton of interest. Traditional laboratory based cytometers have not been able to encompass this dynamic range of particle size analysis in one instrument without adding complexity to the system (Beckman Coulter Astrios EQ and Beckton Dickinson Fortessa), or designing systems specific to a portion of that dynamic range (Apogee A50 and Union Biometrica COPAS).

STATE OF THE ART

--Autonomous vehicle benefits and constraints

Autonomous surface vehicles (ASVs) are robot boats that move along the sea surface, and may be equipped with profilers for acquiring data at depth. Autonomous underwater vehicles (AUVs) move in three dimensions and may be propeller driven, buoyancy driven in the case of gliders, or a hybrid of both kinds of propulsion. Autonomous vehicles provide a number of benefits compare to manned vessels:

Automated surveys: autonomous vehicles can be dispatched to a specific location, and with underwater vehicles, to specific depths, enabling 2D and 3D surveys. Search can be commanded by operator, another vehicle or mooring, to regions of interest identified by other methods; or vehicle itself may alter search patterns in response to in situ sensor data.

Cost/scalability: as a point of comparison, a Wave Glider ASV (Manley and Wilcox, 2010) costs MBARI about $900/day to run; while the 41-meter RV Rachel Carson costs about $8,000/day. Due to lower cost of equipment and operation, autonomous vehicles are relatively easy scalable assets: multiple autonomous vehicles can be used easier than multiple research vessels.

Endurance: Some autonomous vehicles are capable of long deployments lasting weeks or even months, spanning thousands of kilometers; AUV examples include AutoSub LR (Furlong et al, 2012), LRAUV (Hobson et al, 2012), and the Spray glider (Davis et al, 2002). These vehicles enable continuous presence in region of interest, provide opportunities to observe microbial community changes over time, and enable quick response to events such as blooms.

Persistent presence: combination of lower cost, scalability, and endurance can greatly enhance data gathering in special and temporal domains.

Access to extreme environments. Rapid environmental and microbial change is occurring underneath polar ice, yet direct access by manned vehicles or divers is difficult. Recent discovery of major phytoplankton blooms beneath Arctic ice in the Chukchi Sea (Arrigo et al, 2012) points to our current limited knowledge of these important ecosystems. AUVs have already performed CTD and ice-thickness surveys underneath Arctic and Antarctic sea ice, and a cytometer payload could enable under-ice microbial surveys.

These benefits could be exploited for many cytometer applications, including projects evaluated at Monterey Bay Aquarium Research Institute.

CANON project:  this MBARI project aims to monitor microbial populations and their effects on ecosystems in and around Monterey Bay, emphasizing long-term measurements with autonomous platforms, vehicles and instruments (mbari.org, 2015). CANON includes studies of small phytoplankton (Prochlorococcus, Synechococcus, and eukaryotes) in offshore mesoscale gyres. Other studies emphasize the development lifecycle of algal blooms of larger cells in the inner bay, including potentially toxic Pseudo-nitzschia species. A mobile cytometer would complement physical, chemical sensors, samplers, and ecogenomic instruments such as the Environmental Sample Processor (see below).

Changes in microbial distributions in ice-covered Arctic waters. Li et al (2009) documented microbial population changes in the Arctic Ocean in concert with climate-driven changes, with picoplankton (< 2 microns) replacing nanoplankton such as diatoms (2 - 20 microns) over a period of several years. These changes may be associated with thinning ice thickness and areal extent as well as freshening and stratification of surface water. Potential shifts in phytoplankton and bacteria community structure will have major repercussions on carbon sequestration at depth in the ocean (Forest et al. 2011). Thus far most sampling has been performed in open water, and much more would be learned from under-ice sampling with a mobile cytometer.

But before mobile autonomous cytometers become reality, we must also consider the technical challenges of operating instruments aboard autonomous vehicles.

Power: Autonomous vehicles are designed to minimize non-propulsive power ("hotel load") in order to maximize endurance. Many of today’s cytometers use on order of 50 watts when sampling, and must sample for several minutes at least. These power levels could seriously decrease the endurance of small, long-duration AUVs.

Size: Even “small” marine cytometers such as SeaFlow in its present configuration, are roughly the size of a large microwave oven, and exceed the payload capacity of many vehicles.

Telemetry: Most autonomous vehicles use cell phone networks close when in range, but must use low-bandwidth expensive satellite links when far from shore or in remote regions. Limited telemetry links pose challenges for applications that require near real-time data telemetry to shore.

Environment: the vehicle and its cytometer should be operable within the depth and temperature range of the target phytoplankton, i.e. the photic zone; between 0 and about 200 meters, and from -1.8C to about 38C (in the tropics). The instrument should be rugged enough to withstand mechanical shock and vibration during a deployment and recovery cycle, and should be operable within expected tilt/pitch angles during a deployment.

Risk of vehicle loss: Today’s cytometers generally cost on order of $100K and more; if the vehicle is lost, then so is an expensive instrument. High cytometer cost also discourages procurement of multiple instruments, e.g. for deployment on a fleet of vehicles.

Table 1 shows characteristic payload volume and power characteristics for just a few autonomous vehicles with endurance of a week or longer; images of the vehicles themselves are shown in Figure 3.

While these conditions are daunting, we note that increasingly sophisticated instruments are being integrated with autonomous vehicles. MBARI's Environmental Sample Processor (ESP) is an eco-genomic “laboratory in a can”, processing samples with heat and reagents to extract and analyze microbial genetic material (Scholin, 2013), made possible by low-power electronics, microfluidic technologies, and stringent power-management strategies. The 3rd generation 3G-ESP is under development for low power mobile platforms, MBARI's Long Range AUV (LRAUV) in particular. 3G-ESP is contained in a nose section 28 cm diameter by 61 cm long, draws between 4 and 20 watts, and is rated to 50m depth. The prototype underwent successful initial sea trials in Monterey Bay in early 2015. Based on the 3G-ESP and other examples we believe that development of small low-power cytometers for autonomous vehicles is possible in the near future.

STAE OF THE ART

--Cytometer Integration and Design

At this point only one integrated flow cytometer carried by an AUV has been reported. Cunningham et al (2003) describe a flow cytometer integrated with the AutoSub AUV, which measured microbial populations across a bloom front in the North Atlantic in 2001. AutoSub was equipped with a basic science package as well as a water sampler and  was deployed into area of interest defined by SeaWIFS ocean color imaging. Cytometric data recorded during this brief hours-long deployment were largely consistent with onboard instruments as well as lab analysis of water samples.

Since 2003, several other cytometers have been developed for autonomous applications aboard ships or stationary platforms (buoys, cabled observatories).

The SeaFlow cytometer is designed to automatically acquire data over long periods with minimal human interaction; its primary deployment mode is aboard ships of opportunity where the instrument continually samples water from the ship's intake for weeks at a time (Swalwell et al, 2011). The SeaFlow "virtual core" design eliminates the need for sheath fluid by adding two additional detectors to determine when a cell is optimally positioned for  best optical measurement. The instrument analyzes cells in the size range of 0.5 - 20 m (pico- and nanoplankton), including prochlorococcus and synechococcus. SeaFlow enables extended two-dimensional surveys of near surface water (Figure 4). Palevsky et al (2013) used continuous underway SeaFlow data to help determine phytoplankton's contribution to the carbon dioxide air-sea flux in Alaskan waters.

Imaging Flow Cytobot (IFCB) is designed to automatically image, quantify and identify cells larger than 5 microns (Sosik and Olson, 2007). IFCB typically draws in a 5 ml water sample every twenty minutes, and captures detailed images when scatter or fluorescence PMT signal exceeds a preset threshold, which triggers a flash lamp and camera. Image resolution is typically about 0.3 microns per pixel, allowing many cells to be analyzed to the genus or even species level (Figure 5). IFCB has been deployed for up to six months on piers and cabled observatories. Campbell et al (2010) describe detection of toxic algal blooms (Dinophysis) with IFCB in a Texas estuary; public health officials quickly responded to the detection by suspending shellfish harvesting until the bloom subsided. IFCB draws considerable electrical power (35 W continuously) and generates several gigabytes of data per day; thus the instrument is usually deployed at locations with wired connections to shore power and data. IFCB was recently deployed on a moored raft in Salt Pond (Nauset, MA), with no wired shore connection. The raft was equipped with solar panels, a bank of batteries and a gasoline generator that automatically recharged the batteries as required. Communication to shore was via an ethernet radio link to a shore-based internet connection (Olson, personal communication).

THE FUTURE

Electronics and fluidics have evolved in the years since the first AutoSub cytometer deployments (Cunningham et al, 2003), resulting in smaller, lower-power instruments which will likely be compatible with mobile marine robots in the near future.

Sosik and Olson are currently modifying their Imaging Flow Cytobot for integration with autonomous surface vehicles such as Wave Glider, JetYak (whoi.edu, 2014), and AutoNaut (AutonautUsv.com, 2015). The instrument will be towed behind the vehicle at a depth of several meters. They anticipate the vehicle will provide sufficient power to sample with the instrument once every 20 minutes. All raw data and images will be stored onboard for post-recovery analysis. Selected images and perhaps some processed data will be returned to shore over the available telemetry channels in near real-time; 3G/4G celluluar modem (0.5-4 Mbps) in coastal areas, Iridium satellite (2400 bps) in more remote regions (Sosik, personal communication).

While the Imaging Flow Cytobot vehicle will result in invaluable data from near-surface waters, the instrument is currently too large and consumes too much power for small, long duration AUVs such as LRAUV, capable of surveys throughout the photic zone. New marine cytometers now under development begin to address these AUV constraints. J. Swalwell of the University of Washington is developing an "immersion flow cytometer" that increases signal-to-noise, simplifies flow cell design, and potentially lowers cost (US Provisional Patent Application Serial No. 62/101,036). The instrument utilizes a low-cost primary ball lens with high index of refraction that is partly immersed in the seawater sample. Analogous to oil or water immersion microscopy, the immersion results in an estimated 60% higher numerical aperture than found in SeaFlow and BD Influx cytometers. Chromatic aberration in the ball lens causes laser and fluorescent colors to be dispersed behind the lens, similar to the effects of a prism or grating. Filters and detectors for the wavelengths of interest can be optimally positioned behind the lens, resulting in lower contamination by scattered light at each wavelength and a corresponding signal-to-noise increase. Since the primary lens is immersed directly in the sample, the need for small orifices and sample-delivery tubing may be reduced or eliminated. The instrument utilizes the same "virtual core" technology as the SeaFlow cytometer (Swalwell et al, 2011), eliminating the need for sheath fluid. The cheaper lens, improved signal-to-noise, and simplified fluidic design is expected to result in a smaller (shoebox sized), lower power (roughly 15W) and lower cost instrument (Swalwell, personal communication), with a high potential for deployment on an autonomous vehicle.

Wu et al (2013) describe an imaging approach where a plane of 450 nm laser light (“light sheet”) is used to illuminate  cross-sectional fluorescent images , acquired sequentially, as phytoplankton flow through the light sheet. The instrument generates an integrated cross sectional image of each cell, and is capable of flow rates of at least 1 ml/minute (four times the flow rate of Imaging Flow Cytobot). The image quality is such that cells larger than about 5 microns can often be taxonomically identified; Orientation of the cells can vary as they pass through the light sheet, and the authors discuss taxonomic identification approaches that take this into account. The authors intend to miniaturize their prototype such that it is compatible with small autonomous platforms such as moorings and AUVs (R. Chan, personal communication).

Though it might not qualify as "traditional" cytometry, holographic microscopy is another technique to automatically measure individual cell optical characteristics. Conventional microscope optics are replaced by a laser, pinhole, and digital camera chip. The device acquires holograms, recording three dimensional information from particles within the analyzed volume, eliminating the need to physically align the cells in single-file. Cell images can be reconstructed as viewed from multiple angles. Graham and Smith (2010) describe a submersible holographic microscope with 7.4 microns spatial resolution; a commercial version of the instrument has been integrated with MBARI's Dorado AUV and is being evaluated for analysis of large phytoplankton and zooplankton (sequoiasci.com, 2015).  Bochdanksy et al (2013) describe a submersible instrument that acquires several holograms per second; a commercial version of this instrument claims sub-micron spatial resolution (4deep.com, 2015); its physical dimensions (30 cm length and 10 cm diameter) make it a potential candidate for integration with a small autonomous vehicle.

Small low-powered cytometers may someday be capable of the many applications that require cell pre-staining and sorting.  Recent advances in microfluidics and manipulation techniques embodied by small ecogenomic sensors such as the AUV-integrated Environmental Sample Processor may point the way in this regard (Scholin, 2013).

Consumer markets such as mobile phones are rapidly driving the miniaturization and cost-reduction of batteries, computers, cameras, and other sensors. Small, simple low-cost imaging cytometers have been implemented on mobile phones for blood analysis and other medical applications in the developing world (Mudyani et al, 2012). Perhaps some of these innovations could be explored for marine applications as well.

Once a cytometer is integrated with a vehicle, "adaptive sampling" could be used to conserve power and telemetry bandwidth, turning on the instrument only when there is likely something interesting to sample.  Zhang et al describe water sampling of thin phytoplankton layers by MBARI’s Dorado AUV. The AUV flies a vertical saw-tooth profile through regions that may contain layers, and their algorithm triggers a water sampling bottle when fluorescence and backscatter sensors indicate that the vehicle is moving through the center of a layer. Godin et al describe an algorithm to track moving patches of phytoplankton. The algorithm adjusts vehicle speed, heading and depth based on the chlorophyll signal from an onboard fluorometer, enabling the vehicle to remain near the centroid of the phytoplankton patch as it drifts with the current. The algorithm could be adapted to power on a cytometer only when the vehicle is in the patch, e.g. allowing the instrument to capture the growth and decline of microbes with time.

Some applications require real-time telemetry of cytometer data to shore, e.g. to issue health warnings in case of a harmful algal bloom. But many cytometers generate gigabytes of raw data each day, and telemetry of this data volume is problematic. In coastal regions the vehicle may use a 3G/4G cell modem, but beyond 100 km from shore an Iridium satellite link (2400 bits per second, compressed) may be the only option (Kocak and Clark (2013) note that faster satellite links may be available in the near future). These slow links can result in very long transmission times, high modem energy consumption, high airtime costs, and long surface times for AUVs.  Instead of telemetering the entire dataset, onboard processing could return a brief data summary. In the case of cytometer fluorescence and scatter data, onboard cluster analysis could produce a simple list of which groups are detected [REF - Peter]. Alternatively Li (1997) describes a “cytometric diversity index" in the form of a few floating point numbers that is calculated based on light scatter and chlorophyll fluorescence, and can be used to estimate the structure of phytoplankton communities in real-time, e.g. to track blooms or biological fronts. In the case of image data, Sosik and Olsen (2007) describe image processing "machine learning" software that classifies individual phytoplankton cells to the genus or species level, based on size, shape, symmetry, texture and other characteristics. Their software currently runs on shore-side machines, but they hope to adapt it to the smaller processors on autonomous vehicles.

In summary, although autonomous vehicles present a challenging platform for deployment of cytometric systems , progress in this integration has been rapid and viable candidate solutions are available.

Acknowledgements

Brent Jones, Brian Kieft, Yanwu Zhang, Amy Zimmerman (MBARI), Jim Bellingham, Heidi Sosik, Rob Olsen (WHOI), George Dubelar (CytoSense), Jeffrey T Sherman (UCSD), Harry Nelson (Fluid Imaging Technologies), Stephen Jones and John Samson (4Deep)

|  | Wave Glider SV3 | Spray glider | Long Range AUV |
| --- | --- | --- | --- |
| Type | wave-propelled autonomous surface vehicle | buoyancy-driven glider | propeller-buoyancy hybrid |
| Approximate payload dimensions (cm) | deck: 380x380x127 (2) 10x37x17 (1) submerged: 15 dia x 100 len | 11 dia x 15 len | 30 dia x 33 len |
| Max payload weight (kg) | 45 | 3.5 | 10 |
| Hotel load (watts) | 5 - 20 | 1 | 50+ |
| Payload energy | 150W solar panels, 16 MJ storage batteries | 13 MJ (Li primaries) | 18 MJ (Li secondaries) 40 MJ (Li primaries) |
| Typical endurance | months | 88 days, 354 profiles | 27 days, 1000 km |

Table 1: Some key characteristics of the Liquid Robotics Wave Glider SV3, Scripps' Spray glider, and MBARI's Long Range AUV. Actual endurance will depend on energy needs of integrated instruments, including a cytometer.

References

4deep.com. 2015. Submersible microscope. Available at http://4-deep.com/submersible-microscope/ (accessed 24 February 2015).

Arrigo, K. R., Perovich, D. K., Pickart, R. S., Brown, Z. W., van Dijken, G. L., Lowry, K. E., ... & Swift, J. H. (2012). Massive phytoplankton blooms under Arctic sea ice. Science, 336(6087), 1408-1408.

AutoNautUsv.com. 2015. http://www.autonautusv.com (accessed 24 February 2015).

Bochdansky, A. B., Jericho, M. H., & Herndl, G. J. (2013). Development and deployment of a point‐source digital inline holographic microscope for the study of plankton and particles to a depth of 6000 m. Limnology and Oceanography: Methods, 11(1), 28-40.

Chisholm, S. W., Olson, R. J., Zettler, E. R., Goericke, R., Waterbury, J. B., & Welschmeyer, N. A. (1988). A novel free-living prochlorophyte abundant in the oceanic euphotic zone. Nature, 334, 340-343.

Cunningham, A., McKee, D., Craig, S., Tarran, G., Widdicombe, C. 2003. Fine-scale variability in phytoplankton community structure and inherent optical properties measured from an autonomous underwater vehicle. Journal of Marine Systems, 43 (1–2) (2003), pp. 51–59

Davis, R. E., Eriksen, C. C., & Jones, C. P. (2002). Autonomous buoyancy-driven underwater gliders. The Technology and Applications of Autonomous Underwater Vehicles, 37-58.

Forest, A., Tremblay, J. É., Gratton, Y., Martin, J., Gagnon, J., Darnis, G., ... & Piepenburg, D. (2011). Biogenic carbon flows through the planktonic food web of the Amundsen Gulf (Arctic Ocean): A synthesis of field measurements and inverse modeling analyses. Progress in Oceanography, 91(4), 410-436.

Frankel, S. L., Binder, B. J., Chisholm, S. W., & Shapiro, H. M. (1990). High‐sensitivity flow cytometer for studying picoplankton. Limnology and oceanography, 35(5), 1164-1169.

Frankel, D. S., Olson, R. J., Frankel, S. L., & Chisholm, S. W. (1989). Use of a neural net computer system for analysis of flow cytometric data of phytoplankton populations. Cytometry, 10(5), 540-550.

Furlong, M. E., Paxton, D., Stevenson, P., Pebody, M., McPhail, S. D., & Perrett, J. (2012, September). Autosub Long Range: A long range deep diving AUV for ocean monitoring. In Autonomous Underwater Vehicles (AUV), 2012 IEEE/OES (pp. 1-7). IEEE.

Godin, M. A., Ryan, J. P., Zhang, Y., & Bellingham, J. G. (2012, December). Localization and Tracking of Submerged Phytoplankton Bloom Patches by an Autonomous Underwater Vehicle. In AGU Fall Meeting Abstracts (Vol. 1, p. 1869).

Graham, G. W., & Nimmo Smith, W. A. M. (2010). The application of holography to the analysis of size and settling velocity of suspended cohesive sediments. Limnology and Oceanography: Methods, 8(1), 1-15.

Gucker Jr, F. T., O'Konski, C. T., Pickard, H. B., & Pitts Jr, J. N. (1947). A Photoelectronic Counter for Colloidal Particles1. Journal of the American Chemical Society, 69(10), 2422-2431.

Hobson, B. W., Bellingham, J. G., Kieft, B., McEwen, R., Godin, M., & Zhang, Y. (2012, September). Tethys-class long range AUVs-extending the endurance of propeller-driven cruising AUVs from days to weeks. In Autonomous Underwater Vehicles (AUV), 2012 IEEE/OES (pp. 1-8). IEEE.

Li W.K. 1997. Cytometric diversity in marine ultraphytoplankton. Limnol Oceanogr 42:874–80.

Li, W.K., McLaughlin, F. A., Lovejoy, C., & Carmack, E. C. (2009). Smallest algae thrive as the Arctic Ocean freshens. Science, 326(5952), 539-539.

Lomas, M. W., Bronk, D. A., & van den Engh, G. (2011). Use of flow cytometry to measure biogeochemical rates and processes in the ocean. Annual review of marine science, 3, 537-566.

Manley, J., & Willcox, S. (2010, May). The wave glider: A persistent platform for ocean science. In OCEANS 2010 IEEE-Sydney (pp. 1-5). IEEE.

MBARI.org. 2015. CANON: Controlled, agile, and novel observing network. Available at http://www.mbari.org/canon/ (accessed 25 February 2015)

McEwen, R. S., Hobson, B. W., McBride, L., & Bellingham, J. G. (2008). Docking control system for a 54-cm-diameter (21-in) AUV. Oceanic Engineering, IEEE Journal of, 33(4), 550-562.

Mudanyali,O., Dimitrov S., Sikora U,Padmanabhan S.,Navruza I., and Ozcan A. Integrated rapid-diagnostic-test reader platform on a cellphone Lab Chip, 2012,12, 2678–2686

Olson, R. J., & Sosik, H. M. (2007). A submersible imaging‐in‐flow instrument to analyze nano‐and microplankton: Imaging FlowCytobot. Limnology and Oceanography: Methods, 5(6), 195-203.

Palevsky, H. I., Ribalet, F., Swalwell, J. E., Cosca, C. E., Cokelet, E. D., Feely, R. A., & Quay, P. D. (2013). The influence of net community production and phytoplankton community structure on CO2 uptake in the Gulf of Alaska. Global Biogeochemical Cycles, 27(3), 664-676.

Scholin CA. 2013. Ecogenomic sensors. In: Levin SA, (ed.) Encyclopedia of Biodiversity 2nd edn Volume 2. Academic Press: Waltham, MA, 690–700.

See, J. H., Campbell, L., Richardson, T. L., Pinckney, J. L., Shen, R., & Guinasso, N. L. (2005). Combining new technologies for determination of phytoplankton community structure in the Northern Gulf of Mexico 1. Journal of phycology, 41(2), 305-310.

sequoiasci.com. 2015. Testing Sequoia's LISST-Holo on an autonomous underwater vehicle. Available at http://www.sequoiasci.com/article/testing-sequoias-lisst-%C2%ADholo-on-an-autonomous-underwater-vehicle/ (accessed 24 February 2015).

Shapiro, H.M. Practical Flow Cytometry, 4th ed, 2003

Sosik, H. M., & Olson, R. J. (2007). Automated taxonomic classification of phytoplankton sampled with imaging‐in‐flow cytometry. Limnology and Oceanography: Methods, 5(6), 204-216.

Sosik, H.M., Olson, R.J., & Armbrust, E.V. 2010. Flow cytometry in phytoplankton research. Chlorophyll a Fluorescence in Aquatic Sciences: Methods and Applications. pp.171–185

Swalwell, J. E., Ribalet, F., & Armbrust, E. (2011). SeaFlow: A novel underway flow‐cytometer for continuous observations of phytoplankton in the ocean. Limnology and Oceanography: Methods, 9(10), 466-477.

Wadhams P, Wilkinson JP, McPhail SD. 2006. A new view of the underside of Arctic sea ice. Geophysical Research Letters. 33:L04501. doi: 10.1029/2005GL025131

Whoi.edu. 2014. The JetYak. Available at https://vimeo.com/113751116 (accessed 25 February 2015).

Willcox, J. S., Bellingham, J. G., Zhang, Y., & Baggeroer, A. B. (2001). Performance metrics for oceanographic surveys with autonomous underwater vehicles. Oceanic Engineering, IEEE Journal of, 26(4), 711-725.

Williams, G.D., Wilkinson, J., Maksym, T., Kunz, C. & Singh, H. 2014. Very thick and heavily deformed Antarctic sea ice captured in 3-D by autonomous underwater vehicle, International Symposium on Sea Ice in a Changing Environment - Proceedings of the Hobart Symposium, 10-14 March 2014, Hobart, Tasmania, Australia, pp. 69A834

Wu, J, Chan, RKY. 2013. A fast fluorescence imaging flow cytometer for phytoplankton analysis. Opt Express Oct 7;21(20):23921-6. doi: 10.1364/OE.21.023921

Yentsch, C. S., & Yentsch, C. M. (2008). Single cell analysis in biological oceanography and its evolutionary implications. Journal of plankton research,30(2), 107-117.

Zhang, Y., McEwen, R.S., Ryan,  J.P., Bellingham,  J.G., Harvey, J. & Vrijenhoek, J. 2010. Acquiring peak samples from phytoplankton thin layers and intermediate nepheloid layers by an autonomous underwater vehicle with adaptive triggering. AGU 2010 Fall Meeting, Abstract OS51C-1334, San Francisco, CA, U.S.A., December

2010.

Figure captions

Figure 1. Schematic diagram of a flow cytometer.

Figure 2. A 'dot plot' showing correlation between fluorescence and scattered light signals, measured by a BD Influx flow cytometer.

Figure 3. Some long-duration autonomous vehicles; (a) Wave Glider (b) Spray glider (c) Long Range AUV

Figure 4. Distribution of (a) prochlorococcus, (b) Synechococcus and (c) nanoplankton in Hawaiian waters, as measured by the SeaFlow cytometer. Large spatial scale views are the left, finer scale on the right.  (used with permission, from Swalwell et al, 2011).

Figure 5. Monterey Bay phytoplankton images captured by Imaging Flow Cytobot. (Instrument loaned to MBARI courtesy of Heidi Sosik and Rob Olson)