| SV2 Wave Glider Pre-Launch Checklist |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Date: 08DEC2014 |  |  |  |  | Glider Name: OA Wave Glider |  |  |  |  |
| Operator: Chris Wahl |  |  |  |  | Payload: pCO2, CTD + DO and VR2C |  |  |  |  |
| Ship/Op Area: Monterey Bay |  |  |  |  | Project: 901310 |  |  |  |  |
| ##### Communication |  |  |  |  |  |  |  |  |  |
| Item/Procedure |  | # Readings |  |  | # Specifications |  |  |  | Initials |
| Xbee Connection |  |  |  |  | Coms Established |  |  |  | CDW |
| WGMS Updating |  |  |  |  | Coms Established |  |  |  | CDW |
| ##### Configuration |  |  |  |  |  |  |  |  |  |
| Print xbee commands |  |  |  |  | <CR>,26,27,97 (attach to this doc) |  |  |  | CDW |
| RDF Frequency |  | Top:N/A expired Bottom:N/A expired |  |  |  |  |  |  | CDW |
| Tracker name/# |  | Tracker 45 |  |  |  |  |  |  | CDW |
| Float name |  | OA Wave Glider |  |  |  |  |  |  | CDW |
| Glider name |  | OA Wave Glider |  |  |  |  |  |  | CDW |
| Drybox name |  | OA Wave Glider |  |  |  |  |  |  | CDW |
| # Vehicle Status |  |  |  |  |  |  |  |  |  |
| Battery Wh |  | 646.29 |  |  | > 550 Watt-hours |  |  |  | CDW |
| Solar Power Indicators: |  | Panel 1 | Panel 2 |  |  |  |  |  |  |
|  |  | 248mW | 1937mW |  | > 1000 mW |  |  |  | CDW |
| Environmental: |  | Pressure | Temp |  | > 135 kpa, < 35 C <50% < 105 kpa, < 35 C > 135 kpa, < 35 C (Plastic PL ~100 kpa) < 50% > 135 kpa, < 35 C < 50% |  |  |  |  |
| C&C Drybox (Float) |  | 127 | 28 |  |  |  |  |  | CDW |
| C&C humidity |  | 46 |  |  |  |  |  |  | CDW |
| Rudder Module (Sub) |  | 112 | 38 |  |  |  |  |  | CDW |
| Payload 1 |  | 100 | 20 |  |  |  |  |  | CDW |
| Payload humidity |  | 66! |  |  |  |  |  |  | CDW |
| Payload 2 |  | 115 | 20 |  |  |  |  |  | CDW |
| Payload humidity |  | 9 |  |  |  |  |  |  | CDW |
| # Mechanical Checks |  |  |  |  |  |  |  |  |  |
| Float |  |  |  |  |  |  |  |  |  |
| Wet-mates connected/lubed |  |  |  |  |  |  |  |  | CDW |
| Connector covers secure |  |  |  |  | 30 in/lbs |  |  |  | CDW |
| Drybox secure |  |  |  |  | 30 in/lbs |  |  |  | CDW |
| Turtledeck secure |  |  |  |  | 30 in/lbs |  |  |  | CDW |
| Contact info secure |  |  |  |  |  |  |  |  | CDW |
| Remove RDF magnet |  |  |  |  | Top & Bottom RDF |  |  |  | N/A |
| Orange flag attached to mast |  |  |  |  |  |  |  |  | CDW |
| Cable tie on shorting pin |  |  |  |  | Secure shorting pin |  |  |  | CDW |
| Glider |  |  |  |  |  |  |  |  |  |
| Line Terminator secure |  |  |  |  | Rubber mallet treatment |  |  |  | CDW |
| Umbilical visual inspection |  |  |  |  | Free of voids/defects |  |  |  | CDW |
| Roll pin inserted |  |  |  |  |  |  |  |  | CDW |
| ###### Vehicle Functionality – WGMS |  |  |  |  |  |  |  |  |  |
| Tracker report rate/reporting |  | Time |  | Yes/No |  |  |  |  |  |
|  |  | 6 hrs |  | Yes |  |  |  |  | CDW |
| Light on/off |  |  |  |  |  |  |  |  | CDW |
| IPIB sensors powered on |  |  |  |  | Check all IPIB sensors; Current Draw ~350mA |  |  |  | CDW |
| Rudder moves 360 |  |  |  |  |  |  |  |  | CDW |
| Rudder center |  |  |  |  |  |  |  |  | CDW |
| Rudder left |  |  |  |  |  |  |  |  | CDW |
| Rudder right |  |  |  |  |  |  |  |  | CDW |
| Fixed heading test (spin) |  | 100 |  |  | Heading used |  |  |  | CDW |
| Vehicle Functionality – WGMS Cont. |  |  |  |  |  |  |  |  |  |
| Item/Procedure |  | Readings |  |  | Specifications |  |  | Initials |  |
| Waypoint course test (spin) |  | 2 |  |  | Waypoint number used, full vehicle spin |  |  | CDW |  |
| Op area defined |  |  |  |  | Monterey Bay, c1 to m1 to m2 loop |  |  | CDW |  |
| Photo document |  |  |  |  | Turtledeck, umbilical, rudder, spring bar, wing spring hardware, etc. |  |  | CDW |  |
| Event definitions |  |  |  |  | Compare against standard list |  |  | ? Not changed |  |
| Set default comms |  |  |  |  | Change to iridium or alt-iridium |  |  | CDW |  |
| Check alarms |  |  |  |  |  |  |  | CDW |  |
| Payload Checks |  |  |  |  |  |  |  |  |  |
| Sensor | Power On/Off |  |  |  |  | Current Time | Last Packet |  |  |
| AIS | Yes |  |  |  |  | 1432 | 1446 |  |  |
| Weather Station | Yes |  |  |  |  | 1432 | 1450 |  |  |
| Water Speed | Yes |  |  |  |  | 1432 | 1450 |  |  |
| CTD | Yes |  |  |  |  | 1432 | 0959 |  |  |
| pCO2 | Yes |  |  |  |  | 1432 | 0923 |  |  |
| VR2C | Yes |  |  |  |  | 1432 | 0949 |  |  |
| Comments |  |  |  |  |  |  |  |  |  |
| -Solar panels may have problems? Power output low, but batteries are fully charged? |  |  |  |  |  |  |  |  |  |
| -Umbilical rubber damaged in some spots, but SS cable is fine, should be fine for our short test deployment |  |  |  |  |  |  |  |  |  |
| -Oxygen data from test tank testing does not look good |  |  |  |  |  |  |  |  |  |
| -Desiccant was forgotten in payload 1, this is okay for this short test deployment and will be added for future deployments |  |  |  |  |  |  |  |  |  |

Technician Signature:		Chris Wahl

Note:  Once vehicle is launched the following tasks must be completed

- Clear alarms

- Vehicle on mission

- Create LAUNCHED comment into WGMS.

- Upload photos to box.com with album name: YYYY-MM-DD VehicleName

26

Pyld1=-1mA, Pyld2=-1mA, Umb=4mA, iPib=0mA (-1=Off, -32768=Over Current)

Pyld3=-1mA, Pyld4=-1mA, PEP=-1mA, Lightbar=-1mA (-1=Off, -32768=Over Current)

Total Pwr=645086mWh, Bat Cur=0mA, Sys Cur=47mA, Sys Volt=21924mV

Pyld Volt=13902mV, Pyld Cur=25mA, Modem Cur=13mA

Panel0=20mW, Panel1=2198mW, Aux=22mW, Rsvrd=0mW

27

0x01=Present, 0x02=Discharged, 0x04=Charged, 0x08=Discharging

0x10=Discharge Alarm, 0x20=Charge Alarm, 0x40=Temp Alarm, 0x80=On-Line

Battery 0: Status=2d, Temp = 20C

Power=9303 (10mWh), Current=0mA, Voltage=16688mV

Battery 1: Status=2d, Temp = 20C

Power=9337 (10mWh), Current=0mA, Voltage=16676mV

Battery 2: Status=2d, Temp = 20C

Power=9402 (10mWh), Current=0mA, Voltage=16275mV

Battery 3: Status=2d, Temp = 20C

Power=9090 (10mWh), Current=0mA, Voltage=16688mV

Battery 4: Status=2d, Temp = 20C

Power=9159 (10mWh), Current=0mA, Voltage=16678mV

Battery 5: Status=2d, Temp = 20C

Power=9172 (10mWh), Current=0mA, Voltage=16686mV

Battery 6: Status=2d, Temp = 21C

Power=9046 (10mWh), Current=0mA, Voltage=16681mV

97

Float SW Version 01.05 Bld 0826

Float Boot SW Version 01.00 Bld 0001

Rudder SW Ver 01.06 -- HW Ver 02.00

Radome SW Ver 20.240 -- HW Ver 01.02

Sensor Module SW Ver 00.00 -- HW Ver 00.00

Battery Charger SW Ver 22.62 -- HW Ver 255.255

Hardware ID:  0x430176293639473005D7FF39

Battery #0: manufactured date = 03/10/2011, SerNum = 02463, Version = 3.1

Battery #1: manufactured date = 03/10/2011, SerNum = 02391, Version = 3.1

Battery #2: manufactured date = 03/10/2011, SerNum = 02360, Version = 3.1

Battery #3: manufactured date = 03/10/2011, SerNum = 02486, Version = 3.1

Battery #4: manufactured date = 03/10/2011, SerNum = 02380, Version = 3.1

Battery #5: manufactured date = 03/10/2011, SerNum = 02495, Version = 3.1

Battery #6: manufactured date = 03/10/2011, SerNum = 02362, Version = 3.1

System is DB 3.5