| MISSION PLAN |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Customer: |  | MBARI – Thom Maughan |  |  | Op Area & Wave Gliders: |  | Monterey Bay, within 20 miles of Moss Landing |  |  |
| Mission Date Range: |  | Mar 26,2013 – Apr 30, 2013 |  |  | Broad Mission Description: |  | Launch / Recovery training, Waveglider ‘Tex’ shakeout mission |  |  |
| Primary Client Contact Info: |  | Thom Maughan [tm@mbari.org] [831-775-1865] |  |  | LR Program Manager: |  | ? |  |  |
| LR Account Representative: |  | Jamie Griffith [email@domain.com] [phone] |  |  | LR WGOC Director: |  | John Appelgren john.appelgren@liquidr.com +1 408.636.4260 |  |  |
| Project Team: | Thom Maughan, Project Mgr, Software Eng, WG Ops, MBARI, tm@mbari.org, 831-775-1865 Paul Coenen, Electro-Mech Tech, WG Ops, RV Paragon Captain, [name – title – company – contact info] [name – title – company – contact info] [name – title – company – contact info] [name – title – company – contact info] |  |  |  | Deployment (LR Resources): |  |  | [name] [email@domain.com] [phone] |  |
|  |  |  |  |  | Local Authorities: |  |  | [national agency – contact info] [local agency – contact info] |  |
| MISSION SCHEDULE |  |  |  |  |  |  |  |  |  |
|  |  |  | Date |  | Port |  | Vessel |  |  |
| Sortie 1 (launch): |  |  | 130326 |  | Moss Landing, California |  | RV Paragon, Paul Coenen, |  |  |
| Sortie 1 (recover): |  |  | 130326 |  | Moss Landing, California |  | RV Paragon, Paul Coenen, |  |  |
| Sortie 2 (launch): |  |  | 130327 |  | Moss Landing, California |  | RV Paragon, Paul Coenen, |  |  |
| Sortie 2 (recover): |  |  | 130327 |  | Moss Landing, California |  | RV Paragon, Paul Coenen, |  |  |
| Sortie 3 (launch): |  |  | 130328 |  | Moss Landing, California |  | RV Paragon, Paul Coenen, |  |  |
| Sortie 3 (recover): |  |  | 130402 |  | Moss Landing, California |  | RV Paragon, Paul Coenen, |  |  |
| Sortie 4 (launch): |  |  | 130417 |  | Moss Landing, California |  | RV Paragon, Paul Coenen, |  |  |
| Sortie 4 (recover): |  |  | 130508 |  | Moss Landing, California |  | RV Paragon, Paul Coenen, |  |  |
| WAVE GLIDERS & WAVE GLIDER MANAGEMENT SYSTEM |  |  |  |  |  |  |  |  |  |
| WGMS Org: |  |  |  | mbari.wgms.com |  |  |  |  |  |
| Wave Gliders: |  |  |  | [name = ‘Tex’] [Tracker #87]; - See vehicle parameter for configuration/payload details |  |  |  |  |  |
| WGMS Provisioning: |  |  |  | Submit request to LR Wave Glider Operations Center (opscenter@liquidr.com) - User name, email address, phone#, & LR Account Representative authorization required |  |  |  |  |  |

| VEHICLE PARAMETERS |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| WG1 - Configuration/Payload Sensors |  |  |  |  |  |  |  |  |  |  |  |
| Glider Name: | AIS | Wx | WS | Light | Flag | Color | RDF Top (mhz) | RDF Bottom (mhz) | Unique ID |  | Security Manifest |
| Tex | Y | Y | Y | Y | Y | Y | 151.635 | 151.623 | 65E51C59 |  | YN |
|  |  | weatherstation | Speed sensor |  |  |  |  |  |  |  |  |
| Tracker Name: | [name] |  |  |  |  |  | Track Report Freq.: |  | [##] hours |  |  |
| Device Details: | Address: Hex & Dec |  |  |  |  |  |  | Port: PL1/2/3 |  | Sub PL Pwr (Y/N) |  |
| Aft Payload SMC and GPCTD | 0x#### (####) |  |  |  |  |  |  | PL1 |  | YN |  |
| Not Connected | 0x#### (####) |  |  |  |  |  |  | PL2 |  | YN |  |
| Active Radar Reflector | 0x#### (####); Unique name |  |  |  |  |  |  | PL3 |  | YN |  |
| Not Connected | 0x#### (####) |  |  |  |  |  |  | AUX Pwr |  | YN |  |
| Payload Duty Cycle Operational Notes: None |  |  |  |  |  |  |  |  |  |  |  |
| WG2 - Configuration/Payload Devices |  |  |  |  |  |  |  |  |  |  |  |
| Glider Name: | AIS | Wx | WS | Light | Flag | Color | RDF Top (mhz) | RDF Bottom (mhz) | Unique ID |  | Security Manifest |
| [name] | YN | YN | YN | YN | YN |  | ###.### | ###.### | ######## |  | YN |
| Tracker Name: | [name] |  |  |  |  |  | Track Report Freq.: |  | [##] hours |  |  |
| Device Details: | Address: Hex & Dec |  |  |  |  |  |  | Port: PL1/2/3 |  | Sub PL Pwr (Y/N) |  |
| [device name] | 0x#### (####) |  |  |  |  |  |  | # |  | YN |  |
| [device name] | 0x#### (####) |  |  |  |  |  |  | # |  | YN |  |
| [device name] | 0x#### (####); Unique name |  |  |  |  |  |  | # |  | YN |  |
| [device name] | 0x#### (####) |  |  |  |  |  |  | # |  | YN |  |
| Payload Duty Cycle Operational Notes: [if any] |  |  |  |  |  |  |  |  |  |  |  |
| WG3 - Configuration/Payload Sensors |  |  |  |  |  |  |  |  |  |  |  |
| Glider Name: | AIS | Wx | WS | Light | Flag | Color | RDF Top (mhz) | RDF Bottom (mhz) | Unique ID |  | Security Manifest |
| [name] | YN | YN | YN | YN | YN |  | ###.### | ###.### | ######## |  | YN |
| Tracker Name: | [name] |  |  |  |  |  | Track Report Freq.: |  | [##] hours |  |  |
| Device Details: | Address: Hex & Dec |  |  |  |  |  |  | Port: PL1/2/3 |  | Sub PL Pwr (Y/N) |  |
| [device name] | 0x#### (####) |  |  |  |  |  |  | # |  | YN |  |
| [device name] | 0x#### (####) |  |  |  |  |  |  | # |  | YN |  |
| [device name] | 0x#### (####); Unique name |  |  |  |  |  |  | # |  | YN |  |
| [device name] | 0x#### (####) |  |  |  |  |  |  | # |  | YN |  |
| Payload Duty Cycle Operational Notes: [if any] |  |  |  |  |  |  |  |  |  |  |  |
| Details of devices listed above: Sensors; Make and model; Function. Fluorometer and Backscatter sensor; Wetlabs ECOBB2F; Fluorometer and Backscatter sensor for chlorophyll and turbidity-like measurements. |

| MISSION OBJECTIVES |  |
| --- | --- |
| Aim: Sortie 1 is training by Liquid Robotics for MBARI (Thom Maughan and Paul Coenen) on Waveglider launch and recovery using the RV Paragon and the ‘as built’ stock Waveglider (‘Tex’). We anticipate going in and out of the water several times with someone from Liquid Robotics operations team providing instruction and guidance and then finally observing and certifying the proper operation. We’ll also try launch and recovery with the towbody. Sortie 2: is Thom Maughan and Paul Coenen practicing and documenting waveglider launch and recovery Sortie 3 is the Waveglider shakeout cruise of several days to insure the ‘as built’ Waveglider provided to MBARI on the academic lease program is verified seaworthy. Sortie 4: is deployment of a reconfigured waveglider. Aft payload box removed and converted to shore side development SMC. Forward payload integrated with GPCTD on the float and Wetlabs fluorometer on the sub. Also integrated on the float will be a Freewave (or similar) radio. Background: [narrative]. Study Area: Monterey Bay operating over the Canyon out of the way of nominal ship traffic between former M0 mooring location and M1 mooring location. Deployment: [narrative with vicinity coordinates]. Sorties: Make best speed following systems validation to tasks: - [narrative with detailed coordinates]. - [narrative with detailed coordinates]. - [narrative with detailed coordinates]. Recovery: Make best speed following tasks to loiter area (vicinity coordinates). |  |
| DELIVERABLES |  |
| Mission Plan Authorization: [operational risk assessment; executive authorization to proceed] LR Field Resource Logistics: [as may be applicable]. Pre-Launch: Core systems, sensor calibration verification, Ops and safety review preceding each sortie. Training: [as may be applicable]. Vessel Readiness: Minimum hoisting specs (130 kg nominal weight in air; hoisting forces to recover will be higher; recovery typically done in two lifts… float, and then glider; each weighs ~65 kg (configuration dependent). Acceptance Tests: [as may be applicable]. Freight Forwarding Logistics: [as may be applicable]. Field Replaceable Units (FRUs/Spares): [as may be applicable]. Proposed Sea Trials: [as may be applicable]. Data: [as may be applicable]. Reports: Post-deployment; mission operations; post-recovery. - Post-deployment - authored by LR deployment resource; delivered to WGOC; mission case appended to; distributed to Field Operations (expanded distribution to effect quality improvement initiatives). - Mission operations - weekly reports authored by WGOC and Field Operations distributed to LR program manager by 12:00pm UTC every Monday; LR program manager distributes consolidated report to client. - Post-recovery - authored by LR recovery resource; delivered to WGOC; mission case appended to; distributed to Field Operations (expanded distribution to effect quality improvement initiatives). Communications: Use of Project Team roster indicated in opening section. - In case of an emergency refer to the emergency escalation diagram on the last page. WGMS offices (Wave Glider Operations Center): LR HQ in Sunnyvale will pilot Wave Glider and update Mission Plan as necessary. Sr. LR Field Tech may pilot Wave Glider on location. Sortie 1: [narrative]. Objectives/Goals: - [narrative]. - [narrative]. Post-Recovery: Perform SL1 inspection and servicing requirements, drive resulting continuous improvement initiatives, and stage for next mission. |
| COURSE ARCHETECTURE |
| Overall Operating Area: [insert] |  |
| Close-Ups - Task Areas or Other Points of Interest: [insert] (Prospect Area with Surface Expressions and Hold-Station Positions) [insert] (Wave Glider Courses) [insert] |  |
| OPERATIONAL AREA DESCRIPTION |
| Loiter Areas: Operating areas for deployment, vehicle performance and sensor correlative data observations, and recovery operations. Transit Lanes: Swim lanes between operating areas circumventing hazards to navigation. - Shipping safety fairways intersecting transit lanes traversed under heightened observation. - Transit lanes established at >1nm distance from known hazards to navigation where possible. Operation Polygons: Defined operational areas structured within a Wave Glider to notify pilots of “out-of-bounds” operating conditions. - Inclusion boundaries alert pilots of unplanned operating area departures. Containment boundaries typically established during near shore or known hazard operations. - Exclusion boundaries alert pilots of unplanned encroachment of undesired operating areas. - Buoy station boundaries established at published watch circles (minimum safe operating distance). Unspecified watch circles established at radius equivalent to buoy depth. - Other surface expression boundaries (i.e., platforms) established at 1nm radius. - Wave Glider operations in less than 10 fathoms of water are avoided. - Exclusion boundaries established when course intersects Shipping Safety Fairways to alert pilots for heightened awareness. |  |

| ENVIRONMENTAL |  |  |  |
| --- | --- | --- | --- |
| Bathymetry: [narrative]. Wind/Waves: [narrative]. Currents/Eddies: [narrative]. Tides: [narrative]. Solar Availability: [narrative]. |  |  |  |
| POTENTIAL ISSUES / CONCERNS |  |  |  |
| Known Hazards to Navigation: [narrative]. Vessel Traffic: Vessel traffic alerts derived through Wave Glider equipped AIS receiver enables evasive piloting maneuvers. Encounters with non-AIS transmitting vessels aided by Wave Glider equipped active radar reflection. [narrative]. Environmental: As cited above. |  |  |  |
| PILOTING TASKS |  |  |  |
| Things To Look For: | Probable Response: |  | When to Escalate: |
| - Event Notifications | - Navigate (evasion, drift management); observe power management criteria; take corrective action; report accordingly |  | - Unsafe operation conditions - Course change required |
| - Power Management | - Camera: NA - Light: Launch/recovery/emergency use only - Telemetry: 5min standard, 10min (250wh), 15min (200wh - auto adjust controlled), 20min (150wh), 30min (100wh) - Water Speed Sensor: Off at 250wh - Payloads: Adj sample rate, or off at <200wh - Weather Station: Off at 150wh - AIS and radar Target Enhancer: Off at 100wh - At 100wh consider positioning glider in area of greatest sea-room in the event of power loss/drift state. * Restore power in reverse order +25wh (Increase thresholds by 100Wh when entering periods of deteriorating weather forecasts) |  | - Notify Program Manager if total available power reaches 300Wh - If course change required |
| RISK ASSESSMENT |  |  |  |
| Refer to Wave Glider Risk Assessment addendum. |  |  |  |
| EMERGENCY EQUIPMENT RETREIVAL |  |  |  |
| If escalation warrants emergency retrieval of a Wave Glider, Marine Operations is engaged to coordinate search and recovery operations. |  |  |  |
| RESOURCES |  |  |  |
| For access to mission monitoring and piloting resources, please contact the Liquid Robotics Wave Glider Operations Center at +1 408. 636.4205, or email opscenter@liquidr.com. |  |  |  |
| AUTHORIZATIONS |  |  |  |
| Sales Engineer: Program Manager: VP Sales: |  | WGOC Director: VP Operations: Chief Operations Officer: |  |

"CONFIDENTIALITY NOTICE: This report and any attachments contain information from Liquid Robotics, Inc., and are intended solely for the use of the named recipient or recipient's employer and affiliates (collectively the "Recipient"). The contents of this report are intended to be covered by any non-disclosure agreement(s) signed between LRI and the Recipient as "Confidential/Proprietary Information" and any dissemination of this report to anyone other than the Recipient is strictly prohibited. If you are not the intended Recipient, you are prohibited from any further viewing of this report or any attachments or from making any use of the report or attachments. If you believe you have received this report or access to this report in error, please notify Liquid Robotics, Inc. immediately at 650-493-6300 or legal@liquidr.com."

APPENDIX

Related Workflow: Build to Operations

Mission Planning Logistics

Day to Day Interfaces LR [client] Onshore Offshore John Appelgren Wave Glider Operations Center [name] Operations Support Manager [name] VP Operations Manager [name] [title] Daily ops & routine reporting etc. [vessel] Party Manager Chief Navigator Navigation Department Mission Support Specialists Wave Glider Operations Center [name] [title] Emergency Escalation On shore Client Management John Appelgren Wave Glider Operations Center Mission Support Specialists Wave Glider Operations Center [vessel] Navigation Department [name] Operations Support Manager Emergency [name] [client - title] [name] LR - VP Operations Manager [name] [client - title]