# Regulus Expansion Port REST Interface (XPREST)

## Contents

Contents	1

Introduction	1

Technologies	2

IP Addresses	2

REST Using CURL	2

REST Using libcurl	4

HTTP GET	4

HTTP POST	6

About JSON	9

Functions	9

Install JAR (jar)	9

Payload Registration (meta)	9

Payload Registration Complete (metadone)	11

Delay Shutdown (offdelay)	12

Poll the VMC (poll)	12

Set AMPS Power Switch (pwr)	13

Reply to Command (reply)	13

Run Script (script)	14

Send File to Shore as Bulk Data (upload)	14

Send File to Shore using WGMS File Transfer Protocol (uploadlegacy)	14

Send Packet to Shore (packet)	14

Send Raw Packet to Shore (rawpacket)	15

# Introduction

The SV3 Vehicle Management Computer (VMC) implements a new network-based interface for payload computers that connect via Expansion Ports; I’ll abbreviate this as XPREST for Expansion Port REST interface.  XPREST is implemented using HTTP GET and POST methods and is compatible with the curl utility and libraries, which are available on many platforms.  The client always initiates communications, which is easier to implement and has a lower overhead for payload developers (vs. implementing a client side server to receive asynchronous notifications; future Regulus releases may allow for additional communication scenarios.)

The interface implements most of the features of the serial-based Payload Protocol, including:

- Receiving and responding to command invocations

- Sending telemetry back to shore-side systems

- Sending files back to shore-side systems

- Invoking commands on the VMC

- Receiving notification when the vehicle is about to be shut down

It also provides some new features:

- Registration of individual components on payload computers

- Command descriptions forwarded back to shore-side systems as part of the vehicle manifest

- Forwarding GPS from the VMC

- Parallel access to the VMC from individual components on payload computers (multiple operations can proceed concurrently and don’t have to come through a single client side driver)

The following features have not been implemented yet:

- Implementation of communication channels

- Forwarding core sensor feeds (AIS, Weather, Water Speed), core sensor telemetry or telemetry from other payloads

# Technologies

## IP Addresses

Regulus assigns addresses according to the expansion port:

| Expansion Port | Connector | IP Address |
| --- | --- | --- |
| xp1 | G1 | 192.168.103.X |
| xp2 | G2 | 192.168.105.X |
| xp3 | G3 | 192.168.104.X |
| xpSub | (Sub) | 192.168.102.X |

In each case, the DHCP server is configured to begin assigning addresses at 100 and the VMC is configured at address 1.  For repeatability, it may be desirable to configure the payload not to use DHCP with a manually assigned address less than 100.

## REST Using CURL

REST (Representational State Transfer) interfaces use HTTP methods like GET to retrieve data from the server and POST to create resources on the server.  Note that REST does not necessarily implement Web Services, which implement a WSDL (Web Services Description Language) to advertise methods and standardize on XML to describe parameters.

Parameters and small amounts of data can be passed directly in the URL.  For example:

http://192.168.103.1/xp?op=poll&id=2000&gps=true

In this case, “192.168.103.1” is the address of the server.  The service is identified by the text after the slash and before the question mark (“xp”)  The parameters are “op”, “id” and “gps”

Note that  URL’s can only contain ASCII characters, and some values are reserved.  Alpha-numeric characters (both upper-case and lower-case) as well as dash (“-“), dot (“.”), underscore (“_”) and tilde (“~”) are specifically not reserved.  Any byte value can be encoded using percent notation.  For example, an exclamation point (“!”) has an ASCII value of 33, or 21 as a hexadecimal digit.  It would be encoded as “%21”  Any data, ASCII or binary, can be encoded in this manner.

Encoding must be used when there are reserved or binary values in parameter names or values or in the service name.  For example, if the service name were “xp!” instead of “xp”, the URL would be:

http://192.168.103.1/xp%21?op=poll&id=2000&gps=true

The HTTP GET method supplies a URL to the server, so it can carry some name/value pairs; but it is primarily used to return data from the server to the client.  Here’s a curl invocation to return data using the sample URL:

$ curl --silent 'http://192.168.105.1/xp?op=poll&id=2000&gps=true'

{"gps":{"latitude":37.410943,"longitude":-122.004451,"altitude":"NaN","timeFix":1398964173,"timeCurrent":1398964173,"seedToGoal":"NaN","sog":"NaN","fog":"NaN"}}

The data returned from the server, a JSON fragment containing the next GPS fix received, is returned on standard output.

The HTTP POST method also supplies a URL, but it can carry attachments along it.  For example, a JSON fragment is sent to the server when a payload device registers.  Here’s an example from the PayloadInterface program on the SMC:

{"boardAddress":0,

"corbaName":"PayloadInterface",

"description":"Payload interface",

"hostID":"1022102 SOMDM3730-20-2780AGCR-B 0213A00074",

"interface":[{"swver":"2.1.0"}],

"reportingAddress":8192,

"tag":"PayloadInterface",

"ui":[{"label":"Get Modules",

"method":"GetModules",

"paramLabels":["Command"],

"paramTypes":["java.lang.String"],

"template":"%tgetmodules",

"timeout":600,

"tooltip":"Report the configured modules running on the SMC"},

{"label":"Run Shell Command",

"method":"RunShell",

"paramLabels":["Command"],

"paramTypes":["java.lang.String"],

"template":"%trunshell %p",

"timeout":600,

"tooltip":"Run shell command and return result as a console ack"},

{"label":"Run Shell (Return File)",

"method":"RunShellReturnFile",

"paramLabels":["Command"],

"paramTypes":["java.lang.String"],

"template":"file:%trunshell %p",

"timeout":600,

"tooltip":"Run shell command and return result as a file transfer"}]}

Here’s how that fragment, passed in a file called “meta.txt”, would be sent using curl:

$ curl -X POST -F 'file=@'"meta.txt" ‘http://192.168.103.1/xp?op=meta&id=2000’

OK

It is beneficial to bound operations with well-known timeouts.  curl accepts a command line option (“--max-time”) to limit how long it will wait for a response.  For example, curl will wait for no more than 60 seconds to complete the following operation:

$ curl --max-time=60 -X POST -F ‘file=@meta.txt’ ‘http://192.168.103.1/xp?op=meta&id=2000’

OK

For more information on URL (URI) formatting, see RFC 3986 (http://www.ietf.org/rfc/rfc3986.txt); for more information on curl, see the Linux man page (manual page; http://linux.about.com/od/commands/l/blcmdl1_curl.htm) or documentation accompanying the version of curl that you are using.

## REST Using libcurl

libcurl is an open source library that implements many transfer protocols, including HTTP GET and POST, and is available on Linux and Windows from the curl web site (http://curl.haxx.se)  The SMC interface library (SV3RemotingLib) is based on libcurl and jsoncpp and is also available as a working sample.

### HTTP GET

Here’s some sample code to show how to implement an HTTP GET operation in C++ where the URL is passed as the first argument to the program:

#include <iostream>

#include <sstream>

#include <stdexcept>

#include <string>

#include <typeinfo>

#include <vector>

#include <curl/curl.h>

using namespace std;

// ThrowEasy simplifies checking return codes and throws an exception in the case of failure

inline void ThrowEasy(CURLcode code, const std::string& func)

{

if (code != 0) {

stringstream ss;

ss << func << " returned " << code << " (" << curl_easy_strerror(code) << ")";

throw runtime_error(ss.str());

}

}

// SetOpt simplifies setting various options on the CURL instance

// and checking for errors

template<typename T>

void SetOpt(CURL* curl, CURLoption opt, T t)

{ ThrowEasy(curl_easy_setopt(curl, opt, t), "curl_easy_setopt()"); }

// This function overrides the default behavior and saves the body

// of the response to a string, which can be analyzed after the

// curl invocation completes.

typedef std::vector<std::string> StringArray;

size_t WriteDataFunc(char* ptr, size_t size, size_t nmemb, void* userdata) {

string& body = *((string*) userdata);

const size_t newBytes = size*nmemb;

size_t result;

try {

body.insert(body.end(), ptr, ptr+newBytes);

result = newBytes;

} catch(...) {

result = 0;

}

return result;

}

// This function overrides the default behavior and saves header

// lines to an array of strings, which can be analyzed later after

// the curl invocation completes

size_t WriteHeaderFunc(void *ptr, size_t size, size_t nmemb, void *userdata) {

StringArray& headers = *((StringArray*) userdata);

size_t result;

try {

const char* p2 = (const char*) ptr;

const size_t newBytes = size*nmemb;

const string header(p2, p2+newBytes);

headers.push_back(header);

result = newBytes;

} catch(...) {

result = 0;

}

return result;

}

int main(int argc, char* argv[])

{

int result = 0;

if (argc == 2) {

CURL* curl = NULL;

try {

StringArray headers;

std::string body;

if ((curl = curl_easy_init()) == NULL)

throw runtime_error(“curl_easy_init() failed”);

const long connectTimeout = 10;

const long responseTimeout = 60;

const long totalTimeout = 61;

const bool verbose = false;

SetOpt(curl, CURLOPT_URL, argv[1]);

SetOpt(curl, CURLOPT_NOPROGRESS, 1L);

SetOpt(curl, CURLOPT_NOSIGNAL, 1L);

SetOpt(curl, CURLOPT_CONNECTTIMEOUT, connectTimeout);

SetOpt(curl, CURLOPT_TIMEOUT, totalTimeout);

SetOpt(curl, CURLOPT_FTP_RESPONSE_TIMEOUT, responseTimeout);

SetOpt(curl, CURLOPT_VERBOSE, verbose ? 1L : 0L);

SetOpt(curl, CURLOPT_WRITEFUNCTION, &WriteDataFunc);

SetOpt(curl, CURLOPT_HEADERFUNCTION, &WriteHeaderFunc);

SetOpt(curl, CURLOPT_WRITEHEADER, &headers);

SetOpt(curl, CURLOPT_WRITEDATA, &body);

ThrowEasy(curl_easy_perform(curl), "curl_easy_perform");

cout << "Headers=" << endl;

for (StringArray::const_iterator ci = headers.begin(); ci != headers.end(); ++ci) cout << *ci;

cout << "Body=" << endl << body << endl;

} catch(exception& ex) {

cerr << endl

<< "Caught " << typeid(ex).name() << ": " << ex.what() << endl;

result = 1;

}

if (curl != NULL) curl_easy_cleanup(curl);

}

return result;

}

Note that the headers from the invocation are saved in the headers string array.  The headers should be checked to ensure that an OK status was returned.

### HTTP POST

Regulus accepts posts that use the multi-part form method available through curl.  In addition, posts require and “Expect:” header and can optionally specify the MIME type for the data passed.

#include <iostream>

#include <sstream>

#include <stdexcept>

#include <string>

#include <typeinfo>

#include <vector>

#include <curl/curl.h>

using namespace std;

// ThrowEasy simplifies checking return codes and throws an exception in the case of failure

inline void ThrowEasy(CURLcode code, const std::string& func)

{

if (code != 0) 	{

stringstream ss;

ss << func << " returned " << code << " (" << curl_easy_strerror(code) << ")";

throw runtime_error(ss.str());

}

}

// ThrowFormAdd performs the same function for form add return codes

inline void ThrowFormAddError(CURLFORMcode res)

{

if (res != 0)

{

stringstream ss;

ss << "curl_formadd() returned " << res;

throw runtime_error(ss.str());

}

}

// SetOpt simplifies setting various options on the CURL instance

// and checking for errors

template<typename T>

void SetOpt(CURL* curl, CURLoption opt, T t)

{ ThrowEasy(curl_easy_setopt(curl, opt, t), "curl_easy_setopt()"); }

// This function overrides the default behavior and saves the body

// of the response to a string, which can be analyzed after the

// curl invocation completes.

typedef std::vector<std::string> StringArray;

size_t WriteDataFunc(char* ptr, size_t size, size_t nmemb, void* userdata) {

string& body = *((string*) userdata);

const size_t newBytes = size*nmemb;

size_t result;

try {

body.insert(body.end(), ptr, ptr+newBytes);

result = newBytes;

} catch(...) {

result = 0;

}

return result;

}

// This function overrides the default behavior and saves header

// lines to an array of strings, which can be analyzed later after

// the curl invocation completes

size_t WriteHeaderFunc(void *ptr, size_t size, size_t nmemb, void *userdata) {

StringArray& headers = *((StringArray*) userdata);

size_t result;

try {

const char* p2 = (const char*) ptr;

const size_t newBytes = size*nmemb;

const string header(p2, p2+newBytes);

headers.push_back(header);

result = newBytes;

} catch(...) {

result = 0;

}

return result;

}

int main(int argc, char* argv[])

{

int result = 0;

if (argc == 3) {

CURL* curl = NULL;

curl_httppost* formpost = NULL;

curl_httppost* lastptr = NULL;

curl_slist* headerlist = NULL;

try {

StringArray headers;

std::string body;

if ((curl = curl_easy_init()) == NULL) throw runtime_error("curl_easy_init() failed");

headerlist = curl_slist_append(headerlist, "Expect:");

const long connectTimeout = 10;

const long responseTimeout = 60;

const long totalTimeout = 61;

const bool verbose = false;

SetOpt(curl, CURLOPT_URL, argv[1]);

SetOpt(curl, CURLOPT_NOPROGRESS, 1L);

SetOpt(curl, CURLOPT_NOSIGNAL, 1L);

SetOpt(curl, CURLOPT_CONNECTTIMEOUT, connectTimeout);

SetOpt(curl, CURLOPT_TIMEOUT, totalTimeout);

SetOpt(curl, CURLOPT_FTP_RESPONSE_TIMEOUT, responseTimeout);

SetOpt(curl, CURLOPT_VERBOSE, verbose ? 1L : 0L);

////////////////////////////////////////

/// Adding expect header, optionally ///

/// add MIME data type header        ///

////////////////////////////////////////

SetOpt(curl, CURLOPT_HTTPHEADER, headerlist);

////////////////////////////////////////

//////////////////////////////////////

/// Required for multi-part forms ////

//////////////////////////////////////

ThrowFormAddError(

curl_formadd(&formpost, &lastptr,

CURLFORM_COPYNAME, "file",

CURLFORM_FILE, argv[2],

CURLFORM_END));

ThrowFormAddError(

curl_formadd(&formpost, &lastptr,

CURLFORM_COPYNAME, "filename",

CURLFORM_COPYCONTENTS, argv[2],

CURLFORM_END));

ThrowFormAddError(

curl_formadd(&formpost,&lastptr,

CURLFORM_COPYNAME, "submit",

CURLFORM_COPYCONTENTS, "send",

CURLFORM_END));

SetOpt(curl, CURLOPT_HTTPPOST, formpost);

//////////////////////////////////////

//////////////////////////////////////

/// Required to save response body ///

/// and headers to local variables ///

//////////////////////////////////////

SetOpt(curl, CURLOPT_WRITEFUNCTION, &WriteDataFunc);

SetOpt(curl, CURLOPT_HEADERFUNCTION, &WriteHeaderFunc);

SetOpt(curl, CURLOPT_WRITEHEADER, &headers);

SetOpt(curl, CURLOPT_WRITEDATA, &body);

//////////////////////////////////////

ThrowEasy(curl_easy_perform(curl), "curl_easy_perform");

cout << "Headers=" << endl;

for (StringArray::const_iterator ci = headers.begin(); ci != headers.end(); ++ci) cout << *ci;

cout << "Body=" << endl << body << endl;

} catch(exception& ex) {

cerr << endl

<< "Caught " << typeid(ex).name() << ": " << ex.what() << endl;

result = 1;

}

if (curl != NULL)	curl_easy_cleanup(curl);

if (formpost != NULL)	curl_formfree(formpost);

if (headerlist != NULL)	curl_slist_free_all(headerlist);

}

return result;

}

## About JSON

JSON (JavaScript Object Notation; see http://www.json.org/) is a lightweight data-interchange format, easy for both humans and machines to read and write.  XPREST passes fragments of JSON to perform registrations and (optionally) to return data from the polling command.

Regulus implements a JSON parser and formatters; the SMC uses the open source jsoncpp library, and there are many other implementation.  It’s not that hard to implement if you can’t find an implementation you like.

# Functions

## Install JAR (jar)

This HTTP POST message sends a JAR file (Java archive) to the VMC and installs it, allowing the payload to extend the functionality of the VMC.  The JAR content is posted.

http://192.168.103.1/xp?op=jar&id=polling-id

id is a previously-registered polling ID (using the meta command)

## Payload Registration (meta)

This HTTP POST message registers a polling ID (id) along with a description of the payload and its commands.  The polling ID will be used in all subsequent messages and should be unique for the IP address of the computer.  Once registered, the VMC aggregate the description into its manifest files and report it to the shore-side system.  Here’s the format of the URL:

http://192.168.103.1/xp?op=meta&id=polling-id&hostid=registration-group

The hostid parameter is optional.  Whenever specified, the VMC will look for matching “hostid” settings in registration data for other payloads and cancel/delete those registrations.  The idea is to include the hostid parameter for the first registration message of a group, only; registrations of other payload controlled by the same interface would not include it.

The registration data is posted as a JSON fragment:

{"boardAddress":0,

"corbaName":"PayloadInterface",

"description":"Payload interface",

"hardware":[{"connector":"ICPwr1"}],

"hostID":"1022102 SOMDM3730-20-2780AGCR-B 0213A00074",

"interface":[{"swver":"2.1.0"}],

"reportingAddress":8192,

"tag":"PayloadInterface",

"ui":[{"label":"Get Modules",

"method":"GetModules",

"paramLabels":["Command"],

"paramTypes":["java.lang.String"],

"template":"%tgetmodules",

"timeout":600,

"tooltip":"Report the configured modules running on the SMC"},

{"label":"Run Shell Command",

"method":"RunShell",

"paramLabels":["Command"],

"paramTypes":["java.lang.String"],

"template":"%trunshell %p",

"timeout":600,

"tooltip":"Run shell command and return result as a console ack"},

{"label":"Run Shell (Return File)",

"method":"RunShellReturnFile",

"paramLabels":["Command"],

"paramTypes":["java.lang.String"],

"template":"file:%trunshell %p",

"timeout":600,

"tooltip":"Run shell command and return result as a file transfer"}]}

The JSON fragment contains the following values that have special meaning, but can contain additional fields that are ignored by the VMC:

boardAddress is the 16-bit address to which Write Console Script operations are sent.  For expansion ports, this is always set to zero (0)

hardware contains an array of elements with the following optional values:

- connector names a power switch (“ICPwr1”.. “ICPwr8”, depending on the installed hardware) associated with the payload box; the “pwr” function affects the state of the switches;

- manurfacturer is the name of the manufacturer of the attached device;

- model is the model of the attached device;

- serial is the serial number of a device attached to the payload; ideally, this value is determined by communicating with the device.  Data reported by the payload may be associated with this serial number;

- swver is the software version running on the attached device;

- tty is the serial port associated with the payload.

hostID should uniquely indicate the registration group of which this payload is a part.  There are at least two scenarios for registration groups:

- If the payload implementing its own interface to the VMC and registering on its own, hostID should uniquely identify not only the CPU but also the payload on the CPU.

- If there is a program registering multiple payloads, hostID should uniquely the CPU and the registration program.

interface is an array of elements associated with the interface software, including the following optional values:

- swver is the version string associated with the interface software.

reportingAddress is the 16-bit address that is stamped on telemetry coming from this payload.

tag is the name of the payload that uniquely identifies the payload within the domain of the expansion port.

ui is an array of elements that describes the commands that this payload exposes:

- label is a text string to put on a button on the UI;

- method is the name sent back to the VMC when the command is invoked (the method must not include spaces);

- paramLabels is an array of strings describing each parameter to the command;

- paramTypes is an array of strings that describe the type of each command parameter, for example:

“boolean” is a logical value (true or false);

“int” is a 32-bit signed integer;

“float” is a single-precision IEEE 754 floating point value;

“double” is a double-precision IEEE 754 float point value;

“java.lang.String” is a sequence of characters;

- template indicates how to convert the parameters to a command string when sending it back to the payload.  The string can contain printf-style format sequences that start with a percent sign (“%”); two percent signs (“%%”) will expand to a single percent sign.  The following format sequences are defined:

%m	The method name

%nnffp	Parameters with optional formatters.  If a number (nn) is specified, it indicates the which parameter to expand; otherwise, all parameters are expanded.  Other format character (ff) are as follows:

*	Use spaces to separate parameters (default)

,	Use commas to separate parameters

C	Quote parameters using C language standards (defaults to shell rules)

U	Quote parameters using URL rules (defaults to shell rules)

%fft	The command tag that must be passed back to reply to the command with optional formatters

- tooltip is a string to give more information about the command when the mouse hovers over the button (or whenever it would be appropriate.)

## Payload Registration Complete (metadone)

This HTTP GET message indicates that a registration sequence is complete and it the new resulting manifest should be published to shore-side systems.  Here’s the format of the URL:

http://192.168.103.1/xp?op=metadone&id=polling-id

## Delay Shutdown (offdelay)

This HTTP GET message, typically sent after a shutdown notification from a poll message, delays power off to the expansion port by a given number of seconds.  The new setting can be optionally set as the default delay.

http://192.168.103.1/xp?op=offdelay&id=polling-id&delay=nnn&setdefault=flag

id is a previously-registered polling ID (using the meta command);

delay is the number of seconds to delay before turning power off to the payload;

if the setdefault parameter is given and set to a non-zero value, the delay value will be saved as the default shutdown time.

## Poll the VMC (poll)

Using HTTP methods, all communications start with the client, so the HTTP GET poll message is a way of reading waiting command invocations, GPS and shutdown indications.  The command contains a timeout value, but the command returns as soon as data is available.

http://192.168.103.1/xp?op=poll&id=polling-id&id=polling-id2&...&id=polling-idn&format=nnn&gps=true&shutdown=true&maxcommands=ccc&timeout=ttt

id is a previously-registered polling ID (using the meta command); if multiple polling ID’s are given, queued commands for any of the given payloads are returned;

format indicates how responses are formatted:

1 (default) = JSON format, commands are assumed to for from the same payload, so no ID (tag) is given

$ curl --silent 'http://192.168.105.1/xp?op=poll&id=2000&format=2&shutdown=true'

{"commands":[{"command":"'YXxutd6L44'runshell 'ls -l'"}]}

2 = JSON format, commands can be for multiple payloads, so ID’s (tags) are included

$ curl --silent 'http://192.168.105.1/xp?op=poll&id=2000&id=20F1&format=2&shutdown=true'

{"commands":[{"id":"PayloadInterface","command":"'YXxutd6L44'runshell 'ls -l'"}]}

gps indicates whether the next GPS sample is returned (true); if not specified or set to false, GPS samples are not returned.  This is an example of a GPS sample:

$ curl --silent 'http://192.168.105.1/xp?op=poll&id=2000&id=20F1&format=2&gps=true&shutdown=true'

{"gps":{"latitude": 37.410782,"longitude":-122.004585,"altitude":"NaN","timeFix":1399073419,"timeCurrent":1399073419,"seedToGoal":"NaN","sog":"NaN","fog":"NaN"}}

Note that timeFix is the timestamp from the GPS receiver, while timeCurrent was the timestamp on the VMC when the fix was received.  Both are in seconds since 1-January 1970 UTC.

maxcommands allows the caller to specify how many commands the VMC can return; by default, this value is set to 1.  Setting maxcommands to zero disables returning any commands.

shutdown indicates whether client wants to be notified if power to the payload is about to be cut (true); if not specified or set to false, no notification is given.  This is an example of being notified that shutdown is about to occur:

$ curl --silent 'http://192.168.105.1/xp?op=poll&id=2000&id=20F1&format=2&gps=true&shutdown=true'

{"gps":{"latitude": 37.410782,"longitude":-122.004585,"altitude":"NaN","timeFix":1399073419,"timeCurrent":1399073419,"seedToGoal":"NaN","sog":"NaN","fog":"NaN"}}

Note that if shutdown notification is received, the client can send a message to delay shutdown long enough for the client computer to close files, etc. (offdelay)

timeout indicates the number of milliseconds to wait for one of the return conditions to be met.  If no conditions are met within the given time, a timeout message is returned:

$ curl --silent 'http://192.168.105.1/xp?op=poll&id=2000&id=20F1&format=2&shutdown=true&timeout=15000'

{"timeout":true}

Note that a timeout is not necessarily an error.  Typically, a client would specify a relatively short timeout (like 15000 milliseconds) and continually issue the command.  This represents a very light load on the VMC.

If any error conditions occur during execution of the command, an error message will be returned:

$ curl -- silent 'http://192.168.105.1/xp?op=poll&id=2000&format=2&maxcommands=0&timeout=1000'

{"error":"Command did not request GPS or a command"}

## Set AMPS Power Switch (pwr)

This HTTP GET message sets the state of the power switch(es) registered with the meta message or any named power switch.

http://192.168.103.1/xp?op=pwr&id=polling-id&cmd=on-or-off&sw=switch-name

id is a previously-registered polling ID (using the meta command);

cmd is the new state of the switch: on to turn power on, off to turn power off;

sw is an optional parameter indicating which switch to affect ("ICPwr1".."ICPwr8", depending on how many switches exist on the installed hardware); if not specified, the switch(es) specified by the meta command are affected.

## Reply to Command (reply)

This HTTP POST message sends reply data for a command previously received using poll.  The body of the reply is posted.

http://192.168.103.1/xp?op=reply&id=polling-id&tag=ttt&complete=flag&success=flag

id is a previously-registered polling ID (using the meta command);

tag is the command tag forwarded along with the command (the text returned by the “%t” format sequence in the template for the command);

complete is an optional flag set to true (default) if the command is now complete, or false if there is more data. [This feature is not implemented yet.]

success is an optional flag set to true (default) if the command succeeded, or false if the command failed. [This feature is not implemented yet.]

## Run Script (script)

This HTTP GET message allows the payload to run any advertised method on the VMC.

http://192.168.103.1/xp?op=script&id=polling-id&cmd=command-text

id is a previously-registered polling ID (using the meta command);

cmd is the text of the script invocation; for example:

$ curl --silent ' http://192.168.105.1/xp?op=script&id=2000&cmd=uptime.uptime%28%29'

Uptime: 0 days, 07:26:46

## Send File to Shore as Bulk Data (upload)

This HTTP POST message sends a file to shore using the bulk data service, which is typically restricted to high speed communication channels.  The file to send is posted.

http://192.168.103.1/xp?op=upload&id=polling-id

id is a previously-registered polling ID (using the meta command)

## Send File to Shore using WGMS File Transfer Protocol (uploadlegacy)

This HTTP POST message sends a file to shore using the WGMS file transfer protocol over any WGMS communication channel.  The file to send is posted.

http://192.168.103.1/xp?op=uploadlegacy&id=polling-id

id is a previously-registered polling ID (using the meta command)

## Send Packet to Shore (packet)

This HTTP POST message sends a level 2 message to shore over the default communications channel, but accepts parameters for some of the fields and performs the final message assembly.  The level 1 message content is posted.

http://192.168.103.1/xp?op=packet &id=polling-id&type=L2-type&wid=L2-packet-ID

id is a previously-registered polling ID (using the meta command);

type is the Level 2 type ID (byte 1 of the packet);

wid is the 32-bit Level 2 packet ID (bytes 2-5 of the packet)

## Send Raw Packet to Shore (rawpacket)

This HTTP POST message sends a raw level 2 message to shore over the default communications channel.  The formatted level 2 message is posted.

http://192.168.103.1/xp?op=rawpacket&id=polling-id

id is a previously-registered polling ID (using the meta command)