Setting up MMSI Ignore for Regulus

This is a separate step from the MMSI ignore list XML note in WGMS. That only stops shore-side generated alarms, such as the path and proximity alarms, and has no influence on the actual behavior of the vehicle.

The below set of commands are for the vehicle itself, and will stop the glider auto-avoiding a defined MMSI, which will in turn stop the “avoiding mmsixxxxxxxxx” alarms. This is useful in a situation where there is a known support vessel working in close proximity of the vehicle, and the vessel knows to avoid the wave glider.

There are four commands (write console scripts) that control auto-avoidance behavior:

- %ship.setTreatmentIgnore(mmsi) - Causes the ship with the indicated MMSI code to be ignored by the navigator. It will not be avoided.

- %ship.setTreatmentNormal(mmsi) - Causes the ship with the indicated MMSI code to be treated like all other ships, using the default standoff radius.

- %ship.setTreatmentCustom(mmsi,standoff,fpf) - Causes the ship with the indicated MMSI code to use the indicated standoff distance (in meters) and forward projection factor.

- %ship.setTreatmentCircle(mmsi,radius,trigger) – This is a circling command, more information can be found here.

To get the glider to ignore a specific MMSI, simply send the first write console script with the MMSI you want the glider to ignore (War Machine’s AIS transmitter is used in this example): %ship.setTreatmentIgnore(338060163)

This should return an ‘Acked’ if done correctly. The only way to check this is through the waypoint page on the vehicle’s webpage. When the command has been Acked you should see this in the vehicle webpage (note the ‘ignore’):