| Function | WGMS Tab | Pwr Domain | Module # | Port # |
| --- | --- | --- | --- | --- |
| Weather Station | AMPS Output | 1 | 4 | 1 |
| Sensor Port 3 (S3) | AMPS Output | 1 | 4 | 2 |
| Light Bar | AMPS Output | 1 | 4 | 3 |
| Water Speed Sensor | AMPS Output | 1 | 4 | 5 |
| Cell Modem | AMPS Output | 1 | 4 | 6 |
| Radar Reflector | AMPS Output | 1 | 5 | 1 |
| Float Recovery Device | AMPS Output | 1 | 5 | 2 |
| Ethernet Switch | AMPS Output | 1 | 5 | 3 |
| MAS Power (Surfer config only) | AMPS Output | 1 | 5 | 6 |
| 48V Surfer Streamer (Surfer config only) | AMPS Output | 1 | 5 | 7 |
| High Voltage Port (HV) | AMPS Output | 1 | 5 | 8 |
| Sensor Port 2 (S2) | AMPS Output | 1 | 7 | 1 |
| Sensor Port 1 (S1) | AMPS Output | 1 | 7 | 2 |
| AIS | AMPS Output | 1 | 7 | 4 |
| Iridium Modem | AMPS Output | 1 | 32 | 1 |
| Ublox GPS Module | AMPS Output | 1 | 32 | 2 |
| Vehicle Management Computer | AMPS Output | 1 | 32 | 3 |
| Thruster | AMPS Output | 2 | 1 | 1 |
| Expansion Port 1 (G1), [Vcons, Vprod] | AMPS Bridge | 1 | 11 | 1,2 |
| Expansion Port 2 (G2), [Vcons, Vprod] | AMPS Bridge | 1 | 12 | 1,2 |
| Expansion Port 3 (G3), [Vcons, Vprod] | AMPS Bridge | 1 | 8 | 1,2 |
| Umbilical Expansion Port (U1), Vcons | AMPS Bridge | 1 | 15 | 1 |
| Debug Expansion Port (D1) [Vcons, Vprod] | AMPS Bridge | 1 | 14 | 1,2 |
| Sub Expansion Port, Vcons | AMPS Bridge | 2 | 0 | 5 |
| Fore Solar Panel | AMPS Solar Input | 1 | 9 | 1 |
| Mid Solar Panel | AMPS Solar Input | 1 | 10 | 1 |
| Aft Solar Panel | AMPS Solar Input | 1 | 10 | 2 |

distance

maneuver