| Customer: | MBARI |
| --- | --- |
| Region: | Central California Coast |
| Requestor: | Thom Maughan Jamie Griffith |
| Pre-Mission Checklist No.: | 140122 |
| Sales Order No.: |  |
| Program Phase: | Pre-Proposal Proposal/Quote Mission Planning |

Mission Parameters

| Mission Start/End Date: | 2014-02-05 to 2014-02-20 |
| --- | --- |
| Mission Duration: | 20 days |
| Op Area: | 35.1185, -122.9752 |

Wave Glider Configuration

| Model |  |
| --- | --- |
| SV2 | SV3 |
| Standard Options |  |
| METOC (600hz) METOC+ (300 or 600hz) Hydrocarbon Gateway Custom Configuration [explain below] | 5.8 Meter Umbilical 4.0 Meter Umbilical Tow Body Additional devices [explain below] |
| Custom Configuration |  |
| - Using Wave Glider ‘Tex’ to communicate with a Benthic Rover |  |

Risk Assessment Summary

The assessment is:

- Bathymetry = Green Light

- Navigational Authority = Green Light

- Power Margin

  - February = Yellow Light

- Vessel Traffic = Green Light

- Hazards to Navigation =  Green Light

- Political Interests = Green Light

Bathymetry

The op area is waters with depths well over 1000 fathoms, bathymetry will not be an issue for  this mission.

Navigational Authority

Navigational authority in this area should not be a problem for the duration of this mission.  The vehicle is expected to be able to hold station for approximately 90% of the time or greater.  Tidal currents should not have any affect on the vehicle this far offshore.  The southward flowing California current dominates this area.  Significant wave heights in this area will average about 3.1 meters with periods between seven to ten seconds.  Average surface currents for this area are expected to be less than 0.5 knots.

Power Margin

Power is the highest environmental risk factor for the proposed mission.  At the latitudes of interest, there is limited solar energy available.  For a February deployment, battery life is expected to last about two weeks with a 7W payload.  If the payload draw is increased to 10W continuous, battery life may be depleted in as little as one week.  Core payload requirements for an SV2 will draw about 5W.  Due to the unique nature of this payload, continuous operation may not be necessary for mission success.

Vessel Traffic

Minimal vessel traffic can be expected, however commercial shipping traffic travelling south to Long Beach is likely.

Hazards to Navigation

Although unlikely this far out, kelp patties are a possible encounter and may encumber the vehicle.

Political Interests

Operations will be within US waters, as such proper permits and notices with the US Coast Guard are required.