| Wave Glider – SV3 Pre-Launch Checklist |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Date / Time |  |  |  |  |  |  | Project: |  |  |  |  |  |
| Technician(s): |  |  |  |  |  |  | Ship / Op Area: |  |  |  |  |  |
| ##### Cycle Power on CCU Prior to Pre-Launch Checks. Ensure Auto Avoidance settings = 0, and no check in WSS:assume in water |  |  |  |  |  |  |  |  |  |  |  |  |
| ##### Vehicle / WGMS Configurations |  |  |  |  |  |  |  |  |  |  |  |  |
| Item/Procedure |  |  | # Readings |  |  |  |  | # Specifications |  |  |  | Initials |
| SV3 WGMS Name |  |  | Tiny |  |  |  |  |  |  |  |  |  |
| Float Name |  |  | Tiny Float |  |  |  |  |  |  |  |  |  |
| Glider Name |  |  | Tiny Glider |  |  |  |  |  |  |  |  |  |
| Thrudder Serial Number |  |  | N/A |  |  |  |  | Hardware ID |  |  |  |  |
| Onboard Server WiFi IP |  |  | 192-168-100-1 |  |  |  |  | URL |  |  |  |  |
| Onboard Server Virtual Cell IP |  |  |  |  |  |  |  | URL10.20.15.XXX |  |  |  |  |
| Tracking |  |  |  |  |  |  |  |  |  |  |  |  |
| Resilient Tracker IMEI |  |  |  |  |  |  |  | Every 10 min (1st hour), then every 12 hrs. |  |  |  |  |
| (O2) |  |  | (S4) |  |  |  |  | (O1) (S4) |  |  |  |  |
| Payloads | RTE ☒ |  |  | AIS ☒ |  | Weather Station ☒ |  |  | Water Speed ☒ |  | Lightbar ☒ |  |
| XP 1 (G1) SV3 PL or MPU |  |  | NA see comments section |  |  |  |  | Description |  |  |  |  |
| XP 2 (G2) |  |  | NA |  |  |  |  | Description |  |  |  |  |
| XP 3 (G3) |  |  | NA |  |  |  |  | Description |  |  |  |  |
| Payload 1 (sensor 1) |  |  | NA |  |  |  |  | Description |  |  |  |  |
| Payload 2 (S2) |  |  | NA |  |  |  |  | Description |  |  |  |  |
| Payload 3 (S3) |  |  | NA |  |  |  |  | Description |  |  |  |  |
| WGMS |  |  |  |  |  |  |  |  |  |  |  |  |
| Organization |  |  |  |  |  |  |  |  |  |  |  |  |
| Escalation List |  |  |  |  |  |  |  | Created and Populated |  |  |  |  |
| Event Definitions |  |  |  |  |  |  |  | Compare Against Standard List |  |  |  |  |
| Vehicle Op Area Defined |  |  |  |  |  |  |  | Poly Name(s). Alarms Enabled |  |  |  |  |
| ##### Communications |  |  |  |  |  |  |  |  |  |  |  |  |
| WiFi Connection |  |  |  |  |  |  |  | Onboard Server Accessible |  |  |  |  |
| Virtual Relay Cell |  |  |  |  |  |  |  | WGMS Raw Outputs – VR Cell |  |  |  |  |
| Iridium |  |  |  |  |  |  |  | WGMS Raw Outputs - IridiumStatus |  |  |  |  |
| Vehicle Telemetry Received |  |  |  |  |  |  |  | Packet Data Displayed in WGMS |  |  |  |  |
| Resilient Tracker |  |  |  |  |  |  |  | WGMS Raw Outputs |  |  |  |  |
| ##### Vehicle Status |  |  |  |  |  |  |  |  |  |  |  |  |
| Battery [Wh] > 500 Wh |  |  |  |  |  |  |  |  |  |  |  |  |
| Solar Power Indicators: |  | Panel 1 |  |  | Panel 2 |  |  | Panel 3 |  | Panel 4 |  |  |
| Output > 0 W |  |  |  |  | 0 |  |  |  |  |  |  |  |
| Drybox Environmentals: |  | CCU |  |  | XP 1 |  |  | XP 2 |  | XP 3 |  |  |
| Pressure > 120 kpa (+/-2) |  |  |  |  |  |  |  |  |  |  |  |  |
| Temp |  |  |  |  |  |  |  |  |  |  |  |  |
| Drybox Environmentals: |  | Thrudder (170 kpa) |  |  | Payload 1 |  |  | Payload 2 |  | Payload 3 |  |  |
| Pressure > 120 kpa (+/-2) |  |  |  |  |  |  |  |  |  |  |  |  |
| Temp |  |  |  |  |  |  |  |  |  |  |  |  |

| ##### Vehicle Status: WGMS, SV3 Commands, SV3 vehicle form (AMPS, WX , AIS) Adaptable modular power system |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Item/Procedure |  | Readings |  |  |  |  | Specifications |  |  | Initials |
| Linux Uptime |  |  |  |  |  |  | Time Since Last Boot |  |  |  |
| Regulus Uptime |  |  |  |  |  |  | Time Since Last Boot |  |  |  |
| Weather Station |  |  |  |  |  |  | Accurate Reporting |  |  |  |
| Airmar Weather GPS |  |  |  |  |  |  | Latest Report |  |  |  |
| Torpedo GPS |  | Vehicle status page |  |  |  |  | Latest Report |  |  |  |
| UBLOX GPS |  | Vehicle status page |  |  |  |  | Latest Report |  |  |  |
| Weather Station Heading |  |  |  |  |  |  |  |  |  |  |
| Thrudder Heading |  |  |  |  |  |  |  |  |  |  |
| Water Speed |  |  |  |  |  |  | Current Draw |  |  |  |
| ##### Mechanical Checks |  |  |  |  |  |  |  |  |  |  |
| Float | Wet-mates connected/lubed |  |  |  |  |  |  |  |  |  |
|  | Connector covers secure |  |  |  |  |  |  |  |  |  |
|  | Drybox secure |  |  |  |  |  |  |  |  |  |
|  | Contact info secure |  |  |  |  |  |  |  |  |  |
|  | Hull |  |  |  | Free of cracks and dings |  |  |  |  |  |
|  | Skeg/Fin |  |  |  | Secure |  |  |  |  |  |
| Sub | Hardware secure |  |  |  |  |  |  |  |  |  |
|  | Umbilical visual inspection |  |  |  | Free of voids/defects |  |  |  |  |  |
|  | Roll pins inserted |  |  |  | Flushed with surface |  |  |  |  |  |
|  | Wings |  |  |  | Hardware secure with Loctite |  |  |  |  |  |
|  | Thrudder |  |  |  | Hardware secure. No protrusion of fin and bottom back fairing fasteners. |  |  |  |  |  |
| ##### Final Checks |  |  |  |  |  |  |  |  |  |  |
| Thruster Spin |  |  |  |  |  | Slow, Fast |  |  |  |  |
| Thrudder Range of Motion |  |  |  |  |  | Manual and Command |  |  |  |  |
| Rudder Right, Left, Center |  |  |  |  |  |  |  |  |  |  |
| Fixed Heading Test (spin) |  |  |  |  |  | Heading used, Avoidance Off |  |  |  |  |
| Waypoint Course Test (spin) |  |  |  |  |  | Waypoint used, Avoidance Off |  |  |  |  |
| Buoy Release Mechanism |  |  |  |  |  | Tested and Secure |  |  |  |  |
| Photo Document |  |  |  |  |  |  |  |  |  |  |
| RDF Beacon(s) Test |  |  |  |  |  | Test with receiver |  |  |  |  |
| Autonomous Obstacle Avoidance |  |  |  |  |  | Enabled |  |  |  |  |
| WGMS Alarms Checked / Cleared |  |  |  |  |  | “Out of Bounds” = Red. All others = Yellow/Green |  |  |  |  |

Comments

Port side Octopus 1: CCU, water speed, solar panel mid and aft (3, 4)

Stbd side Octopus 2: CCU, buoy release, solar panel fwd (1), radar reflector, light

Technician Signature:

Note:  Once vehicle is launched the following tasks must be completed

- Clear alarms

- Log vehicle modifications in notebook

- Vehicle on mission

- Create LAUNCHED comment into WGMS

- Upload photos to gallery website with album name: VehicleName YYYYMMDD