Talking Points – RP_Pressure Cases

- Added as Characterization of CAMTAG failure. (Pic’s and Sample)

- Ppt. table of failure pressures(3350psi test v.s. 1967psi. calc’d)

- Possible Build Direction differences (Horiz= X-Y, Vert.=Z).

- Definite Build Resolution differences (HD, UHD, XHD)

- Currently – Dogbone Tensile tests (6,121 psi. as advertized)

  - Vertical UHD build 3,492 psi.

  - Horizontal UHD build 4,343 psi.

  - Horizontal XHD build 5,375 psi.

- Currently – Compression Test Coupons.

- Currently – building repeat test cases – test to failure

  - Horiz. Build Dir. & Vert. Build Dir.

- Future - Build Double Size Cases (Same R/t ratio)

- Why?  H.S / College student with access to a “MakerBot” could produce pressure cases to experiment with.