DeepPIV on ____________________________________________	Deployment #______

v 10.0		07_October_2019

Dive Objective/Results: 	______________________________________________________

______________________________________________________________________________

______________________________________________________________________________

______________________________________________________________________________

______________________________________________________________________________

______________________________________________________________________________

Deployment Date: 	______________

Recovery Date: 	______________

Person performing topside checklist: 		_____________________

Pre-deployment:

- Verify that DeepPIV laptop and peripherals are connected

  - Ethernet (for laser control and temp/humidity sensors) – check IP settings (WF: sci interface panel)

  - USB-to-serial PIV camera control (WF: PIVcam on sci interface back panel)

  - USB-to-serial dye pump control (WF: SCI3 on sci interface)

- Verify that Shogun recorder (if desired or as backup) is connected

  - Ship network ethernet to PoE injector

  - PoE injector to LTC (linear timecode converter)

  - LTC to Shogun Sync

  - Input & output signals (WF: need to pass through Shogun to record in VARS)

  - As of April 2019, recording of PIV main stream happens in VARS on MBARI ships. Only secondary stream may be on Shogun.

- Power up DeepPIV

| Western Flyer/Doc Ricketts | Rachel Carson/Ventana | Falkor/Subastian |
| --- | --- | --- |
| Science can power |  |  |
| Science GigE 110 |  |  |
| PIV CAM 24 V |  |  |
| Ether 1 110V |  |  |
| CAM 1 & CAM 2 AUX cams |  |  |
| Dye pump 24 V |  |  |

- Run predive test program and verify correct operation of the following systems:

  - DeepPIV camera (ISO, shutter speed, f-stop, record)

    - Open Sony Camera Software

    - Change camera settings

  - DeepPIV laser

    - Power up the laser, slide the power control to 1.5 V

    - Record output ADC:		__________

  - If using Shogun HD recorder

    - Change file names to correspond to deployment number

    - Check timecode input is received and recorded

    - Record sample footage

  - Dye injector/vortex generator (disconnect tubing if dye injector being used, move piston, reset piston, set different increments)

    - Note COM port on ROV: ________

    - Note COM port on Serial-to-USB converter: ________

    - In DeepPIV software, enable motor

    - Check if green light in software illuminates. If not, navigate to settings file in Public Documents and verify COM port

    - Test motor

- Check that all cable connectors and dummies are installed and tight

- Check that all dye/vortex generator tubing is installed and tight

- Open all dye valves and connectors

- Remove protective covers on laser and camera housings

Deployment:

- Deploy ROV, noting exact time and position of launch.

  - ROV launch time: 	____________ PDT

  - ROV dive number: ____________

  - Latitude: 		__________________ deg

  - Longitude: 		__________________ deg

- Record time it takes for ROV to submerge: 	___________

- Dive plan: refer to 	___________________________________________

- Start DeepPIV and AUX stream recording when event of interest occurs; stop when finished

Recovery:

- Record the time that ROV is on deck: 	____________ PDT

- Power down DeepPIV

- Close all dye injector/vortex generator valves

- Rinse instrument housings and attachments with fresh water

- Reinstall protective covers for laser and camera housings

- Remove dye pump body and soak in fresh water