DeepPIV on MiniROV Deployment #__ Checklist

v 1.2.0		11Nov15

K. Katija

Dive Objective: 	______________________________________________________

________________________________________________________________________

________________________________________________________________________

________________________________________________________________________

________________________________________________________________________

________________________________________________________________________

Deployment Date: 	______________

Recovery Date: 	___________

Person performing topside checklist: 		_____________________

Pre-deployment:

- Ensure that comm and power systems on MiniROV are functioning normally.

- Align the laser and camera

  - Attach protective shroud and cloth to MiniROV’s body

    - Zero digital level based on orientation of MiniROV sitting on deck

    - Verify alignment of shroud using a digital level

    - Record digital level reading on shroud:	________

  - Enable Tilt and Laser controls in LabVIEW

  - Zero the Science Camera

    - Click Zero button in Tilt dialog tab

    - Click Home button in Tilt dialog tab

    - Record digital level reading on camera: 	________

  - Remove laser housing protective cover, and put on protective goggles

  - Power up the laser

    - Rotate the dial to 1.5 V in Laser dialog tab

    - Record output of ADC:	________

  - Verify that laser sheet is perpendicular to Science camera

    - Measure angle of laser sheet on shroud using a digital level

    - Record digital level reading of laser sheet:	________

  - Power down the laser

  - Replace laser housing protective cover

- Remove protective shroud from MiniROV.

- Secure protective shroud on deck of ship.

- Run predive test program and verify correct operation of the following systems:

  - MiniROV LED lights

  - Science camera (adjust zoom, focus, shutter speed, and IR filter settings)

  - AJA HD recorder (record, stop, playback)

  - Dye injector/vortex generator (disconnect tubing if dye injector being used, move piston, reset piston, set different increments)

  - Sizing lasers (measure separation at multiple distances)

- Sync all components (including Science camera) with ship’s NTP

- Start MiniROV dive log

- Check that all cable connectors and dummies are installed and tight.

- Check that all dye/vortex generator tubing is installed and tight.

- Open all dye valves and connectors.

- Remove laser housing protective cover.

- Check that the MiniROV tool sled is secure.

- Check for tools left on and in MiniROV frame.

- Remove MiniROV hold-down straps.

Deployment:

- Start AJA recorder

- Deploy MiniROV, noting exact time and position of launch.

  - MiniROV launch time: 	____________ PDT

  - Latitude: 	_____________ deg

  - Longitude: 	_____________ deg

- Record time it takes for MiniROV to submerge: 	___________

- Dive plan: refer to 	___________________________________________

Recovery:

- Check that the laser is powered down.

- Change the Science camera settings (focus and others) to auto and zoom to wide.

- Complete MiniROV dive log

- Stop recording video with Aja

- Recover MiniROV.

- Secure MiniROV and DeepPIV on the ship deck.

- Record the time that MiniROV is on deck: 	________ PDT

- Close all dye injector/vortex generator valves.

- Replace laser housing protective cover.