DeepPIV on MiniROV Deployment #__ Checklist

v 3.0.0		26April16

K. Katija

Dive Objective: 	______________________________________________________

________________________________________________________________________

________________________________________________________________________

________________________________________________________________________

________________________________________________________________________

________________________________________________________________________

Deployment Date: 	______________

Recovery Date: 	______________

Person performing topside checklist: 		_____________________

Pre-deployment:

- Ensure that comm and power systems on MiniROV are functioning normally

- Verify laser and camera alignment

  - Place protective shroud around DeepPIV instrumentation

  - Remove protective covers on laser and camera housings

  - Check that housing markings on laser and camera housing are aligned

  - Power up the laser

    - Rotate the dial to 2 V in Laser dialog tab

    - Record output of ADC:	________

  - Using digital level, compare laser sheet alignment with glass port on camera housing

    - Record digital level reading on laser sheet: 		________

    - Record digital level reading on camera housing: 	________

  - Power down the laser

  - Replace protective covers on laser and camera housings

  - Remove protective shroud from DeepPIV instrumentation

- Run predive test program and verify correct operation of the following systems:

  - MiniROV LED lights

  - Science camera (adjust zoom, focus, shutter speed, and IR filter settings)

  - DeepPIV camera (ISO, shutter speed, f-stop, record)

  - AJA HD recorder (record, stop, playback)

  - Dye injector/vortex generator (disconnect tubing if dye injector being used, move piston, reset piston, set different increments)

  - Sizing lasers (measure separation at multiple distances)

- Sync all components (including Science and DeepPIV camera) with ship’s NTP

- Start MiniROV dive log

- Check that all cable connectors and dummies are installed and tight

- Check that all dye/vortex generator tubing is installed and tight

- Open all dye valves and connectors

- Remove protective covers on laser and camera housings

- Check that the MiniROV tool sled is secure

- Check for tools left on and in MiniROV frame.

- Remove MiniROV hold-down straps.

Deployment:

- Start AJA recorder

- Deploy MiniROV, noting exact time and position of launch.

  - MiniROV launch time: 	____________ PDT

  - Latitude: 	_____________ deg

  - Longitude: 	_____________ deg

- Record time it takes for MiniROV to submerge: 	___________

- Dive plan: refer to 	___________________________________________

Recovery:

- Check that the laser is powered down

- Complete MiniROV dive log

- Stop recording video with AJA

- Recover MiniROV

- Secure MiniROV and DeepPIV on the ship deck

- Record the time that MiniROV is on deck: 	____________ PDT

- Close all dye injector/vortex generator valves

- Replace protective covers for laser and camera housings