DeepPIV on ______________Deployment #__ Checklist

v 4.0.0		21June16

K. Katija

Dive Objective: 	______________________________________________________

________________________________________________________________________

________________________________________________________________________

________________________________________________________________________

________________________________________________________________________

________________________________________________________________________

Deployment Date: 	______________

Recovery Date: 	______________

Person performing topside checklist: 		_____________________

Pre-deployment alignment:

- Verify that DeepPIV laptop and peripherals are connected (Ethernet, usb, mouse, 4K monitor, AJA/Shogun recorder)

- Power up DeepPIV

- Open Silex USB Controller and connect

- Open Sony Remote Camera Controller and change camera settings to show image on 4K monitor

- Verify laser and camera alignment

  - Place protective shroud around DeepPIV instrumentation

  - Remove protective covers on laser and camera housings

  - Check that housing markings on laser and camera housing are aligned

  - Power up the laser

    - Rotate the dial to 2 V in Laser dialog tab

    - Record output of ADC:	________

  - Using digital level, compare laser sheet alignment with glass port on camera housing

    - Record digital level reading on laser sheet: 	________

    - Record digital level reading on camera housing: 	________

  - Power down the laser

  - Replace protective covers on laser and camera housings

  - Remove protective shroud from DeepPIV instrumentation

  - Close Remote Camera Controller software, disconnect Silex USB connection

- Power down DeepPIV

Pre-deployment:

- Verify that DeepPIV laptop and peripherals are connected (Ethernet, USB, mouse, 4K monitor, AJA/Shogun recorder)

- Power up DeepPIV

- Run predive test program and verify correct operation of the following systems:

  - DeepPIV camera (ISO, shutter speed, f-stop, record)

    - Open Silex USB Controller and connect

    - Open Sony Remote Camera Controller

  - AJA/Shogun HD recorder (record, stop, playback)

  - Dye injector/vortex generator (disconnect tubing if dye injector being used, move piston, reset piston, set different increments)

    - Note COM port on ROV: ________

    - Note COM port on Serial-to-USB converter (either 1/2): ________

    - Open ELMO Controller software

    - Direct connect to motor

    - Set baud rate to 19200

    - Set COM port (either 3/4): ________

    - Connect

    - Use Velocity Mode, hit stop to prevent dye leakage

    - Set speed to 100 rpm and hit go, stop; reverse direction, go, stop

  - Translational stage

    - Ensure that Ethernet cable is connected

    - Open ELMO Controller software

    - Direct connect to motor

    - Set baud rate to 19200

    - Set COM port (either 1/2): ________

    - Connect

    - Use Velocity Mode, hit stop to prevent screw from turning

    - Set speed to 500 RPM and hit go, stop; reverse direction, go, stop

  - Sizing lasers

- Sync all components (including Science and DeepPIV camera) with ship’s NTP and record video on both cameras

- Check that all cable connectors and dummies are installed and tight

- Check that all dye/vortex generator tubing is installed and tight

- Open all dye valves and connectors

- Remove protective covers on laser and camera housings

Deployment:

- Deploy ROV, noting exact time and position of launch.

  - ROV launch time: 	____________ PDT

  - Latitude: 	_____________ deg

  - Longitude: 	_____________ deg

- Record time it takes for ROV to submerge: 	___________

- Dive plan: refer to 	___________________________________________

- Start AJA/Shogun recorder when event of interest occurs; stop when finished

Recovery:

- Record the time that ROV is on deck: 	____________ PDT

- Power down DeepPIV

- Close all dye injector/vortex generator valves

- Rinse instrument housings and attachments with fresh water

- Replace protective covers for laser and camera housings