DeepPIV on ____________________________________________	Deployment #______

v 8.0.0		17August17

K. Katija

Dive Objective/Results: 	______________________________________________________

______________________________________________________________________________

______________________________________________________________________________

______________________________________________________________________________

______________________________________________________________________________

______________________________________________________________________________

Deployment Date: 	______________

Recovery Date: 	______________

Person performing topside checklist: 		_____________________

Pre-deployment alignment:

- Verify that DeepPIV laptop and peripherals are connected (Ethernet, usb, mouse, monitor, AJA/Shogun recorder)

- Power up DeepPIV

- Open Sony Camera Software and modify camera settings

- Verify laser and camera alignment

  - Place protective shroud around DeepPIV instrumentation

  - Remove protective covers on laser and camera housings

  - Check that housing markings on laser and camera housing are aligned

  - Power up the laser

    - Rotate the dial to 2 V in Laser dialog tab

    - Record output of ADC:	________

  - Using focus board after attaching to laser housing, ensure

    - Laser sheet and camera axis are aligned: 	________

    - Laser sheet is aligned with camera plane: 	________

  - Power down the laser

  - Replace protective covers on laser and camera housings

  - Remove protective shroud from DeepPIV instrumentation

  - Close Camera software

- Power down DeepPIV

Pre-deployment:

- Verify that DeepPIV laptop and peripherals are connected (Ethernet, USB/serial, mouse, monitor, AJA/Shogun recorder)

- Power up DeepPIV

- Run predive test program and verify correct operation of the following systems:

  - DeepPIV camera (ISO, shutter speed, f-stop, record)

    - Open Sony Camera Software

    - Change camera settings

  - DeepPIV laser

    - Power up the laser, rotate the dial to 2 V

    - Record output ADC:		__________

    - Measure laser power:		__________

  - AJA/Shogun HD recorder (record, stop, playback)

    - Change file names to correspond to deployment number

    - Record sample footage of ROV science and DeepPIV cameras

  - Dye injector/vortex generator (disconnect tubing if dye injector being used, move piston, reset piston, set different increments)

    - Note COM port on ROV: ________

    - Note COM port on Serial-to-USB converter: ________

    - Open ELMO Controller software

    - Direct connect to motor

    - Set baud rate to 19200 and set COM port: ________

    - Connect

    - Use Velocity Mode, hit stop to prevent dye leakage

    - Set speed to 100 rpm and hit go, stop; reverse direction, go, stop

  - Sizing lasers

- Sync all components (including Science and DeepPIV camera) with ship’s NTP and record video on both cameras of timestamp (using time.gov if NTP not available)

- Check that all cable connectors and dummies are installed and tight

- Check that all dye/vortex generator tubing is installed and tight

- Open all dye valves and connectors

- Remove protective covers on laser and camera housings

Deployment:

- Deploy ROV, noting exact time and position of launch.

  - ROV launch time: 	____________ PDT

  - Latitude: 		_____________ deg

  - Longitude: 		_____________ deg

- Record time it takes for ROV to submerge: 	___________

- Dive plan: refer to 	___________________________________________

- Start AJA/Shogun recorder when event of interest occurs; stop when finished

Recovery:

- Record the time that ROV is on deck: 	____________ PDT

- Power down DeepPIV

- Close all dye injector/vortex generator valves

- Rinse instrument housings and attachments with fresh water

- Replace protective covers for laser and camera housings

- Rinse off DeeppIV