DeepPIV on ____________________________________________	Deployment #______

v 9.0		16_April_2019

Dive Objective/Results: 	______________________________________________________

______________________________________________________________________________

______________________________________________________________________________

______________________________________________________________________________

______________________________________________________________________________

______________________________________________________________________________

Deployment Date: 	______________

Recovery Date: 	______________

Person performing topside checklist: 		_____________________

Pre-deployment alignment:

- Verify that DeepPIV laptop and peripherals are connected

  - Ethernet (for laser control and temp/humidity sensors) – check IP settings (WF: sci interface panel)

  - USB-to-serial PIV camera control (WF: PIVcam on sci interface back panel)

  - USB-to-serial dye pump control (WF: SCI3 on sci interface)

- Set up Shogun if desired (or as backup)

  - Ship network ethernet to PoE injector

  - PoE injector to LTC (linear timecode converter)

  - LTC to Shogun Sync

  - Input & output signals. Note: On WF, need to pass through shogun to record in VARS

  - As of April 2019, recording of PIV main stream happens on VARS. Only secondary stream may be on Shogun.

- Power up DeepPIV

  - WF:

    - Science can power

    - Science GigE 110

    - PIV CAM 24 V

    - Ether 1 110V

    - CAM 1 & CAM 2 crossing camera

    - Dye pump 24 V

- Open Sony Camera Software and modify camera settings (1080p30 or 1080p60)

- Verify laser and camera alignment

  - Place protective shroud around DeepPIV instrumentation

  - Remove protective covers on laser and camera housings

  - Power up the laser

    - Set laser to 2 V in software

    - If laser does not respond and temperature/humidity values are not updated, navigate to config file in Public documents and verify COM port.

    - Record output of ADC:	________

  - Use alignment board after attaching to laser housing and follow detailed procedure “Pre-deployment alignment procedure”

  - Power down the laser

  - Replace protective covers on laser and camera housings

  - Remove protective shroud from DeepPIV instrumentation

  - Close Camera software

- Power down DeepPIV

Pre-deployment:

- Verify that DeepPIV laptop and peripherals are connected.

- Power up DeepPIV

- Run predive test program and verify correct operation of the following systems:

  - DeepPIV camera (ISO, shutter speed, f-stop, record)

    - Open Sony Camera Software

    - Change camera settings

  - DeepPIV laser

    - Power up the laser, rotate the dial to 2 V

    - Record output ADC:		__________

  - If using Shogun HD recorder

    - Change file names to correspond to deployment number

    - Check timecode input is received and recorded

    - Record sample footage of ROV science and DeepPIV cameras

  - Dye injector/vortex generator (disconnect tubing if dye injector being used, move piston, reset piston, set different increments)

    - Note COM port on ROV: ________

    - Note COM port on Serial-to-USB converter: ________

    - In DeepPIV software, enable motor

    - Check if green light in software illuminates. If not, navigate to settings file in Public Documents and verify COM port

    - Test motor

  - Sizing lasers

- Check that all cable connectors and dummies are installed and tight

- Check that all dye/vortex generator tubing is installed and tight

- Open all dye valves and connectors

- Remove protective covers on laser and camera housings

Deployment:

- Deploy ROV, noting exact time and position of launch.

  - ROV launch time: 	____________ PDT

  - ROV dive number: ____________

  - Latitude: 		_____________ deg

  - Longitude: 		_____________ deg

- Record time it takes for ROV to submerge: 	___________

- Dive plan: refer to 	___________________________________________

- Start VARS auxiliary stream recording when event of interest occurs; stop when finished

Recovery:

- Record the time that ROV is on deck: 	____________ PDT

- Power down DeepPIV

- Close all dye injector/vortex generator valves

- Rinse instrument housings and attachments with fresh water

- Replace protective covers for laser and camera housings

- Remove dye pump body and soak in fresh water