Kakani Katija and Alana Sherman

Figure 1. DeepPIV integrated on Rachel Carson’s port manipulator during a test cruise (for benthic operations) in October 2016.

DeepPIV: From the surface to the benthos

Since the start of 2015, we have been developing DeepPIV, an instrument that enables fine temporal and spatial scale measurements of fluid motion in situ. Since then, we have built and integrated DeepPIV on all of MBARI’s ROVs. In the spring of 2016, DeepPIV was used during both midwater (with Bruce Robison and Postdoctoral Fellow Anela Choy) and benthic (with Jim Barry and Postdoctoral Fellow Joshua Lord) cruises on R/V Rachel Carson. These efforts addressed several science questions including the following:

- How do benthic organisms survive in a highly variable, dynamic environment? How is feeding of benthic organisms (including deep sea corals and sponges) linked to environmental flows, and do feeding mechanisms change in response to their environment?

- What are the distributions of microplastics between the surface and the benthos? How do we conduct vertical transect sampling routines that are robust and repeatable during midwater cruises to most accurately quantify environmental microplastics?

On the development side, the project team is working to integrate a new camera to the DPIV system. The new camera will have the additional features of zoom and focus. It will be housed in a significantly smaller housing than our current camera, which will make deployments much easier both on our ROVs and ships/ROVs of opportunity. We also expect to be able to bring up higher resolution video. A separate development effort currently underway in the lab is to integrate a galvanometer into the system. The galvanometer is a spinning optic that creates a laser sheet of X times the intensity of our current laser sheet. We have made progress on testing these components on the bench in the lab with the goal of moving towards a deployable addition to our instrument.

Finally, the first publication from this project was published in Science Advances. The response from the press was impressive, news of our results was covered in The New York Times, NPR Science Friday, and BBC world news amoung others.

Figure 2. (A) Giant larvacean Bathochordaeus stygius in house using white-light illumination. (B) DeepPIV scans yield three-dimensional reconstructions of the larvacean house structure. (C) DeepPIV has also been used to quantify volumetric flow rates into the suction sampler, streaklines of incoming particles shown in (D), for determining environmental concentrations of plastics in midwater.

Publications and Presentations (* - shared first authorship)

- Katija K., Sherlock R.E., Sherman A.D., Robison B.H. (2017), New technology reveals role of giant larvaceans in oceanic carbon cycling. Conditionally accepted by Science Advances.

- Katija* K., Choy* C.A., Sherlock R.E., Sherman A.D., Robison B.H. (2017). From the surface to the seafloor: How giant larvaceans can move microplastics into the deep sea. Submitted.