Kakani Katija and Alana Sherman

DeepPIV: From the surface to the benthos

Since the start of 2015, we have been developing DeepPIV, an instrument that enables fine temporal and spatial scale measurements of fluid motion in situ. Since then, we have built and integrated DeepPIV on all of MBARI’s ROVs. Thus far the DeepPIV instrument has been deployed  on 60 ROV dives, and has the extraordinary track record of collecting science data on every dive. The instrument was used during both midwater (with Bruce Robison and Postdoctoral Fellow Anela Choy) and benthic (with Jim Barry and Postdoctoral Fellow Joshua Lord) deployments. These efforts addressed several science questions including the following:

- How do benthic organisms survive in a highly variable, dynamic environment? How is feeding of benthic organisms (including deep sea corals and sponges) linked to environmental flows, and do feeding mechanisms change in response to their environment?

- What are the distributions of microplastics between the surface and the benthos? How do we conduct vertical transect sampling routines that are robust and repeatable during midwater cruises to most accurately quantify environmental microplastics?

The latest improvement to the system has been an upgraded camera which delivers HD video to the ROV control room with minimal loss from video compression or decoding. In addition, this new camera has provided us with the ability to remotely adjust the zoom and focus settings. Finally, it is significantly smaller than our previous camera, which is important for mounting the package the ROV manipulator. In July and August of 2017, on two separate Ventana dives, we were able to test the new camera with the rest of the existing DPIV system. The resulting video quality was excellent and we were able to validate our measurements by collecting flow from a known source.

In addition to our technical upgrades, the second publication from this project was published in Science Advances. This publication focused on the role Larvaceans play in transporting microplastics to the deep sea. The response from the press was impressive; news of our results was covered in Wired, National Geographic News, and PBS Newshour among others.

Figure 1. This image shows the particle field surrounding a benthic sponge. This video frame grab was collected using the upgraded camera on the DPIV system.

Publications and Presentations (* - shared first authorship)

- Katija K., Sherlock R.E., Sherman A.D., Robison B.H. (2017), New technology reveals role of giant larvaceans in oceanic carbon cycling. Science Advances. Vol. 3, no 5, e1602374.

- Katija* K., Choy* C.A., Sherlock R.E., Sherman A.D., Robison B.H. (2017). From the surface to the seafloor: How giant larvaceans can move microplastics into the deep sea. Science Advances. Vol 3, no 8, e1700715.