Seaconn Contact Current Handling

### ELECTRICAL CONTACT SIZES

| CONTACT AWG | DIAMETER (INCHES) | AMPACITY (AMPS) |
| --- | --- | --- |
| #28 AWG | 0.015 | 0.5 |
| #22 AWG | 0.031 | 2.5 |
| #20 AWG | 0.040 | 4 |
| #16 AWG | 0.062 | 13 |
| #14 AWG | 0.093 | 16 |
| #10 AWG | 0.125 | 23 |
| #8 AWG | 0.156 | 50 |
| #6 AWG | 0.218 | 65 |
| #4 AWG | 0.255 | 90 |
| #1/0 AWG | 0.355 | 200 |

NOTES:
The ampacity is computed at temperature range of +30°F +68°F.
Above 70°F the ampacity drops.
Pins and sockets are brass alloy. Pins are gold plated per MIL-G-45204.
Most contacts conform to MIL-C-24217.