Jan 16, 2019 DeepPIV Improvement Meeting

In attendance: Knute, Ben, Bryan, Marko, Mike B., Denis, Joost, Alana, Kakani

- Discussion to decrease the size of manipulator package

  - Pump Improvements

    - Pump comp is very large and can be replaced with tygon tubing

    - Pump can be moved to position near handle (or possibly on arm)

    - Handle can be placed between two currently existing handles (Randy, Marko, and Mike B. to discuss handle design amongst themselves)

    - Alternative idea is to move the pump to the core vehicle (will need to determine if longer tubing on pump has any effect)

  - Laser probe head improvements

    - Make connector perpendicular to current orientation- probably by adding a block not a right-angle connector

    - Add port to back of probe housing (where connector is now) to help with alignment during assembly

    - Possible issue with orientation of sheet being unknown when housing sealed

  - Hydraulic Ram mount for Ricketts

    - All interested in continuing to pursue this option

    - Marko will work with Jon E. on design

    - Would be nice to use for 3D reconstruction scans but not essential

  - Crossing Camera Orientation

    - See image below of what we hope will be the go-to configuration

    - Add index to the mount for crossing camera

  - Other ideas

    - Move bar higher, make into a U-channel for the cable runs, connects higher up on probe housing

    - Add j-box with focal 914 to reduce cabling on manip (Alana to look into)

    - Do we need two Aux cameras? Answer is yes for now.