# DeepPIV remote head assembly procedure

This list outlines the steps to take in assembling the DeepPIV remote head, as a guideline for the design. Important consideration is decoupling assembly, test and qualify of ALL optical components before final remote head assembly.

- Assemble, test and qualify optical head subassembly in lab

  - Check fiber output quality: beam pattern, NA and power

  - Attach fiber to collimator, tighten set screw and adjust collimator focus to desired distance (in water)

  - Install desired Powell lens into “spider”, and pay attention to lens orientation

  - Attach collimator to spider/lens subassembly [3 set screws; set final torque]

  - Align Powell lens to collimator output

  - Fine-tune collimator output angle with set screws on collimator body to optimize laser line symmetry

  - Attach collimator+spider assembly to window assembly (we may need to clock correctly at this stage)

  - Adjust collimator beam angle to reduce ghost images while maintaining laser line symmetry

    - It might be necessary to shim the spider to the front window assembly

  - Tighten collimator screws

  - Mark subassembly with optical properties on outside

  - Optical head assembly is aligned. Fiber can now be detached (add protective cap).

- Pre-assemble fiber penetrator to DeepPIV remote head main body

- Select appropriate optical subassembly and connect to fiber

  - Tighten collimator set screw

- Attach optical subassembly to main body of remote head

  - DESIRED: Clock optical head subassembly appropriately

  - Fiber management

  - Add desiccant

  - Close waterproof remote head housing

- Verify