Specifications for interfacing to the Coherent Taipan 639-2000 STM OPS Laser

- Inputs to the Laser

  - Power to the Laser: 110 VAC, 6 Amax powers AC-DC, this produces 12V at 20 Amax which feeds in to the LEC UDD2 Power Pack controller.

  - Laser Output Control: Analog signal: 0-5 VDC.  5VDC = 2 Watts output

  - Laser Interlock Connection: Two Interlock pins shorted means laser is enabled, an open connection means the laser is powered on but the output is disabled.

  - Frame Gnd connection: connects the earth ground of the AC-DC to the pressure case and then to the ROV frame which is then connected to the hull of the ship.

- Outputs from the laser

  - Current Monitor: 1mV=1A, current at 12V

  - Temperature: ? serial

  - Photo diode?

- I/O from laser housing

  - Temperature sensing:  HTM 25000