3/7/2018 Harbor Conference room 13:00-15:00

PW: Peter Walz;

DG: Dale Graves;

BE: Ben Erwin ;

JD: Joost Daniels;

DK: Denis Klimov;

AS: Alana Sherman;

PM: Paul McGill;

JE: Jon Erickson

KK: Kakani Katija

13:00 AS slides electric schematic presentation

PW: grounding of the laser housing?

BE: Rickets is using ground clamp on the deck. Procedural item. Grounding discussion.

DG: not common way between different ROV.

DK: is monitoring connector present here?

AS: not all monitoring pins on schematic – current monitoring, not specified

DK: add monitoring of photodiode in front of the laser head?

AS: 12V Power Supply slide

PM: what about 24V?

AS: low power, 12V sufficient

AS: How we safely use on the ship?

DK: spontaneous lasing was observed in the lab on 2 occasions

AS: Laser Interlock and Control Board. Clipper circuitry, limiting laser control voltage to 0.25V, clamping down. Need to engage relay to bring from eye-safe mode. Second relay – disable interlock. So, 2 things needs to be done to bring from those safe modes. Filtering circuitry: needed for min load

PM: what is safe level? Can it be measured?

PW: what is power where it is stable?

DG: what about hardware instead of software? Hardware button? GUI button.

AS: software srash: default: low power mode.

DG: DGH – does it have watchdog timer?

PM: this needs to be done, restart DGH.

PW: we had mechanical shutter in front of the laser. Hydraulic rotator. Control from vehicle.

DG: mini does not have hydraulics.

AS: would prefer non-electric design

EW: implement inside of housing?

AS: remaining electric issues slid: inrush current; simplify clipper circuitry; replace serial and DGH with Ethernet data acq module; add second serial channel to camera housing for temp/ humidity sensor

AS: GUI panel. Two additional buttons.

KK: slider control instead of round?

PM: is there is room…

JE: need 3 failures.

DK: more dasngerous bringing back to safe mode

PM: conformation? Power monitor? Monitoring current…

DG: you have analog input, feedback?

PM: monitor laser, to monitor current

AS: need to contact Coherent to check what pint to monitor current - instead of camera housing.

AS: could check serial channel to free up analog channel.

AS: that is all for electrical and software

BE: RS232 configuration 485?

AS: will figure out.

JE: mechanical design slides. Assembly on the manipulator. Can itself, same length; smaller diameter, 28 lb lighter. Way ahead! Wedge clamp to hold heatsink against housing. Need to reach from back opening and set torque on the screw.

DG: connected to one end plate?

JE: 1 psi pressure, compression of the housing under pressure.

JE: compliance in the plate, it will spring.

PW: what about port, opening?

AS: use to have on floats, not easy, need guide to drive the screwdriver inside.

PM: minor thing, could take care of it later.

JE: Philips or hex/ ball drive.

DK: risk of thermal run-away pf the laser/ hot side.

PM: are there fans in the laser can?

JE: 2 fans inside, on the driver. Power supply dumps heat to inside air. Laser heat goes to heatsink on the bottom.

PM: what about running on deck?

JE: relatively low power anticipated on the deck.

DG: is there thermal shutdown in the laser?

DK: yes there are number of parameters monitored inside/ shutdowns.

AS: what about serial port for monitoring? We can check on that.

DG: that is the form of thermal probe – can you measure heatsink temperature?

AS: yes! Running out of analog inputs. Buy a new Ethernet interface/ module ACD.

PW: does it need very long run?

AS: 10-15 minutes.

PW: it is not too long.

AS: short time.

PM: concern – power supply heat/ overheating? Dumping 10s of Watts into small volume.

JE: did a calculation.

DK: problem for calculations - amount of heat goes up exponentially, what amount use for calculations?

PM: worst case: on deck, in the sunlight. What is warmest water?

PW: 18C perhaps.

DK: would be good to monitor 2 temperatures: air and heatsink

DG: do you have parameters logging?

AS: free up 2 channels? Or replace (to Ethernet module?). Temperature and laser power?

DK: laser power useful/ needed for data processing?

KK: could be. Variations of power/ stability?

DG: just log them all (parameters).

DK: what really do we need, what parameters? Camera zoom settings for instance?

PM: those are 2 separate interfaces.

AS: ……. working on Sony interface.

KK: not a high priority.

PM: make it “desirable”. Log everything, including state of safety parameters.

JE: slide on Laser Remote Head.

PM: so better if pointed down for safety?

DG: the assembly seems to be too tall for mini-ROV in point-down position!

BE: better management for bags, peristaltic pumps? Zip ties, axillary cameras. Better mounts for cameras.

DK: camera orientation is important for pilots/ needs to be consistent. Especially if instrument orientation is changed from horizontal to vertical / or back.

KK: we now log this in a mob document.

AS: good point on bags/ mounting.

BE: and do not forget cable management! What about 3D-printed camera brackets? Like on mini-ROV?

AS: containment for zip bags. And brackets.

JE: Fiber assembly slide. 8 meters oil filled fiber connector.

PM: why is it so long?

JE: length of manipulator and location of laser can on ROV

JE: buy fibers, assemble here, ship to Coastal Connections, have them terminate connectors.

JE: Potted Fiber Terminals slide. Shipping to Coastal Connection.

PW: why ship there?

DK: we use high-power cap connectors, that is why.