- The current version of the DeepPIV v.4 LabView software can be found in the MBARI bitbucket repository: https://bitbucket.org/mbari/dpiv4/

- The version of the software which runs the camera, injector, and laser is the branch “SonyCoherentGUI”. The master just operates the laser. I will change this when the software is finished.

- You will need an account on Atlassian.com to access the repository. You can clone the repository to get a local copy of the latest version. I use Sourcetree which allows has a user interface for push/pull/cloning. But you can just do a clone request on the command line.

- To run the software you need a LabView version 18.

- Install the LabJack UD drivers for windows

- You need the following libraries added to labview (you can use the package manager to load them): LabJack LJUD, OpenG (for timer fcns), NI GOOP (not positive on this one)

- Open the Project DeepPIV4MultiQ.lvproj