Tank Testing Objectives

Out-of-water:

- Align laser on vehicle with camera

  - Ensure that camera is pointed horizontal using levels

  - Ensure that laser sheet is vertical using levels

  - Measure angle between two planes:

  - Verify that (c) is 90 deg

- Control of laser

  - Measure power output of laser at 1.5 V: 		~30 mW

  - Measure power output of laser at 5 V:		~1200 mW

  - Measure duration of laser power before beam is stable

- Control of vortex generator/dye injector

  - Test L/D ~2

  - Test L/D ~4

  - Test L/D ~8

  - Reverse piston direction to refill/reset generator

  - Does dye bag actuation work

- Control of camera alignment

  - Ensure that camera is pointed horizontal using levels

  - Move camera to opposite ends of the position spectrum

  - Home the camera and verify it’s in correct location

- Align sizing lasers

  - With object of known size, place in center of camera FOV with sizing lasers on

  - Step away from camera at 1 ft/m intervals, recording video the whole time

  - Zoom in on object, keeping its size consistent with each interval

  - Verify spacing between lasers consistent at all distances

- Ensure that Giancarlo’s sensor is logging

- Control of recorder

  - Record

  - Rewind and playback

  - Export

In-water:

- Verify alignment of laser

  - Turn on camera and laser; zoom in camera such that entire FOV is illuminated by laser; manually focus to ensure that particles are crisp

  - Record; stop; rewind; look at individual frames

  - Look closely at particles on edges of frame; if particles are blurry on the right or left side, correct laser housing in yaw; if particles are blurry on the top or bottom edges, correct laser housing in roll

- Verify camera settings at 1080p 60

  - Adjust shutter speed, setting where laser too dim:

  - Adjust iris, setting where laser too dim:

  - With laser sheet and sizing lasers on, camera recording and zoomed in until entire FOV is illuminated by laser

  - Zoom all the way to find minimum FOV:

- Measure laser sheet spread angle

  - With laser sheet and sizing lasers on, camera recording and zoomed out, and hovering near object in FOV of camera

  - Measure spread angle

    - 60 deg line optic:

    - 90 deg line optic:

  - Estimate largest FOV that can be illuminated using optic

    - 60 deg line optic:

    - 90 deg line optic:

- DPIV measurements with HID/LED lighting

  - With laser sheet on, camera recording and zoomed in to illuminated FOV, HID lights off, and vehicle hovering in space

  - With laser sheet on, camera recording and zoomed in to illuminate FOV, HID lights off, and vehicle hovering with object in laser sheet

  - With laser sheet on, camera recording and zoomed in to illuminate FOV, HID lights on, and vehicle hovering in space

  - With laser sheet on, camera recording and zoomed in to illuminate FOV, HID lights on, and vehicle hovering with object in laser sheet

  - Can pilot see laser sheet to align with object?

  - Separate color channels

    - Can you see particles?

    - How does average laser intensity change with separated color channels?

- Measure vortex generator flow

  - With laser sheet on, camera recording and zoomed in to illuminate FOV, HID lights off, and vehicle hovering in space

  - PIV vortex ring with L/D ~2

  - PIV vortex ring with L/D ~4

  - PIV vortex ring with L/D ~8

  - Adjust camera settings to optimize PIV results

    - Iris:

    - Shutter speed:

- Test dye injector settings

  - Attach dye pouch

  - With camera recording and zoomed out, laser off, HID lights on, vehicle hovering near object in FOV

  - Adjust speed and distance piston travels

  - Settings for small vortex rings:

  - Settings for large rings:

  - Settings for continuous stream:

  - Estimate number of dye visualizations before refill is needed:

- Accelerometer tests on vehicle and laser arm

  - Ensure that Giancarlo’s sensor is logging (CTD, DVL, accelerometer)

  - Prescribe motion with sensor on vehicle

  - Prescribe same motion with sensor on laser arm/housing

- Verify DVL speed with separate camera

  - Vary speed

  - Vary distance of water mass DVL is referencing

  - Compare with bottom lock

  - Fixed camera video used as direct comparison

- Hover near objects

  - Practice communication

  - Practice approach

  - Practice 3D reconstruction

  - Practice DeepPIV

- Jellyfish from aquarium?