# PIC32 and FreeRTOS Resources Used by OASIS5

### Project: OASIS5 – Project 901507
Revision - 1.0
Date: 5/25/2016

# Interrupt Priorities

| 7 | PWR_FAIL |
| --- | --- |
| 6 |  |
| 5 | Modbus over SPI |
| 4 | UARTS |
| 3 | Modbus Timers |
| 2 | RTC on INT4 |
| 1 | FreeRTOS Kernel, Ticker |
| 0 |  |

FreeRTOS.h:  
configMAX_SYSCALL_INT_PRIO  is set to 5.  This allows the hardware interrupt handlers to use the FromISR functions.  See http://www.freertos.org/a00110.html#kernel_priority

configKERNEL_INTERRUPT_PRIORITY is set to 1, per the “Kernel” entry above.

# PIC32 Timers

| 1 | FreeRTOS ticker (FreeRTOS/PIC32/port.c) |
| --- | --- |
| 2 | Modbus SPI |
| 3 | Modbus UARTs |
| 4, 5 | Unused |

# FreeRTOS Task Priorities

| 5 | Shutdown Task |
| --- | --- |
| 4 | Kernel Timer Task |
| 3 | Interrupt Completion Threads |
| 2 | Oasis Drivers |
| 1 | Oasis Scheduler |
| 0 | Idle |

FreeRTOS.h:  configTIMER_TASK_PRIORITY = 4