Table 1. Current and proposed parameters measured for integrated time-series

| Parameter | Approximate depth | Instrument | Platform today |
| --- | --- | --- | --- |
| Beam Attenuation | 0m | Chelsea/Seatech/Wetlab Cstar | Ship, Underway |
| Beam Attenuation | 0m to 1000m | CTD: Chelsea/Seatech/Wetlab Cstar | Ship Cast |
| Chla | 0m | WetStar/SeaTech/Turner | Ship, Underway |
| Chla | 0m to 1000m | CTD: Fluorescence, Wetlab ECO-AFL/FL | Ship Cast |
| Chlorophyll | 0m to 1000m | Water samples: Turner Fluorometer | Ship Cast |
| Chlorophyll | 3950 m | SES: fluorescence, image | Mooring |
| Chlorophyll | 4000m | Rover fluorescence | Benthic Rover |
| Conductivity | 0m | SBE | Ship, Underway |
| Conductivity | 0m to 1000m | CTD: SBE | Ship Cast |
| Density | 0m to 1000m | CTD: SBE | Ship Cast |
| Faunal communities | 50-1000m (ROV transects) | HD, color video | ROV |
| Faunal communities | 4000m (time lapse) | moored time lapse camera | Mooring |
| Faunal communities | 4000m (ROV transects) | HD, color video | ROV |
| Fluorescence | 0m | WetStar/SeaTech/Turner | Ship, Underway |
| Fluorescence | 0m to 1000m | CTD: Fluorescence, Wetlab ECO-AFL/FL | Ship Cast |
| Nutrients | 0m to 1000m | Water samples: AlpChem autoanalyzer | Ship Cast |
| Oxygen | 0m to 1000m | CTD: SBE | Ship Cast |
| Oxygen | 0m to 1000m | Water samples | Ship Cast |
| Oxygen | 50-1000m, 1000m | ROV, CTDO | ROV |
| Particulate CHN | 4950 and 3400m | Sediment traps | Mooring |
| Particulate organic matter | 51 and 600 mab | SES | Mooring |
| pH | 0m | Durafet II | Ship, Underway |
| Phytoplankton | 0m to 1000m | CTD: Epifluorescence Microscopy | Ship Cast |
| Phytoplankton | 4950 and 3400m | Sediment traps, microscopy, SES | Mooring |
| Primary Production | 0m to 1000m | Water samples: Beckman LS-3801 liquid scintillation counter | Ship Cast |
| Salinity | 0m | SBE | Ship, Underway |
| Salinity | 0m to 1000m | CTD: SBE | Ship Cast |
| Salinity | 50-1000m, 4000m | ROV, CTDO | ROV |
| Sediment CHN, chlorophyll | 4000m | Sediment cores | ROV |
| Sediment community O2 use | 4000m | Benthic Rover transect | Benthic Rover |
| TCO2 | 0m | Licor 820 | Ship, Underway |
| TCO2/Alkalinity | 0m to 1000m | CTD: Licor 820 | Ship Cast |
| Temperature | 0m | SBE | Ship, Underway |
| Temperature | 0m to 1000m | ROV,CTDO | ROV |
| Transmissivity | 0m | Chelsea/Seatech/Wetlab Cstar | Ship, Underway |
| Transmissivity | 0m to 1000m | CTD: Chelsea/Seatech/Wetlab Cstar | Ship Cast,ROVt |
| Zooplankton (cms) | 50-1000m | HD, color video | ROV |
| Zooplankton (microns - cms) | 0-100m, 100m intervals to 1000m | Vertical camera profiler, acoustic profiler, Holo camera | Ship Cast/AUV |

Table 1 ctd.

| Platform Start of Time Series Project | Platform 3- 5 yr into project | Ultimate platform |
| --- | --- | --- |
| wave glider? | Wave glider | Wave glider |
| ship cast/AUV Gulper (in shore) | Vertical profiler/AUV | Energy harvesting verticle profiler |
| wave glider? | Wave glider | Wave glider |
| ship cast/AUV Gulper (in shore) | Vertical profiler/AUV | Energy harvesting verticle profiler |
| ship cast/AUV Gulper (in shore) | LRAUV water sampler | In situ measurement |
| Mooring | Mooring | Mooring |
| Benthic Rover | Extended Benthic Rover | Extended Benthic Rover |
| wave glider? | Wave glider | Wave glider |
| ship cast/AUV Gulper (in shore) | Vertical profiler/AUV | Energy harvesting verticle profiler |
| ship cast/AUV Gulper (in shore) | Vertical profiler/AUV | Energy harvesting verticle profiler |
| i2map | i2map on AUV3 | i2map on AUV3 |
| Time Lapse Camera | Time Lapse Camera | AUV serviceable Time-lapse camera |
| ROV | Benthic Imaging on AUV3 | Benthic Imaging on AUV3 |
| wave glider? | Wave glider | Wave glider |
| ship cast/AUV (in shore) | Vertical profiler/AUV | Energy harvesting verticle prfiler |
| ship cast/AUV Gulper (in shore) | LRAUV water sampler | In situ measurement |
| ship cast/AUV Gulper (in shore) | Vertical profiler/AUV | Energy harvesting verticle prfiler |
| ship cast/AUV Gulper (in shore) | LRAUV water sampler | In situ measurement |
| ROV | Dorado,0-1000m | LRAUV deep |
| Mooring | Mooring- extended deployment? | In situ Carbon measurement |
| SES | SES upgraded | SES upgraded |
| Mooring | Wave glider | Wave glider |
| ship cast/AUV Gulper (in shore) | LRAUV water sampler | In situ measurement |
| Mooring | Mooring | Mooring |
| ship cast/AUV Gulper (in shore) | LRAUV water sampler | In situ measurement |
| Mooring | Wave glider | Wave glider |
| ship cast/AUV Gulper (in shore) | Vertical profiler/AUV | Energy harvesting verticle prfiler |
| ROV | Dorado, 0-1000m | LRAUV deep |
| ROV | Benthic Rover with corer | Benthic Rover with corer |
| Benthic Rover | Extended Benthic Rover | Extended Benthic Rover |
| wave glider? | Wave glider | Wave glider |
| ship cast/AUV Gulper (in shore) | Vertical profiler/AUV, Dorado | Energy harvesting verticle prfiler |
| wave glider? | Wave glider | Wave glider |
| ship cast/AUV Gulper (in shore) | Vertical profiler/AUV | Energy harvesting verticle prfiler |
| wave glider? | Wave glider | Wave glider |
| ship cast/AUV Gulper (in shore) | Vertical profiler/AUV | Energy harvesting verticle prfiler |
| i2map | i2map on LRAUVdeep | i2map on LRAUVdeep |
| AUV with Holo camera | AUV with Holo camera | AUV with Holo camera |

Table 1 ctd.

| How often today? | How often future? |
| --- | --- |
| every 21 days | (blank) |
| every 21 days | (blank) |
| every 21 days | (blank) |
| every 21 days | (blank) |
| every 21 days | (blank) |
| 3-12 hours | 3-12 hours, adjustable |
| 2-4 day | (blank) |
| every 21 days | (blank) |
| every 21 days | (blank) |
| every 21 days | (blank) |
| daily to weekly | monthly |
| hourly | (blank) |
| 6-12 months | monthly |
| every 21 days | (blank) |
| every 21 days | (blank) |
| every 21 days | (blank) |
| every 21 days | (blank) |
| every 21 days | (blank) |
| Every two months, Midwater, annually benthic | monthly |
| every ten days | (blank) |
| 6-12 hour | (blank) |
| every 21 days | (blank) |
| every 21 days | (blank) |
| every ten days | (blank) |
| every 21 days | (blank) |
| every 21 days | (blank) |
| every 21 days | (blank) |
| Every two months, Midwater, annually benthic | (blank) |
| 6-12 months | (blank) |
| 2-4 day | (blank) |
| every 21 days | (blank) |
| every 21 days | (blank) |
| every 21 days | (blank) |
| every 21 days | (blank) |
| every 21 days | (blank) |
| every 21 days | (blank) |
| 30-60 days | monthly |
| daily to weekly | (blank) |