| YEAR | 2014 |  | 2015 |  |  |  | 2016 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Monitoring of upper ocean parameters at Sta. MMB and Sta. MENP | Monthly with ship cast (Sta. MMB) | Development of water-sampling LRAUV complete | Bi-weekly monitoring and data processing of upper ocean parameters at Sta. MMB and Sta. MENP offshore by water-sampling LRAUV |  |  |  |  |  |
| Monitoring of midwater faunal community at Sta. MMB and Sta. MENP | Monthly with ROV (Sta. MMB) |  | Development of midwater imaging AUV complete | Bi-weekly monitoring of physical and chemical parameters as well as the distribution, abundance and size of pelagic fauna using optical and acoustic imaging at 100m intervals down to 1000 m depth and processing data at Sta. MMB and Sta. MENP using midwater imaging AUV |  |  |  |  |
| Monitoring of benthic community at Sta. MMB and Sta. MENP | Yearly with ROV (Sta. MENP) |  | Development of benthic imaging AUV complete | Begin bi-weekly monitoring of the physical and chemical parameters as well as the distribution, abundance and size of benthic boundary layer fauna using optical and acoustic imaging from the sea floor to 50 m above bottom at both stations Sta. MMB and Sta. MENP, using benthic imaging AUV |  |  |  |  |
| Sediment traps and time-lapse camera mooring, servicing, and data processing | Annual servicing Sta. MENP |  | Install existing mooring of two sediment traps and time-lapse camera at Sta. MMB | Sedimentation and sea floor monitoring with annual servicing of moorings, at both stations. Data analysis automated |  |  |  |  |
| Data management and acquisition system for MBARI time-series data |  | Development complete | Used internally for data management and analysis |  |  |  | Data also available to public domain |  |
| Sediment trap (500m) w/extended mooring at Sta. MENP |  |  | Purchased and Installed | Sedimentation monitoring with annual servicing of mooring. Data analysis automated |  |  |  |  |
| Profiling CTD, benthic winch and upward/looking sonar for MARS (Sta. MMB) |  |  | Purchase and reconfigure for MARS (Sta. MB) |  |  |  | Installed at MARS (Sta. MMB) | Continuous collection of physical, chemical and backscatter data from MARS instrumentation |
| VARS routines for processing AUV images from midwater and benthic transects |  |  | Development complete | VARS used for processing AUV images from midwater and benthic transects |  |  |  |  |
| Export time-series instrumentation and data management system |  |  |  |  |  |  | Exported instrumentation and data management system |  |

Table 1. Schedule for development and implementation of integrated time-series program