1 July 2015

Park-n-Fly AUV proposal

Overview:

Autonomy from ship-based measurements – key to success of long time-series studies

Critical goal of integrated time-series at MBARI.

Two internally funded projects to develop 2 AUVs - sampling/imaging upper 1000 m water column

| Vehicle | LRAUV | Midwater AUV | Deep AUV |
| --- | --- | --- | --- |
| Depth rating | 300m | 1000m | 4000m |
| Power Duration | Weeks | Days | Year |
| Range | 1000km | 100km | 100km |
| Sensors | CTD, O2, Fluorometer | CTD, O2, Fluorometer | CTD, O2, Transmissometer |
| Camera system | Under development | Video w/lights | Still w/strobes |
| Acoustic backscatter | Under development | EK15 | Multibeam (200kHz) |
| Total onboard energy | 6kwhr(rech)/ 18kwhr(primary) | 10kwhr | 20kwhr |

Table 1.  Comparison of existing time-series AUVs and proposed Deep AUV (Park-n-Fly AUV)

Figure 1 – Cartoon of 3 AUVs with depth

Proposed capabilities of Park-n-Fly AUV:

- Working depth to 4000 m

- Bottom photo-transect period (48 hrs over 12 month deployment) seasonal or event triggered transects over same trajectory (events detected by SES and Rover)

- Benthopelagic acoustic backscatter transect period (48 hrs over 12 month deployment) seasonal or event triggered transects over same trajectory (concurrently with photo-transects)

- Record CTD/O2 data on each transect

- Dock on sea floor in sleep-mode between 12-hour surveys

- Communicate via acoustic modem with Wave Glider for status reports only

- Emergency scuttling and recovery

Scientific justification:

26-year time series at Sta. M reveal significant daily to weekly changes in benthic community and environment.

Figure 2 -Salp paper plot w/time

Figure 3 - Peniagone plot w/ time

Figure 4 – Echoview plot of backscatter data from Reson Multibeam data in Monterey Canyon

Why mobile versus series of stationary platforms in study area:

- Seasonal or event triggered surveys not feasible using ship-based facilities

- Series of stationary moorings not realistic given patchy heterogeneity of bottom communities on scales of <10m to > 1km Why do this with a mobile platform?  Justify need for spatial coverage. Patchiness at scale of 100m, need far more than 5 replicate systems. Turnover time of 5 systems would significantly increase needs for ship time. Crissy, Kathy, Henry to get patchiness references

- System flexibility, scalable frequency vs spatial coverage (considering docking error, number of times docking needed)

Table 2. Cost comparison of ship-based versus Park-n-Fly photo transects—CHART OF CROSSOVER TIME (cost vs. time), what is available now in house vs have to buy.

| Cost | Deep AUV | Shiptime with ROV |
| --- | --- | --- |
| Dorado AUV | $600k |  |
| Batteries | $40k |  |
| Camera and strobe* | $105K |  |
| Sonar (dual head T20)* | $320k |  |
| Additional bio-optic Instrumentation(Lisst-100, etc) | $100k |  |
| INS* | $200k |  |
| Acoustic modem | $5k |  |
| USBL* | $60k |  |
| Docking riser | $150k |  |
| Echoview (MBARI-wide) (Kelly as a collaborator?) | $300k |  |
| TOTAL | $1.9M | $ |
| *=in house, could be used by this project |  |  |

Western Flyer/Rickets Day Rate - $28,100

If we did 7 day trips 3 times a year (we’ll call one trip a year “free”, since it is required for turnaround of other instruments; 4 trips total a year is not realistic), this would cost 3*7*$28,100 = $590,00/year

So, the payback time for such a vehicle, not counting development labor (which is a big number, but is considered sunk cost, since no new staff would be hired) would be ~3 years

Proposed development:

- Dorado AUV

- Rechargeable battery packs w/ shutdown feature

- Downward-looking still camera/strobe system (VARS analyses, processing- Crissy figure?)

- Upward-looking multi-beam sonar (Echoview analyses – see Fig. 4)

- Inertial navigation system (running same trajectory for each transect)

- Acoustic modem for Wave Glider communications (status reports)

- Acoustic triggering of surveys from surface

- Mechanical docking using USBL to anchored riser with acoustic beacon (existing USBL)

- Scuttling/emergency anchoring system (for emergency anchoring in case of docking failure)

Proposed Schedule: