From Paul Coenen E-mail

The connectors that we have used are Souriau and Bulgin.  We used the Souriau connectors on the transducers because they seem to be the best quality of the lower cost plastic connectors.  The Bulgins are solid, and they make that battery connector with large enough contacts to make sure that we're not creating pinch points at the bulkheads and dropping too much voltage.

I've attached a spreadsheet that lists all the connectors and specific part numbers, so you can look things up as you see fit.  I've also put the Souriau and Bulgin catalogs in Kelly's tempbox \kbb (I didn't see your tempbox...?).  In this email, I've put links to the connector series overview for each type.

Transducer cable connectors are the Souriau UTS Series:

IP68/69K

UV Resistant

Bayonet style coupler

We're using #20AWG contacts

http://usa.esterline-connection-technologies.com/en-en/products/connectors/circular-connectors/uts-series-plastic-waterproof-ip6869k-connector

https://ipaper.ipapercms.dk/SOURIAU/UTSSeriesUS2015/

Battery connectors are the Bulgin Buccaneer 900 Series:

IP68/69K

32A/600V rated

http://www.bulgin.com/products/circular-power-connectors/900-series.html

http://www.bulgin.com/media/bulgin/data/900%20Series.pdf

Grounding wire connectors are the Bulgin Mini Series:

IP68

http://www.bulgin.com/products/circular-power-connectors/mini-series.html

http://www.bulgin.com/media/bulgin/data/Mini_power.pdf

Ethernet connectors are the Bulgin Standard Series Ethernet:

IP68

CAT5e

http://www.bulgin.com/products/circular-data-connectors/standard-rj45-ethernet.html

http://www.bulgin.com/media/bulgin/data/Standard_ethernet.pdf