Camera test in MBARI test tank

March 6, 2018.

Engineer: François Cazenave

- AIDA camera

The camera was placed in the tank view conference room, just outside the tank window. A target including a checkered pattern and a fake fish were placed in the tank a distances ranging from 0.5m to 4m. Two DSPL light were placed in the water, a short distance above the camera. Two old lights were used because the new light is going to be twice as powerful.

Camera model: AIDA Gen3G Camera

Lens model: AIDA Imaging 2.8mm HD CS Mount Lens for Gen3G Camera

| Files name | Shutter speed (s) | Gain | Distance (m) |
| --- | --- | --- | --- |
| AIDA_Test_01 | 1/180 | -0.3 | 0.5 |
| AIDA_Test_02 | 1/180 | -0.3 | 1 |
| AIDA_Test_03 | 1/180 | -0.3 | 1.5 |
| AIDA_Test_04 | 1/180 | -0.3 | 2 |
| AIDA_Test_05 | 1/180 | -0.3 | 3 |
| AIDA_Test_06 | 1/180 | -0.3 | 4 |
| AIDA_Test_07 | 1/120 | -0.3 | 1 |
| AIDA_Test_08 | 1/120 | -0.3 | 2 |
| AIDA_Test_09 | 1/120 | -0.3 | 3 |
| AIDA_Test_10 | 1/120 | -0.3 | 4 |

- Blackmagic micro camera

A similar test was performed for the blackmagic micro camera. The lens was a Rokinon 7.5mm fisheye. The lens was used fully open at F3.5.  For the latter part of the testing we were able to use the new DSPL light instead of the old ones. This is reflected in the table below.

In files Blackmagic_11 and Blackmagic_4k_05, the camera were set to the same shutter speed, same distance, same white balance and lowest ISO/gain, to compare their sensitivity. It appears that the 4K version of the camera has about half the light sensitivity.

Files Blackmagic_4k-07 to 08 were shot using the 7.5mm Laowa lens.