Benthic Observation Survey System (BOSS) Deployment Installation

PDU

- Place the PDU in an easily accessible location.

  - A dry lab bench top with chair access is preferable.

  - The user will need easy access to the front touch panel.

- Remove the front and rear covers.

- Connect the 10-channel video snake cable to the rear panel.

  - The connections are color coded.

  - Other end goes to the deck video recorder.

- Connect the 2-channel video snake cable to the rear panel.

  - The connections are color coded.

  - The other end goes to the SeeTec (down camera) monitor.

- Connect a long BNC cable to the quad upper video output connection (channel 15).

  - The other end goes to one of the Tote HD monitors.

- Connect a long BNC cable to the quad lower video output connection (channel 16).

  - The other end goes to one of the Tote HD monitors.

- [Optional] Channels 13 and 14 are available for the bridge, if desired.

- Connect the GPS cable to the rear DB9 connector label GPS.

  - The GPS antenna will need to be mounted on the exterior of the ship in a high location.

  - The DB9 side of the cable may need to be run through any portals first if necessary.

- Connect the 2-pin power deck cable [purple end] to the rear panel.

  - The other end [orange end] goes to the winch (junction box).

- Connect the fiber optic deck cable to the rear panel.

  - The other end goes to the winch (junction box).

- Connect the (yellow) power cable from the rear panel to a 115VAC plug.

  - The PDU needs to be on a separate AC circuit as the Deck Video Recorder.

- Secure the PDU to the work surface with ratchet straps.

- For clean cable management, tie-wrap multiple cables together.

Laptop

- Place the laptop next to the PDU

- Plug in the laptop’s wall-wart.

- Plug an Ethernet cable from the laptop to an Ethernet port on the rear of the PDU.

Deck Video Recorder

- Place the deck recorder near the PDU.

  - A dry lab bench top with chair access is preferable.

  - Place with 6-7 feet of the PDU.

  - Place it such that access to the front is available.

    - The front USB ports will be used for transferring videos [may need a usb extender cable].

  - The rear end of the deck recorder should face towards the PDU.

- Remove the front and rear covers.

- Connect the other end of the 10-channel video snake cable.

  - The connections are color coded.

  - Other end is from the PDU.

- Connect the keyboard and mouse to the USB port in the rear.

- Connect the VGA cable to the monitor (permanently plugged into rear of deck recorder).

  - Plug the 115VAC power cord form the monitor to the power strip in the rear of the deck recorder.

- Connect deck recorder power cord to a 115VAC plug.

  - The deck recorder needs to be on a separate AC circuit as the PDU.

- Secure the deck recorder to the work surface with ratchet straps.

- For clean cable management, tie-wrap multiple cables together.

Video Monitors

- The two Tote monitors display the eight stereo cameras.

  - Connect their power supplies to 115Vac plugs.

  - Connect the two long BNC cables for the quad upper and lower videos.

    - SDI IN

- The SeeTec monitor displays the down camera.

  - Connect the power supply to a 115Vac plug.

  - Connect the 2-channel video snake cable to the video switch on the back of the monitor.

    - The other end is from the PDU rear panel.

- Secure the monitors to the work surface with bungee cords.

- For clean cable management, tie-wrap multiple cables together.

Winch

- The winch should be bolted to the ship’s deck with the steel feet plates per the ship’s crew.

- Plug in the 220VAC input power cable to the specified ship connection.

- Plug in the 2-pin power deck cable to the outer junction box on the winch.

- Plug in the fiber optic deck cable to the outer junction box on the winch.

- Physically route the 2-pin power cable and fiber cable to minimize it being walked on.